|  | MICHIGAN DEPARTMENT OF ENVIRONMENTAL QUALITY AIR QUALITY DIVISION |  |
| --- | --- | --- |
| EFFECTIVE DATE: AUGUST 20, 2018 ISSUED TO Faurecia Interior System Saline, LLC and Ford Motor Company – Saline Plant State Registration Number (SRN): B2063 LOCATED AT 7700 Michigan Avenue, Saline, Michigan 48176 |  |  |
| RENEWABLE OPERATING PERMIT Permit Number: MI-ROP-B2063-2018 Expiration Date: August 20, 2023 Administratively Complete ROP Renewal Application Due Between February 20, 2022 and February 20, 2023 This Renewable Operating Permit (ROP) is issued in accordance with and subject to Section 5506(3) of Part 55, Air Pollution Control, of the Natural Resources and Environmental Protection Act, 1994 PA 451, as amended (Act 451). Pursuant to Michigan Air Pollution Control Rule 210(1), this ROP constitutes the permittee’s authority to operate the stationary source identified above in accordance with the general conditions, special conditions and attachments contained herein. Operation of the stationary source and all emission units listed in the permit are subject to all applicable future or amended rules and regulations pursuant to Act 451 and the federal Clean Air Act. |  |  |

SOURCE-WIDE PERMIT TO INSTALL Permit Number: MI-PTI-B2063-2018 This Permit to Install (PTI) is issued in accordance with and subject to Section 5505(5) of Act 451. Pursuant to Michigan Air Pollution Control Rule 214a, the terms and conditions herein, identified by the underlying applicable requirement citation of Rule 201(1)(a), constitute a federally enforceable PTI. The PTl terms and conditions do not expire and remain in effect unless the criteria of Rule 201(6) are met. Operation of all emission units identified in the PTI is subject to all applicable future or amended rules and regulations pursuant to Act 451 and the federal Clean Air Act.

Michigan Department of Environmental Quality

______________________________________

Scott Miller, Jackson District Supervisor 
TABLE OF CONTENTS

AUTHORITY AND ENFORCEABILITY	4

SECTION 1 – FAURECIA INTERIOR SYSTEMS SALINE, LLC	5

A.  GENERAL CONDITIONS	6

Permit Enforceability	6

General Provisions	6

Equipment & Design	7

Emission Limits	7

Testing/Sampling	7

Monitoring/Recordkeeping	8

Certification & Reporting	8

Permit Shield	9

Revisions	10

Reopenings	10

Renewals	11

Stratospheric Ozone Protection	11

Risk Management Plan	11

Emission Trading	11

Permit to Install (PTI)	12

B.  SOURCE-WIDE CONDITIONS	13

C.  EMISSION UNIT CONDITIONS	14

EMISSION UNIT SUMMARY TABLE	14

EU-AutoPlasCoatLn	17

D.  FLEXIBLE GROUP CONDITIONS	21

FLEXIBLE GROUP SUMMARY TABLE	21

FG-BOILERS	23

FG-IMCPULINES1&2	25

FG-IMCPULINES3	30

FG-MACTPPPP	35

FG-MACT-ZZZZ-EMERGENCY RICE	39

FG-BOILERMACT	42

FG-RULE287(2)(C)	51

FG-RULE290	53

FG-COLDCLEANERS	57

E.  NON-APPLICABLE REQUIREMENTS	60

APPENDICES	61

Appendix 1-1.  Acronyms and Abbreviations	61

Appendix 2-1.  Schedule of Compliance	62

Appendix 3-1.  Monitoring Requirements	62

Appendix 4-1.  Recordkeeping	62

Appendix 5-1.  Testing Procedures	62

Appendix 6-1.  Permits to Install	62

Appendix 7-1.  Emission Calculations	63

Appendix 8-1.  Reporting	63

SECTION 2 – Ford Motor Company – Saline Plant	64

A.  GENERAL CONDITIONS	65

Permit Enforceability	65

General Provisions	65

Equipment & Design	66

Emission Limits	66

Testing/Sampling	66

Monitoring/Recordkeeping	67

Certification & Reporting	67

Permit Shield	68

Revisions	69

Reopenings	69

Renewals	70

Stratospheric Ozone Protection	70

Risk Management Plan	70

Emission Trading	70

Permit to Install (PTI)	71

B.  SOURCE-WIDE CONDITIONS	72

C.  EMISSION UNIT CONDITIONS	73

D.  FLEXIBLE GROUP CONDITIONS	74

E.  NON-APPLICABLE REQUIREMENTS	75

APPENDICES	76

Appendix 1-2.  Acronyms and Abbreviations	76

Appendix 2-2.  Schedule of Compliance	77

Appendix 3-2.  Monitoring Requirements	77

Appendix 4-2.  Recordkeeping	77

Appendix 5-2.  Testing Procedures	77

Appendix 6-2.  Permits to Install	77

Appendix 7-2.  Emission Calculations	77

Appendix 8-2.  Reporting	77

# AUTHORITY AND ENFORCEABILITY

For the purpose of this permit, the permittee is defined as any person who owns or operates an emission unit at a stationary source for which this permit has been issued.  The department is defined in Rule 104(d) as the Director of the Michigan Department of Environmental Quality (MDEQ) or his or her designee.

The permittee shall comply with all specific details in the permit terms and conditions and the cited underlying applicable requirements.  All terms and conditions in this ROP are both federally enforceable and state enforceable unless otherwise footnoted.  Certain terms and conditions are applicable to most stationary sources for which an ROP has been issued.  These general conditions are included in Part A of this ROP.  Other terms and conditions may apply to a specific emission unit, several emission units which are represented as a flexible group, or the entire stationary source which is represented as a Source-Wide group.  Special conditions are identified in Parts B, C, D and/or the appendices.

In accordance with Rule 213(2)(a), all underlying applicable requirements are identified for each ROP term or condition.  All terms and conditions that are included in a PTI are streamlined, subsumed and/or is state-only enforceable will be noted as such.

In accordance with Section 5507 of Act 451, the permittee has included in the ROP application a compliance certification, a schedule of compliance, and a compliance plan.  For applicable requirements with which the source is in compliance, the source will continue to comply with these requirements.  For applicable requirements with which the source is not in compliance, the source will comply with the detailed schedule of compliance requirements that are incorporated as an appendix in this ROP.  Furthermore, for any applicable requirements effective after the date of issuance of this ROP, the stationary source will meet the requirements on a timely basis, unless the underlying applicable requirement requires a more detailed schedule of compliance.

Issuance of this permit does not obviate the necessity of obtaining such permits or approvals from other units of government as required by law.

# SECTION 1 – FAURECIA INTERIOR SYSTEMS SALINE, LLC

# A.  GENERAL CONDITIONS

## Permit Enforceability

- All conditions in this permit are both federally enforceable and state enforceable unless otherwise noted. (R 336.1213(5))

- Those conditions that are hereby incorporated in a state-only enforceable Source-Wide PTI pursuant to Rule 201(2)(d) are designated by footnote one.  (R 336.1213(5)(a), R 336.1214a(5))

- Those conditions that are hereby incorporated in a federally enforceable Source-Wide PTI pursuant to Rule 201(2)(c) are designated by footnote two.  (R 336.1213(5)(b), R 336.1214a(3))

## General Provisions

- The permittee shall comply with all conditions of this ROP.  Any ROP noncompliance constitutes a violation of Act 451, and is grounds for enforcement action, for ROP revocation or revision, or for denial of the renewal of the ROP.  All terms and conditions of this ROP that are designated as federally enforceable are enforceable by the Administrator of the United States Environmental Protection Agency (USEPA) and by citizens under the provisions of the federal Clean Air Act (CAA).  Any terms and conditions based on applicable requirements which are designated as “state-only” are not enforceable by the USEPA or citizens pursuant to the CAA.  (R 336.1213(1)(a))

- It shall not be a defense for the permittee in an enforcement action that it would have been necessary to halt or reduce the permitted activity in order to maintain compliance with the conditions of this ROP.  (R 336.1213(1)(b))

- This ROP may be modified, revised, or revoked for cause.  The filing of a request by the permittee for a permit modification, revision, or termination, or a notification of planned changes or anticipated noncompliance does not stay any ROP term or condition.  This does not supersede or affect the ability of the permittee to make changes, at the permittee’s own risk, pursuant to Rule 215 and Rule 216.  (R 336.1213(1)(c))

- The permittee shall allow the department, or an authorized representative of the department, upon presentation of credentials and other documents as may be required by law and upon stating the authority for and purpose of the investigation, to perform any of the following activities:  (R 336.1213(1)(d))

  - Enter, at reasonable times, a stationary source or other premises where emissions-related activity is conducted or where records must be kept under the conditions of the ROP.

  - Have access to and copy, at reasonable times, any records that must be kept under the conditions of the ROP.

  - Inspect, at reasonable times, any of the following:

    - Any stationary source.

    - Any emission unit.

    - Any equipment, including monitoring and air pollution control equipment.

    - Any work practices or operations regulated or required under the ROP.

  - As authorized by Section 5526 of Act 451, sample or monitor at reasonable times substances or parameters for the purpose of assuring compliance with the ROP or applicable requirements.

- The permittee shall furnish to the department, within a reasonable time, any information the department may request, in writing, to determine whether cause exists for modifying, revising, or revoking the ROP or to determine compliance with this ROP.  Upon request, the permittee shall also furnish to the department copies of any records that are required to be kept as a term or condition of this ROP.  For information which is claimed by the permittee to be confidential, consistent with the requirements of the 1976 PA 442, MCL §15.231 et seq., and known as the Freedom of Information Act, the person may also be required to furnish the records directly to the USEPA together with a claim of confidentiality.  (R 336.1213(1)(e))

- A challenge by any person, the Administrator of the USEPA, or the department to a particular condition or a part of this ROP shall not set aside, delay, stay, or in any way affect the applicability or enforceability of any other condition or part of this ROP.  (R 336.1213(1)(f))

- The permittee shall pay fees consistent with the fee schedule and requirements pursuant to Section 5522 of Act 451.  (R 336.1213(1)(g))

- This ROP does not convey any property rights or any exclusive privilege.  (R 336.1213(1)(h))

## Equipment & Design

- Any collected air contaminants shall be removed as necessary to maintain the equipment at the required operating efficiency.  The collection and disposal of air contaminants shall be performed in a manner so as to minimize the introduction of contaminants to the outer air.  Transport of collected air contaminants in Priority I and II areas requires the use of material handling methods specified in Rule 370(2).2  (R 336.1370)

- Any air cleaning device shall be installed, maintained, and operated in a satisfactory manner and in accordance with the Michigan Air Pollution Control rules and existing law.  (R 336.1910)

## Emission Limits

- Unless otherwise specified in this ROP, the permittee shall comply with Rule 301, which states, in part, “Except as provided in subrules 2, 3, and 4 of this rule, a person shall not cause or permit to be discharged into the outer air from a process or process equipment a visible emission of a density greater than the most stringent of the following:”2  (R 336.1301(1))

  - A 6-minute average of 20% opacity, except for one 6-minute average per hour of not more than 27% opacity.

  - A limit specified by an applicable federal new source performance standard.

The grading of visible emissions shall be determined in accordance with Rule 303.

- The permittee shall not cause or permit the emission of an air contaminant or water vapor in quantities that cause, alone or in reaction with other air contaminants, either of the following:

  - Injurious effects to human health or safety, animal life, plant life of significant economic value, or property.1  (R 336.1901(a))

  - Unreasonable interference with the comfortable enjoyment of life and property.1  (R 336.1901(b))

## Testing/Sampling

- The department may require the owner or operator of any source of an air contaminant to conduct acceptable performance tests, at the owner’s or operator’s expense, in accordance with Rule 1001 and Rule 1003, under any of the conditions listed in Rule 1001(1).2  (R 336.2001)

- Any required performance testing shall be conducted in accordance with Rule 1001(2), Rule 1001(3) and Rule 1003.  (R 336.2001(2), R 336.2001(3), R 336.2003(1))

- Any required test results shall be submitted to the Air Quality Division (AQD) in the format prescribed by the applicable reference test method within 60 days following the last date of the test.  (R 336.2001(5))

## Monitoring/Recordkeeping

- Records of any periodic emission or parametric monitoring required in this ROP shall include the following information specified in Rule 213(3)(b)(i), where appropriate.  (R 336.1213(3)(b))

  - The date, location, time, and method of sampling or measurements.

  - The dates the analyses of the samples were performed.

  - The company or entity that performed the analyses of the samples.

  - The analytical techniques or methods used.

  - The results of the analyses.

  - The related process operating conditions or parameters that existed at the time of sampling or measurement.

- All required monitoring data, support information and all reports, including reports of all instances of deviation from permit requirements, shall be kept and furnished to the department upon request for a period of not less than 5 years from the date of the monitoring sample, measurement, report or application.  Support information includes all calibration and maintenance records and all original strip-chart recordings, or other original data records, for continuous monitoring instrumentation and copies of all reports required by the ROP.  (R 336.1213(1)(e), R 336.1213(3)(b)(ii))

## Certification & Reporting

- Except for the alternate certification schedule provided in Rule 213(3)(c)(iii)(B), any document required to be submitted to the department as a term or condition of this ROP shall contain an original certification by a Responsible Official which states that, based on information and belief formed after reasonable inquiry, the statements and information in the document are true, accurate, and complete.  (R 336.1213(3)(c))

- A Responsible Official shall certify to the appropriate AQD District Office and to the USEPA that the stationary source is and has been in compliance with all terms and conditions contained in the ROP except for deviations that have been or are being reported to the appropriate AQD District Office pursuant to Rule 213(3)(c).  This certification shall include all the information specified in Rule 213(4)(c)(i) through (v) and shall state that, based on information and belief formed after reasonable inquiry, the statements and information in the certification are true, accurate, and complete.  The USEPA address is:  USEPA, Air Compliance Data - Michigan, Air and Radiation Division, 77 West Jackson Boulevard, Chicago, Illinois 60604-3507.  (R 336.1213(4)(c))

- The certification of compliance shall be submitted annually for the term of this ROP as detailed in the special conditions, or more frequently if specified in an applicable requirement or in this ROP.  (R 336.1213(4)(c))

- The permittee shall promptly report any deviations from ROP requirements and certify the reports.  The prompt reporting of deviations from ROP requirements is defined in Rule 213(3)(c)(ii) as follows, unless otherwise described in this ROP.  (R 336.1213(3)(c))

  - For deviations that exceed the emissions allowed under the ROP, prompt reporting means reporting consistent with the requirements of Rule 912 as detailed in Condition 25.  All reports submitted pursuant to this paragraph shall be promptly certified as specified in Rule 213(3)(c)(iii).

  - For deviations which exceed the emissions allowed under the ROP and which are not reported pursuant to Rule 912 due to the duration of the deviation, prompt reporting means the reporting of all deviations in the semiannual reports required by Rule 213(3)(c)(i).  The report shall describe reasons for each deviation and the actions taken to minimize or correct each deviation.

  - For deviations that do not exceed the emissions allowed under the ROP, prompt reporting means the reporting of all deviations in the semiannual reports required by Rule 213(3)(c)(i).  The report shall describe the reasons for each deviation and the actions taken to minimize or correct each deviation.

- For reports required pursuant to Rule 213(3)(c)(ii), prompt certification of the reports is described in Rule 213(3)(c)(iii) as either of the following:  (R 336.1213(3)(c))

  - Submitting a certification by a Responsible Official with each report which states that, based on information and belief formed after reasonable inquiry, the statements and information in the report are true, accurate, and complete.

  - Submitting, within 30 days following the end of a calendar month during which one or more prompt reports of deviations from the emissions allowed under the ROP were submitted to the department pursuant to Rule 213(3)(c)(ii), a certification by a Responsible Official which states that; “based on information and belief formed after reasonable inquiry, the statements and information contained in each of the reports submitted during the previous month were true, accurate, and complete.”  The certification shall include a listing of the reports that are being certified.  Any report submitted pursuant to Rule 213(3)(c)(ii) that will be certified on a monthly basis pursuant to this paragraph shall include a statement that certification of the report will be provided within 30 days following the end of the calendar month.

- Semiannually for the term of the ROP as detailed in the special conditions, or more frequently if specified, the permittee shall submit certified reports of any required monitoring to the appropriate AQD District Office.  All instances of deviations from ROP requirements during the reporting period shall be clearly identified in the reports.  (R 336.1213(3)(c)(i))

- On an annual basis, the permittee shall report the actual emissions, or the information necessary to determine the actual emissions, of each regulated air pollutant as defined in Rule 212(6) for each emission unit utilizing the emissions inventory forms provided by the department.  (R 336.1212(6))

- The permittee shall provide notice of an abnormal condition, start-up, shutdown, or malfunction that results in emissions of a hazardous or toxic air pollutant which continue for more than one hour in excess of any applicable standard or limitation, or emissions of any air contaminant continuing for more than two hours in excess of an applicable standard or limitation, as required in Rule 912, to the appropriate AQD District Office.  The notice shall be provided not later than two business days after the start-up, shutdown, or discovery of the abnormal conditions or malfunction.  Notice shall be by any reasonable means, including electronic, telephonic, or oral communication.  Written reports, if required under Rule 912, must be submitted to the appropriate AQD District Supervisor within 10 days after the start-up or shutdown occurred, within 10 days after the abnormal conditions or malfunction has been corrected, or within 30 days of discovery of the abnormal conditions or malfunction, whichever is first.  The written reports shall include all of the information required in Rule 912(5) and shall be certified by a Responsible Official in a manner consistent with the CAA.2  (R 336.1912)

## Permit Shield

- Compliance with the conditions of the ROP shall be considered compliance with any applicable requirements as of the date of ROP issuance, if either of the following provisions is satisfied.  (R 336.1213(6)(a)(i), R 336.1213(6)(a)(ii))

  - The applicable requirements are included and are specifically identified in the ROP.

  - The permit includes a determination or concise summary of the determination by the department that other specifically identified requirements are not applicable to the stationary source.

Any requirements identified in Part E of this ROP have been identified as non-applicable to this ROP and are included in the permit shield.

- Nothing in this ROP shall alter or affect any of the following:

  - The provisions of Section 303 of the CAA, emergency orders, including the authority of the USEPA under Section 303 of the CAA.  (R 336.1213(6)(b)(i))

  - The liability of the owner or operator of this source for any violation of applicable requirements prior to or at the time of this ROP issuance.  (R 336.1213(6)(b)(ii))

  - The applicable requirements of the acid rain program, consistent with Section 408(a) of the CAA.  (R 336.1213(6)(b)(iii))

  - The ability of the USEPA to obtain information from a source pursuant to Section 114 of the CAA.  (R 336.1213(6)(b)(iv))

- The permit shield shall not apply to provisions incorporated into this ROP through procedures for any of the following:

  - Operational flexibility changes made pursuant to Rule 215.  (R 336.1215(5))

  - Administrative Amendments made pursuant to Rule 216(1)(a)(i)-(iv).  (R 336.1216(1)(b)(iii))

  - Administrative Amendments made pursuant to Rule 216(1)(a)(v) until the amendment has been approved by the department.  (R 336.1216(1)(c)(iii))

  - Minor Permit Modifications made pursuant to Rule 216(2).  (R 336.1216(2)(f))

  - State-Only Modifications made pursuant to Rule 216(4) until the changes have been approved by the department.  (R 336.1216(4)(e))

- Expiration of this ROP results in the loss of the permit shield.  If a timely and administratively complete application for renewal is submitted not more than 18 months, but not less than 6 months, before the expiration date of the ROP, but the department fails to take final action before the end of the ROP term, the existing ROP does not expire until the renewal is issued or denied, and the permit shield shall extend beyond the original ROP term until the department takes final action.  (R 336.1217(1)(c), R 336.1217(1)(a))

## Revisions

- For changes to any process or process equipment covered by this ROP that do not require a revision of the ROP pursuant to Rule 216, the permittee must comply with Rule 215.  (R 336.1215, R 336.1216)

- A change in ownership or operational control of a stationary source covered by this ROP shall be made pursuant to Rule 216(1).  (R 336.1219(2))

- For revisions to this ROP, an administratively complete application shall be considered timely if it is received by the department in accordance with the time frames specified in Rule 216.  (R 336.1210(10))

- Pursuant to Rule 216(1)(b)(iii), Rule 216(2)(d) and Rule 216(4)(d), after a change has been made, and until the department takes final action, the permittee shall comply with both the applicable requirements governing the change and the ROP terms and conditions proposed in the application for the modification.  During this time period, the permittee may choose to not comply with the existing ROP terms and conditions that the application seeks to change.  However, if the permittee fails to comply with the ROP terms and conditions proposed in the application during this time period, the terms and conditions in the ROP are enforceable.  (R 336.1216(1)(c)(iii), R 336.1216(2)(d), R 336.1216(4)(d))

## Reopenings

- A ROP shall be reopened by the department prior to the expiration date and revised by the department under any of the following circumstances:

  - If additional requirements become applicable to this stationary source with three or more years remaining in the term of the ROP, but not if the effective date of the new applicable requirement is later than the ROP expiration date.  (R 336.1217(2)(a)(i))

  - If additional requirements pursuant to Title IV of the CAA become applicable to this stationary source.  (R 336.1217(2)(a)(ii))

  - If the department determines that the ROP contains a material mistake, information required by any applicable requirement was omitted, or inaccurate statements were made in establishing emission limits or the terms or conditions of the ROP.  (R 336.1217(2)(a)(iii))

  - If the department determines that the ROP must be revised to ensure compliance with the applicable requirements.  (R 336.1217(2)(a)(iv))

## Renewals

- For renewal of this ROP, an administratively complete application shall be considered timely if it is received by the department not more than 18 months, but not less than 6 months, before the expiration date of the ROP.  (R 336.1210(9))

## Stratospheric Ozone Protection

- If the permittee is subject to Title 40 of the Code of Federal Regulations (CFR), Part 82 and services, maintains, or repairs appliances except for motor vehicle air conditioners (MVAC), or disposes of appliances containing refrigerant, including MVAC and small appliances, or if the permittee is a refrigerant reclaimer, appliance owner or a manufacturer of appliances or recycling and recovery equipment, the permittee shall comply with all applicable standards for recycling and emissions reduction pursuant to 40 CFR Part 82, Subpart F.

- If the permittee is subject to 40 CFR Part 82, and performs a service on motor (fleet) vehicles when this service involves refrigerant in the MVAC, the permittee is subject to all the applicable requirements as specified in 40 CFR Part 82, Subpart B, Servicing of Motor Vehicle Air Conditioners.  The term “motor vehicle” as used in Subpart B does not include a vehicle in which final assembly of the vehicle has not been completed by the original equipment manufacturer.  The term MVAC as used in Subpart B does not include the air-tight sealed refrigeration system used for refrigerated cargo or an air conditioning system on passenger buses using Hydrochlorofluorocarbon-22 refrigerant.

## Risk Management Plan

- If subject to Section 112(r) of the CAA and 40 CFR Part 68, the permittee shall register and submit to the USEPA the required data related to the risk management plan for reducing the probability of accidental releases of any regulated substances listed pursuant to Section 112(r)(3) of the CAA as amended in 40 CFR 68.130.  The list of substances, threshold quantities, and accident prevention regulations promulgated under 40 CFR Part 68, do not limit in any way the general duty provisions under Section 112(r)(1).

- If subject to Section 112(r) of the CAA and 40 CFR Part 68, the permittee shall comply with the requirements of 40 CFR Part 68, no later than the latest of the following dates as provided in 40 CFR 68.10(a):

  - June 21, 1999,

  - Three years after the date on which a regulated substance is first listed under 40 CFR 68.130, or

  - The date on which a regulated substance is first present above a threshold quantity in a process.

- If subject to Section 112(r) of the CAA and 40 CFR Part 68, the permittee shall submit any additional relevant information requested by any regulatory agency necessary to ensure compliance with the requirements of 40 CFR Part 68.

- If subject to Section 112(r) of the CAA and 40 CFR Part 68, the permittee shall annually certify compliance with all applicable requirements of Section 112(r) as detailed in Rule 213(4)(c)).  (40 CFR Part 68)

## Emission Trading

- Emission averaging and emission reduction credit trading are allowed pursuant to any applicable interstate or regional emission trading program that has been approved by the Administrator of the USEPA as a part of Michigan’s State Implementation Plan.  Such activities must comply with Rule 215 and Rule 216. (R 336.1213(12))

## Permit to Install (PTI)

- The process or process equipment included in this permit shall not be reconstructed, relocated, or modified unless a PTI authorizing such action is issued by the department, except to the extent such action is exempt from the PTI requirements by any applicable rule.2  (R 336.1201(1))

- The department may, after notice and opportunity for a hearing, revoke PTI terms or conditions if evidence indicates the process or process equipment is not performing in accordance with the terms and conditions of the PTI or is violating the department’s rules or the CAA.2  (R 336.1201(8), Section 5510 of Act 451)

- The terms and conditions of a PTI shall apply to any person or legal entity that now or hereafter owns or operates the process or process equipment at the location authorized by the PTI.  If a new owner or operator submits a written request to the department pursuant to Rule 219 and the department approves the request, this PTI will be amended to reflect the change of ownership or operational control.  The request must include all of the information required by Subrules (1)(a), (b) and (c) of Rule 219.  The written request shall be sent to the appropriate AQD District Supervisor, MDEQ.2  (R 336.1219)

- If the installation, reconstruction, relocation, or modification of the equipment for which PTI terms and conditions have been approved has not commenced within 18 months of the original PTI issuance date, or has been interrupted for 18 months, the applicable terms and conditions from that PTI, as incorporated into the ROP, shall become void unless otherwise authorized by the department.  Furthermore, the person to whom that PTI was issued, or the designated authorized agent, shall notify the department via the Supervisor, Permit Section, MDEQ, AQD, P. O. Box 30260, Lansing, Michigan 48909, if it is decided not to pursue the installation, reconstruction, relocation, or modification of the equipment allowed by the terms and conditions from that PTI.2   (R 336.1201(4))

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

# B.  SOURCE-WIDE CONDITIONS

Part B outlines the Source-Wide Terms and Conditions that apply to this stationary source.  The permittee is subject to these special conditions for the stationary source in addition to the general conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply to this source, NA (not applicable) has been used in the table.  If there are no Source-Wide Conditions, this section will be left blank.

# C.  EMISSION UNIT CONDITIONS

Part C outlines terms and conditions that are specific to individual emission units listed in the Emission Unit Summary Table.  The permittee is subject to the special conditions for each emission unit in addition to the General Conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply, NA (not applicable) has been used in the table.  If there are no conditions specific to individual emission units, this section will be left blank.

## EMISSION UNIT SUMMARY TABLE

The descriptions provided below are for informational purposes and do not constitute enforceable conditions.

| Emission Unit ID | Emission Unit Description (Including Process Equipment & Control Device(s)) | Installation Date/ Modification Date | Flexible Group ID |
| --- | --- | --- | --- |
| EU-166-77 | Natural gas fired steam boiler (17,250 lb steam/hr York Shipley automatic boiler; 12.9 MMBTU/HR) with fuel oil burning capabilities. (DVBOILER166-77) | 06/01/1977 | FG-BOILERS FG-BOILERMACT |
| EU-403-74 | Natural gas fired steam boiler (10,350 lb steam/hr York Shipley boiler; 20.95 MMBTU/HR) with fuel oil burning capabilities. (DV403-74BOILER) | 01/06/1975 | FG-BOILERS FG-BOILERMACT |
| EU-COLDCLEANERS | This emission group is meant to cover any existing and future cold cleaners located within the facility and covered by Rule 707. | After 07/01/1979 | FG-COLDCLEANERS |
| EU-RULE287(2)(c) | Any emission unit that emits air contaminants and is exempt from the requirements of Rule 201 pursuant to Rules 278, 278a and 287(2)(c). | NA | FG-RULE287(2)(c) |
| EU-1IMCPU1 | This line consists of one robotic in-mold coating (IMC) booth, a manual IMC booth, an IMC flash-off booth, a polyurethane (PU) booth, and a PU curing booth. Cleanup materials for the PU process are included in this emission unit. | 08-2008/ 02-2009 & 01-2010 | FG-IMCPULINES1&2 FG-MACTPPPP |
| EU-1IMCPU2 | This line consists of one robotic (IMC booth, a manual IMC booth, an IMC flash-off booth, a PU booth, and a PU curing booth. Cleanup materials for the PU process are included in this emission unit. | 08-2008/ 02-2009 & 01-2010 | FG-IMCPULINES1&2 FG-MACTPPPP |
| EU-1IMCPU3 | This line consists of one robotic (IMC) booth, a manual IMC booth, an IMC flash-off booth, a PU booth, and a PU curing booth. Cleanup materials for the PU process are included in this emission unit. | 08-2008/ 02-2009 & 01-2010 | FG-IMCPULINES1&2 FG-MACTPPPP |
| EU-1MOLDRELEASE | An enclosed booth used for applying mold release material(s) to parts prior to coating. | 08-2008/ 02-2009 & 01-2010 | FG-IMCPULINES1&2 |
| EU-1MASKSTATION | Station used to apply masking materials to parts which require a two-tone finish. | 08-2008/ 02-2009 & 01-2010 | FG-IMCPULINES1&2 |
| EU-1TOOLCLEAN | An enclosed booth within which the mold tools are cleaned for reuse in the process. | 08-2008/ 02-2009 & 01-2010 | FG-IMCPULINES1&2 |
| EU-2IMCPU1 | This line consists of one robotic IMC booth, a manual IMC booth, an IMC flash-off booth, a PU booth, and a PU curing booth. Cleanup material for the PU process is included in this emission unit. | 02-2009/ 01-2010 | FG-IMCPULINES1&2 FG-MACTPPPP |
| EU-2IMCPU2 | This line consists of one robotic IMC booth, a manual IMC booth, an IMC flash-off booth, a PU booth, and a PU curing booth. Cleanup material for the PU process is included in this emission unit. | 02-2009/ 01-2010 | FG-IMCPULINES1&2 FG-MACTPPPP |
| EU-2IMCPU3 | This line consists of one robotic IMC booth, a manual IMC booth, an IMC flash-off booth, a PU booth, and a PU curing booth. Cleanup material for the PU process is included in this emission unit. | 02-2009/ 01-2010 | FG-IMCPULINES1&2 FG-MACTPPPP |
| EU-2MOLDRELEASE | An enclosed booth used for applying mold release material(s) to parts prior to coating. | 02-2009/ 01-2010 | FG-IMCPULINES1&2 |
| EU-2MASKSTATION | Station used to apply masking materials to parts which require a two-tone finish. There is no exhaust stack directly associated with this station. | 02-2009/ 01-2010 | FG-IMCPULINES1&2 |
| EU-2TOOLCLEAN | An enclosed booth within which the mold tools are cleaned for reuse in the process. | 02-2009/ 01-2010 | FG-IMCPULINES1&2 |
| EU-3IMCPU1 | This line consists of two robotic IMC booths, a manual IMC booth, an IMC flash-off booth, a PU booth, and a PU curing booth. Cleanup material for the PU process is included in this emission unit. | 08-2010 | FG-IMCPULINES3 FG-MACTPPPP |
| EU-3IMCPU2 | This line consists of two robotic IMC booths, a manual IMC booth, an IMC flash-off booth, a PU booth, and a PU curing booth. Cleanup material for the PU process is included in this emission unit. | 08-2010 | FG-IMCPULINES3 FG-MACTPPPP |
| EU-3IMCPU3 | This line consists of two robotic IMC booths, a manual IMC booth, an IMC flash-off booth, a PU booth, and a PU curing booth. Cleanup material for the PU process is included in this emission unit. | 08-2010 | FG-IMCPULINES3 FG-MACTPPPP |
| EU-3MOLDRELEASE | An enclosed booth used for applying mold release material(s) to parts prior to coating. | 08-2010 | FG-IMCPULINES3 |
| EU-3MASKSTATION | Station used to apply masking materials to parts which require a two-tone finish. There is no exhaust stack directly associated with this station. | 08-2010 | FG-IMCPULINES3 |
| EU-3TOOLCLEAN1 | An enclosed booth within which the mold tools are cleaned for reuse in the process. | 08-2010 | FG-IMCPULINES3 |
| EU-3TOOLCLEAN2 | An enclosed booth within which the mold tools are cleaned for reuse in the process. | 01-2011 | FG-IMCPULINES3 |
| EU-3TOOLCLEAN3 | An enclosed booth within which the mold tools are cleaned for reuse in the process. | 07-2011 | FG-IMCPULINES3 |
| EU-Pump612 | 265 HP Diesel Pump 612 (6233) | 1996 | FG-MACT ZZZZ- EMERGENCY RICE |
| EU-Pump613 | 255 HP Diesel Pump 613 (6234) | 1970 | FG-MACT ZZZZ- EMERGENCY RICE |
| EU-Pump614 | 255 HP Diesel Pump 614 (6235) | 1979 | FG-MACT ZZZZ- EMERGENCY RICE |
| EU-WetWellPump | 20 HP Wet Well Emergency Backup | 1972 | FG-MACT ZZZZ- EMERGENCY RICE |
| EU-Generac | 88 HP Generac Natural Gas Generator | 2007 | FG-MACT ZZZZ- EMERGENCY RICE |
| EU-AutoPlasCoatLn | Automotive Interior Plastic Parts Coating Line: The coating line includes a destat booth, two robotic applicators within a single coating booth, flash-off tunnel, and associated natural gas-fired cure oven. The air-dried coatings will be water-based and solvent-based paints, and water or solvent will be used for purge and cleanup. This emission unit includes a parts wipe station with in-plant emissions. | 09/05/2013 | FG-MACTPPPP |
| EU-GlueRobot | The Glue Robot Line is used to glue together a substrate and skin to produce plastic interior automotive panels for passenger vehicles. The Glue Robot sprays glue on both the skins and substrates. A glue kitchen is used to meter and supply glue and purge materials to the Glue Robot. | 2014 | FG-RULE290-1 |
| EU-CastSkins | Solvent Use on Cast Skins | 2012 | FG-RULE290-1 |

## EU-AutoPlasCoatLn

EMISSION UNIT CONDITIONS

DESCRIPTION

Automotive Interior Plastic Parts Coating Line:  The coating line includes a destat booth, two robotic applicators within a single coating booth, flash-off tunnel, and associated natural gas-fired cure oven.  The air-dried coatings will be water-based and solvent-based paints, and water or solvent will be used for purge and cleanup.  This emission unit includes a parts wipe station with in-plant emissions.

Flexible Group ID:  FG-MACTPPPP

POLLUTION CONTROL EQUIPMENT

Filter panels to control particulate matter

I.  EMISSION LIMITS

| Pollutant | Limit | Time Period / Operating Scenario | Equipment | Testing / Monitoring Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- | --- |
| 1. VOCs | 28.4 tpy2 | 12-month rolling time period as determined at the end of each calendar month | Coatings, purge and cleanup portion of EU-AutoPlasCoatLn | SC VI.1 & VI.3 | R 336.1205 R 336.1702(d) |
| 2. VOCs | 6.5 tpy2 | 12-month rolling time period as determined at the end of each calendar month | Parts wipe portion of EU-AutoPlasCoatLn | SC VI.1 & VI.4 | R 336.1205 R 336.1702(d) |
| 3. Acetone (CAS No. 67-64-1) | 11.5 tpy1 | 12-month rolling time period as determined at the end of each calendar month | EU-AutoPlasCoatLn | SC VI.1 & VI.5 | R 336.1224 |
| 4. VOCs | 5.0 lb/gal (minus water)a as applied2 | Daily volume-weighted average | EU-AutoPlasCoatLn | SC V.1 & VI.3 | R 336.1702(d) |
| a The phrase “minus water” shall also include compounds which are used as organic solvents and which are excluded from the definition of volatile organic compound. (R 336.1602(4)) |  |  |  |  |  |

II.  MATERIAL LIMITS

NA

III.  PROCESS/OPERATIONAL RESTRICTIONS

1.	The permittee shall recover and reclaim, recycle, or dispose of all coatings, paints, reducer, thinners, and purge and clean up solvents, etc. (materials) used for EU-AutoPlasCoatLn, in accordance with all applicable regulations.2  (R 336.1224, R 336.1702(a))

2.	The permittee shall capture all waste materials and shall store them in closed containers.  The permittee shall dispose of all waste materials in an acceptable manner in compliance with all applicable state rules and federal regulations.2  (R 336.1224, R 336.1702(a))

3.	The permittee shall dispose of spent filters in a manner which minimizes the introduction of air contaminants to the outer air.2  (R 336.1224, R 336.1370)

4.	The permittee shall handle all VOC and/or HAP containing materials, in a manner to minimize the generation of fugitive emissions.  The permittee shall keep containers covered at all times except when operator access is necessary.2  (R 336.1205, R 336.1224, R 336.1702(a))

5.	The permittee shall not operate the cure oven portion of EU-AutoPlasCoatLn at a temperature in excess of 194°F.2  (R 336.1702(d))

IV.  DESIGN/EQUIPMENT PARAMETERS

1.	The permittee shall not operate EU-AutoPlasCoatLn unless all respective exhaust filters are installed, maintained and operated in a satisfactory manner.2  (R 336.1224, R 336.1301, R 336.1910)

2.	The permittee shall equip and maintain EU-AutoPlasCoatLn with HVLP applicators or comparable technology with equivalent transfer efficiency.  For HVLP applicators, the permittee shall keep test caps available for pressure testing.2  (R 336.1702(a))

3.	The permittee shall install, calibrate, maintain and operate in a satisfactory manner a device to monitor and record the cure oven temperature on a continuous basis.  For the purposes of this condition, “on a continuous basis” is defined as a data point recorded at least once every 15 minutes.2  (R 336.1702(d))

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The permittee shall determine the VOC content, water content and density of any coating, as applied and as received, using federal Reference Test Method 24.  Upon prior written approval by the AQD District Supervisor, the permittee may determine the VOC content from manufacturer’s formulation data.  If the Method 24 and the formulation values should differ, the permittee shall use the Method 24 results to determine compliance.2  (R 336.1205, R 336.1702, R 336.2001, R 336.2003, R 336.2004, R 336.2040(5))

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The permittee shall complete all required calculations in a format acceptable to the AQD District Supervisor by the 15th day of the calendar month, for the previous calendar month, unless otherwise specified in any monitoring/recordkeeping special condition.2  (R 336.1205, R 336.1224, R 336.1225, R 336.1702)

2.	The permittee shall maintain a current listing from the manufacturer of the chemical composition of each material, including the weight percent of each component.  The data may consist of Material Safety Data Sheets, manufacturer’s formulation data, or both as deemed acceptable by the AQD District Supervisor.  The permittee shall keep all records on file and make them available to the Department upon request.2  (R 336.1224, R 336.1225, R 336.1702)

3.	The permittee shall keep the following information on a calendar day basis for EU-AutoPlasCoatLn:

a.	Gallons (with water) of each VOC containing material used and reclaimed.

b.	VOC content (minus water and with water) of each material as applied.

c.	VOC emission calculations determining the volume-weighted average VOC content of the coatings as applied on a calendar day basis.

d.	VOC mass emission calculations determining the monthly emission rate in tons per calendar month.

e.	VOC mass emission calculations determining the annual emission rate in tons per 12-month rolling time period as determined at the end of each calendar month.

The permittee shall keep the records in a format acceptable to the AQD District Supervisor and make them available to the Department upon request.2  (R 336.1205, R 336.1702)

4.	The permittee shall keep the following information on a calendar month basis for EU-AutoPlasCoatLn:

a.	Gallons (with water) of each VOC containing parts wipe material used and reclaimed.

b.	VOC content (with water) of each part wipe material as applied.

c.	VOC mass emission calculations determining the monthly emission rate in tons per calendar month.

d.	VOC mass emission calculations determining the annual emission rate in tons per 12-month rolling time period as determined at the end of each calendar month.

The permittee shall keep the records in a format acceptable to the AQD District Supervisor and make them available to the Department upon request.2  (R 336.1205, R 336.1702)

5.	The permittee shall keep the following information on a calendar month basis for EU-AutoPlasCoatLn:

a.	Gallons of each acetone containing material used and reclaimed.

b.	Acetone content of each material as applied.

c.	Acetone mass emission calculations determining the monthly emission rate in tons per calendar month.

d.	Acetone mass emission calculations determining the annual emission rate in tons per 12-month rolling time period as determined at the end of each calendar month.

The permittee shall keep the records on file in a format acceptable to the AQD District Supervisor and make them available to the Department upon request.1  (R 336.1224)

6.	The permittee shall monitor and record the cure oven temperature on a continuous basis.  The permittee shall keep, in a satisfactory manner, continuous records of the cure temperature for EU-AutoPlasCoatLn.  For the purposes of this condition, “on a continuous basis” is defined as a data point recorded at least once every 15 minutes.2  (R 336.1702(d))

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.   (R 336.1213(4)(c))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTIONS

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| - SVAutoPlasCoatLn-01 (Spray Booth) | 20.42 | 39.62 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |
| - SVAutoPlasCoatLn-02 (Cure Oven) | 18.02 | 36.42 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |

IX.  OTHER REQUIREMENTS

NA

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

# D.  FLEXIBLE GROUP CONDITIONS

Part D outlines the terms and conditions that apply to more than one emission unit.  The permittee is subject to the special conditions for each flexible group in addition to the General Conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply, NA (not applicable) has been used in the table.  If there are no special conditions that apply to more than one emission unit, this section will be left blank.

## FLEXIBLE GROUP SUMMARY TABLE

The descriptions provided below are for informational purposes and do not constitute enforceable conditions.

| Flexible Group ID | Flexible Group Description | Associated Emission Unit IDs |
| --- | --- | --- |
| FG-BOILERS | Two natural gas-fired steam boilers with fuel oil burning capabilities. | EU-166-77 EU-403-74 |
| FG-IMCPULINES1&2 | Two in-mold coating/polyurethane coating processes. Each process consists of three individual coating lines, a single mold release booth, a single masking station, and a single tool cleaning booth. | EU-1IMCPU1 EU-1IMCPU2 EU-1IMCPU3 EU-1MOLDRELEASE EU-1MASKSTATION EU-1TOOLCLEAN EU-2IMCPU1 EU-2IMCPU2 EU-2IMCPU3 EU-2MOLDRELEASE EU-2MASKSTATION EU-2TOOLCLEAN |
| FG-IMCPULINES3 | In-mold coating/polyurethane coating processes consisting of three individual coating lines, a single mold release booth, a single masking station, and three tool cleaning booths. | EU-3IMCPU1 EU-3IMCPU2 EU-3IMCPU3 EU-3MOLDRELEASE EU-3MASKSTATION EU-3TOOLCLEAN1 EU-3TOOLCLEAN2 EU-3TOOLCLEAN3 |
| FG-MACTPPPP | EU-1IMCPU1, EU-1IMCPU2, EU-1IMCPU3, EU-2IMCPU1, EU-2IMCPU2, EU-2IMCPU3, EU-3IMCPU1, EU-3IMCPU2, EU-3IMCPU3, EU-AutoPlasCoatLn as well as associated activities, such as surface preparation, cleaning, mixing, and storage if they are directly related to the application of the coating. | EU-1IMCPU1 EU-1IMCPU2 EU-1IMCPU3 EU-2IMCPU1 EU-2IMCPU2 EU-2IMCPU3 EU-3IMCPU1 EU-3IMCPU2 EU-3IMCPU3 EU-AutoPlasCoatLn |
| FG-MACT-ZZZZ-EMERGENCY RICE | This flexible group includes existing "certified" emergency stationary reciprocating internal combustion engines (RICE) that have a maximum site rating of 500 brake horsepower (HP) and less than 30 liters per cylinder located at either an area or a major source of hazardous air pollutants (HAPs). | EU-Pump612 EU-Pump613 EU-Pump614 EU-WetWellPump EU-Generac |
| FG-BOILERMACT | This flexible group only includes those applicable requirements that would apply to new and existing affected boilers or process heaters as described in 40 CFR Part 63, Subpart DDDDD beginning on January 31, 2016, as they would apply to Faurecia Interior System Saline, LLC located in Saline, MI. A new unit commenced construction or reconstruction after June 4, 2010. A boiler or process heater is existing if it is not new or reconstructed. | EU-166-77 EU-403-74 |
| FG-RULE287(2)(c) | Any emission unit that emits air contaminants and is exempt from the requirements of Rule 201 pursuant to Rules 278, 278a and 287(2)(c). Emission units installed/modified before December 20, 2016, may show compliance with Rule 287 in effect at the time of installation/modification. | EU-RULE287(2)(c) |
| FG-RULE290 | Any emission unit that emits air contaminants and is exempt from the requirements of Rule 201 pursuant to Rules 278, 278a and 290. Emission units installed/modified before December 20, 2016, may show compliance with Rule 290 in effect at the time of installation/modification. | EU-GlueRobot EU-CastSkins |
| FG-COLDCLEANERS | Any cold cleaner that is grandfathered or exempt from Rule 201 pursuant to Rule 278, 278a and Rule 281(2)(h) or Rule 285(2)(r)(iv). Existing cold cleaners were placed into operation prior to July 1, 1979. New cold cleaners were placed into operation on or after July 1, 1979. | EU-COLDCLEANERS |

## FG-BOILERS

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Two natural gas-fired steam boilers with fuel oil burning capabilities.

Emission Units:  EU-403-74, EU-166-77 .

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  Report shall be postmarked or received by appropriate AQD district office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  Report shall be postmarked or received by appropriate AQD district office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-IMCPULINES1&2

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Two in-mold coating/polyurethane coating processes.  Each process consists of three individual coating lines, a single mold release booth, a single masking station, and, a single tool cleaning booth.

Emission Units:  EU-1IMCPU1, EU-1IMCPU2, EU-1IMCPU3, EU-1MOLDRELEASE, EU-1MASKSTATION,

EU-1TOOLCLEAN, EU-2IMCPU1, EU-2IMCPU2, EU-2IMCPU3, EU-2MOLDRELEASE, EU-2MASKSTATION,

EU-2TOOLCLEAN

POLLUTION CONTROL EQUIPMENT

Dry filters on all spray booths

I.  EMISSION LIMIT(S)

| Pollutant | Limit | Time Period | Equipment | Testing/ Monitoring Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- | --- |
| 1. PM2.5 | 0.075 pph2 | Hourly | Each manual IMC spray booth portion of the following emission units: EU-1IMCPU1, EU-1IMCPU2, EU-1IMCPU3, EU-2IMCPU1, EU-2IMCPU2, EU-2IMCPU3 | SC V.3, SC VI.1 & VI.2 | R 336.1205(1)(a) |
| 2. PM 2.5 | 0.075 pph2 | Hourly | Each automatic (robotic) IMC spray booth portion of the following emission units: EU-1IMCPU1, EU-1IMCPU2, EU-1IMCPU3, EU-2IMCPU1 , EU-2IMCPU2, EU-2IMCPU3 | SC V.3, SC VI.1 & VI.2 | R 336.1205(1)(a) |
| 3. PM 2.5 | 0.05 pph2 | Hourly | Each PU spray booth portion of the following emission units: EU-1IMCPU1, EU-1IMCPU2, EU-1IMCPU3, EU-2IMCPU1, EU-2IMCPU2, EU-2IMCPU3 | SC V.3, SC VI.1 & VI.2 | R 336.1205(1)(a) |
| 4. PM10 | 0.1 pph2 | Hourly | Each manual IMC spray booth portion of the following emission units: EU-1IMCPU1, EU-1IMCPU2, EU-1IMCPU3, EU-2IMCPU1, EU-2IMCPU2, EU-2IMCPU3 | SC V.3, SC VI.1 & VI.2 | R 336.1205(1)(a) |
| 5. PM10 | 0.1 pph2 | Hourly | Each automatic (robotic) IMC spray booth portion of the following emission units: EU-1IMCPU1, EU-1IMCPU2, EU-1IMCPU3, EU-2IMCPU1, EU-2IMCPU2, EU-2IMCPU3 | SC V.3, SC VI.1 & VI.2 | R 336.1205(1)(a) |
| 6. PM10 | 0.1 pph2 | Hourly | Each PU spray booth portion of the following emission units: EU-1IMCPU1. EU-1IMCPU2, EU-1IMCPU3, EU-2IMCPU1, EU-2IMCPU2, EU-2IMCPU3 | SC V.3, SC VI.1 & VI.2 | R 336.1205(1)(a) |
| 7. VOCs | 35.9 tpy2 | 12-month rolling time period as determined at the end of each calendar month | All lines within FG-IMCPULINES1&2 combined | SC V.1, SC VI.4 & VI.3 | R 336.1205(1)(a) R 336.1205(3) R 336.1702(a) |
| 8. VOCs | 369 lb/day2 | Calendar Day | All lines within FG-IMCPULINES1&2 combined | SC V.1, SC VI.4 & VI.3 | R 336.1205(1)(a) |

II.  MATERIAL LIMIT(S)

| Pollutant | Limit | Time Period | Equipment | Testing/ Monitoring Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- | --- |
| 1. VOCs | 0.72 lb/gal (minus water)a as applied2 | Instantaneous | Each coating line within FG-IMCPULINES1&2 | SC V.1, SC VI.3 & VI.4 | R 336.1702(a) |
| a The phrase “minus water” shall also include compounds which are used as organic solvents and which are excluded from the definition of volatile organic compound. (R 336.1602(4)) |  |  |  |  |  |

| Material | Equipment | MDI* Emission Factor | Testing/ Monitoring Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- |
| 2. Polyurethane (PU) Coatings | Each PU spray booth associated with FG-IMCPULINES1&2 | 0.005% of the MDI in each material1 | SC V.2 | R 336.1225 |
| *MDI (CAS No. 101-68-8) = Methylene diphenyl isocyanate |  |  |  |  |

III.  PROCESS/OPERATIONAL RESTRICTION(S)

1.	The permittee shall not use any VOC containing materials in any mask station associated with 
FG-IMCPULINES1&2.2  (R 336.1225, R 336.1702(a))

2.	The permittee shall recover and reclaim, recycle, or dispose of, in accordance with all applicable regulations, a minimum of 90 percent by weight of all purge/cleanup solvents used for each coating line within 
FG-IMCPULINES1&2.2  (R 336.1225, R 336.1702(a))

3.	The permittee shall capture all waste materials (i.e. coatings, cleanup solvent(s), mold release, etc.) and shall store them in closed containers.  The permittee shall dispose of all waste materials (i.e. coatings, cleanup solvent(s), mold release, etc.) in an acceptable manner in compliance with all applicable state rules and federal regulations.2  (R 336.1224, R 336.1702(a))

4.	The permittee shall dispose of spent filters in a manner which minimizes the introduction of air contaminants to the outer air.2  (R 336.1224, R 336.1370)

5.	The permittee shall handle all VOC containing materials, including coatings, reducers, solvents and thinners, in a manner to minimize the generation of fugitive emissions.  The permittee shall keep containers covered at all times except when operator access is necessary.2  (R 336.1205(3), R 336.1224, R 336.1225, R 336.1702(a), R 336.1901)

IV.  DESIGN/EQUIPMENT PARAMETER(S)

1.	The permittee shall not operate any spray booth within FG-IMCPULINES1&2 unless all respective exhaust filters are installed, maintained and operated in a satisfactory manner.2  (R 336.1224, R 336.1301, R 336.1331, R 336.1901, R 336.1910)

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii), 40 CFR 63.4531)

- The permittee shall determine the VOC content, water content and density of any coating, as applied and as received, using federal Reference Test Method 24.  Upon prior written approval by the AQD District Supervisor, the permittee may determine the VOC content from manufacturer's formulation data.  If the Method 24 and the formulation values should differ, the permittee shall use the Method 24 results to determine compliance.2  (R 336.1225, R 336.1702(a), R 336.1901, R 336.2001, R 336.2003, R 336.2004, R 336.2040(5))

- The permittee may be required to verify the MDI emission rate and the associated MDI emission factor identified in SC II.2 from the polyurethane booths associated with FG-IMCPULINES1&2 by testing at owner’s expense, in accordance with Department requirements.  No less than 30 days prior to testing, the permittee shall submit a complete test plan to the AQD.  The AQD must approve the final plan prior to testing.  Verification of emission rates includes the submittal of a complete report of the test results to the AQD within 60 days following the last date of the test.2  (R 336.1213(3), R 336.1225, R 336.2001, R 336.2003, R 336.2004)

- The permittee may be required to verify PM10/PM2.5 emission rates from FG-IMCPULINES1&2 by testing at owner’s expense, in accordance with Department requirements. Testing shall be performed using an approved EPA Method listed in 40 CFR Part 51, Appendix M. An alternate method, or a modification to the approved EPA Method, may be specified in an AQD-approved Test Protocol.  No less than 30 days prior to testing, the permittee shall submit a complete test plan to the AQD Technical Programs Unit and District Office.  The AQD must approve the final plan prior to testing, including any modifications to the method in the test protocol that are proposed after initial submittal.  Verification of emission rates includes the submittal of a complete report of the test results to the AQD Technical Programs Unit and District Office within 60 days following the last date of the test.2  (R 336.1205, R 336.1299, R 336.1702, R 336.2001, R 336.2003, R 336.2004)

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii), 40 CFR 63.4531)

1.	The permittee shall perform weekly inspections and monitor operating information of the fabric filters used for particulate control in each spray booth associated with FG-IMCPULINES1&2, except for EU-1MOLDRELEASE, EU-1MASKSTATION, EU-2MOLDRELEASE, EU-2MASKSTATION.  The permittee shall keep inspection and operating information records on file at the facility and make them available to the Department upon request.2  (R 336.1205, R 336.1331)

2.	The permittee shall complete all required calculations in a format acceptable to the AQD District Supervisor by the 15th day of the calendar month, for the previous calendar month, unless otherwise specified in any recordkeeping/reporting special condition.2  (R 336.1205, R 336.1224, R 336.1225, R 336.1299, R 336.1702, R 336.1901)

3.	The permittee shall maintain a current listing from the manufacturer of the chemical composition of each material (coating, mold release, cleanup solvent, etc.), including the weight percent of each component.  The data may consist of Material Safety Data Sheets, manufacturer’s formulation data, or both as deemed acceptable by the AQD District Supervisor.  The permittee shall keep all records on file and make them available to the Department upon request.2  (R 336.1225, R 336.1299, R 336.1702, R 336.1901)

4.	The permittee shall keep the following information on a monthly basis for each coating line within 
FG-IMCPULINES1&2:

a.	Gallons (with water) of each material (i.e. coating, mold release, cleanup solvent, etc.) used.

b.	VOC content (minus water and with water) of each material (i.e. coating, mold release, cleanup solvent, etc.) as applied.

c.	Gallons of cleanup solvent reclaimed.

d.	VOC mass emission calculations determining the monthly emission rate in tons per calendar month for IMC/PU Systems 1 and 2 individually, and combined for both IMC/PU Systems within 
FG-IMCPULINES1&2.

e.	VOC mass emission calculations determining the annual emission rate in tons per 12-month rolling time period as determined at the end of each calendar month for IMC/PU Systems 1 and 2 individually, and combined for both IMC/PU Systems within FG-IMCPULINES1&2.

The permittee shall keep the records in a format acceptable to the AQD District Supervisor.  The permittee shall keep all records on file and make them available to the Department upon request.2  (R 336.1225, R 336.1299, R 336.1702(a), R 336.1901)

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the tables below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| 1. SV-1MOLDRELEASE | 182 | 322 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |
| 2. SV-1TOOLCLEAN | 182 | 322 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |
| 3. SV-1ROBOTICIMC1 SV-1ROBOTICIMC2 SV-1ROBOTICIMC3 | 18 for each stack2 | 38 for each stack2 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |
| 4. SV-1MANUALIMC1 SV-1MANUALIMC2 SV-1MANUALIMC3 | 18 for each stack2 | 38 for each stack2 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |
| 5. SV-1FLASHOFF1 SV-1FLASHOFF2 SV-1FLASHOFF3 | 18 for each stack2 | 32 for each stack2 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |
| 6. SV-1PUBOOTH1 SV-1PUBOOTH2 SV-1PUBOOTH3 | 18 for each stack2 | 38 for each stack2 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |
| 7. SV-1CURE1 SV-1CURE2 SV-1CURE3 | 20 for each stack2 | 32 for each stack2 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |
| 8. SV-2MOLDRELEASE | 182 | 322 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |
| 9. SV-2TOOLCLEAN | 182 | 322 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |
| 10. SV-2ROBOTICIMC1 SV-2ROBOTICIMC2 SV-2ROBOTICIMC3 | 18 for each stack2 | 38 for each stack2 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |
| 11. SV-2MANUALIMC1 SV-2MANUALIMC2 SV-2MANUALIMC3 | 18 for each stack2 | 38 for each stack2 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |
| 12. SV-2FLASHOFF1 SV-2FLASHOFF2 SV-2FLASHOFF3 | 18 for each stack2 | 32 for each stack2 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |
| 13. SV-2PUBOOTH1 SV-2PUBOOTH2 SV-2PUBOOTH3 | 18 for each stack2 | 38 for each stack2 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |
| 14. SV-2CURE1 SV-2CURE2 SV-2CURE3 | 20 for each stack2 | 32 for each stack2 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |

15. The exhaust gases from EU-1MASKSTATION and EU-2MASKSTATION shall not be discharged to the ambient air at any time.2  (R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d))

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-IMCPULINES3

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

In-mold coating/polyurethane coating processes consisting of three individual coating lines, a single mold release booth, a single masking station, and three tool cleaning booths.

Emission Units:  EU-3IMCPU1, EU-3IMCPU2, EU-3IMCPU3, EU-3MOLDRELEASE, EU-3MASKSTATION,

EU-3TOOLCLEAN1, EU-3TOOLCLEAN2 EU-3TOOLCLEAN3

POLLUTION CONTROL EQUIPMENT

Dry filters on all spray booths

I.  EMISSION LIMIT(S)

| Pollutant | Limit | Time Period | Equipment | Testing/ Monitoring Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- | --- |
| 1. PM2.5 | 0.0752 | Hourly | Each manual IMC spray booth portion of the following emission units: EU-3IMCPU1, EU-3IMCPU2, EU-3IMCPU3 | SC V.3, SC VI.1 & VI.2 | R 336.1205(1)(a) |
| 2. PM 2.5 | 0.075 pph2 | Hourly | Each automatic (robotic) IMC spray booth portion of the following emission units: EU-3IMCPU1, EU-3IMCPU2, EU-3IMCPU3 | SC V.3, SC VI.1 & VI.2 | R 336.1205(1)(a) |
| 3. PM 2.5 | 0.05 pph2 | Hourly | Each PU spray booth portion of the following emission units: EU-3IMCPU1, EU-3IMCPU2, EU-3IMCPU3 | SC V.3, SC VI.1 & VI.2 | R 336.1205(1)(a) |
| 4. PM10 | 0.1 pph2 | Hourly | Each manual IMC spray booth portion of the following emission units: EU-3IMCPU1, EU-3IMCPU2, EU-3IMCPU3 | SC V.3, SC VI.1 & VI.2 | R 336.1205(1)(a) |
| 5. PM10 | 0.1 pph2 | Hourly | Each automatic (robotic) IMC spray booth portion of the following emission units: EU-3IMCPU1, EU-3IMCPU2, EU-3IMCPU3 | SC V.3, SC VI.1 & VI.2 | R 336.1205(1)(a) |
| 6. PM10 | 0.1 pph2 | Hourly | Each PU spray booth portion of the following emission units: EU-3IMCPU1, EU-3IMCPU2, EU-3IMCPU3 | SC V.3, SC VI.1 & VI.2 | R 336.1205(1)(a) |
| 7. VOCs | 23.6 tpy2 | 12-month rolling time period as determined at the end of each calendar month | FG-IMCPULINES3 | SC V.1, SC VI.3 & VI.4 | R 336.1205(1)(a), R 336.1205(3), R 336.1702(a) |
| 8. VOCs | 118 lb/day2 | Calendar Day | FG-IMCPULINES3 | SC V.1, SC VI.3 & VI.4 | R 336.1205(1)(a) |

II.  MATERIAL LIMIT(S)

| Pollutant | Limit | Time Period | Equipment | Testing/ Monitoring Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- | --- |
| 1. VOCs | 0.72 lb/gal (minus water)a as applied2 | Instantaneous | FG-IMCPULINES3 | SC V.1 | R 336.1702(a) |
| a The phrase “minus water” shall also include compounds which are used as organic solvents and which are excluded from the definition of volatile organic compound. (R 336.1602(4)) |  |  |  |  |  |

| Material | Equipment | MDI* Emission Factor | Testing/ Monitoring Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- |
| 2. Polyurethane (PU) Coatings | Each PU spray booth associated with FG-IMCPULINES1&2 | 0.005% of the MDI in each material1 | SC V.2 | R 336.1225 |
| *MDI (CAS No. 101-68-8) = Methylene diphenyl isocyanate |  |  |  |  |

III.  PROCESS/OPERATIONAL RESTRICTION(S)

- The permittee shall not use any VOC-containing materials in any mask station associated with 
FG-IMCPULINES3.2  (R 336.1225, R 336.1702(a))

- The permittee shall recover and reclaim, recycle, or dispose of, in accordance with all applicable regulations, a minimum of 90 percent by weight of all purge/cleanup solvents used for each coating line within 
FG-IMCPULINES3.2  (R 336.1225, R 336.1702(a))

- The permittee shall capture all waste materials (i.e. coatings, cleanup solvent(s), mold release, etc.) and shall store them in closed containers.  The permittee shall dispose of all waste materials (i.e. coatings, cleanup solvent(s), mold release, etc.) in an acceptable manner in compliance with all applicable state rules and federal regulations.2  (R 336.1224, R 336.1702(a))

- The permittee shall dispose of spent filters in a manner which minimizes the introduction of air contaminants to the outer air.2  (R 336.1224, R 336.1370)

- The permittee shall handle all VOC containing materials, including coatings, reducers, solvents and thinners, in a manner to minimize the generation of fugitive emissions.  The permittee shall keep containers covered at all times except when operator access is necessary.2  (R 336.1205(3), R 336.1224, R 336.1225, R 336.1702(a), R 336.1901)

IV.  DESIGN/EQUIPMENT PARAMETER(S)

1. The permittee shall not operate any spray booth within FG-IMCPULINES3 unless all respective exhaust filters are installed, maintained and operated in a satisfactory manner.2  (R 336.1224, R 336.1301, R 336.1331, R 336.1901, R 336.1910)

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii), 40 CFR 63.4531)

1.  The permittee shall determine the VOC content, water content and density of any coating, as applied and as received, using federal Reference Test Method 24.  Upon prior written approval by the AQD District Supervisor, the permittee may determine the VOC content from manufacturer’s formulation data.  If the Method 24 and the formulation values should differ, the permittee shall use the Method 24 results to determine compliance.2  (R 336.1205, R 336.1225, R 336.1702(a), R 336.1901, R 336.2001, R 336.2003, R 336.2004, R 336.2040(5))

2.	The permittee may be required to verify the MDI emission rate and the associated MDI emission factor identified in SC II.2 from the polyurethane booths associated with FG-IMCPULINES3 by testing at owner’s expense, in accordance with Department requirements.  No less than 30 days prior to testing, the permittee shall submit a complete test plan to the AQD.  The AQD must approve the final plan prior to testing.  Verification of emission rates includes the submittal of a complete report of the test results to the AQD within 60 days following the last date of the test.2  (R 336.1213(3), R 336.1225, R 336.2001, R 336.2003, R 336.2004)

3.	The permittee may be required to verify PM10/PM2.5 emission rates from FG-IMCPULINES3 by testing at owner’s expense, in accordance with Department requirements.  Testing shall be performed using an approved EPA Method listed in 40 CFR Part 51, Appendix M.  An alternate method, or a modification to the approved EPA Method, may be specified in an AQD-approved Test Protocol.  No less than 30 days prior to testing, the permittee shall submit a complete test plan to the AQD Technical Programs Unit and District Office.  The AQD must approve the final plan prior to testing, including any modifications to the method in the test protocol that are proposed after initial submittal.  Verification of emission rates includes the submittal of a complete report of the test results to the AQD Technical Programs Unit and District Office within 60 days following the last date of the test.2  (R 336.1205, R 336.1299, R 336.1702, R 336.2001, R 336.2003, R 336.2004)

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii), 40 CFR 63.4531)

- The permittee shall perform weekly inspections and monitor operating information of the fabric filters used for particulate control in each spray booth associated with FG-IMCPULINES3, except for EU-3MOLDRELEASE, EU-3MASKSTATION.  The permittee shall keep inspection and operating information records on file at the facility and make them available to the Department upon request.2  (R 336.1205, R 336.1331)

- The permittee shall complete all required calculations in a format acceptable to the AQD District Supervisor by the 15th day of the calendar month, for the previous calendar month, unless otherwise specified in any monitoring/recordkeeping special condition.2  (R 336.1205, R 336.1224, R 336.1225, R 336.1702, R 336.1901)

- The permittee shall maintain a current listing from the manufacturer of the chemical composition of each material (coating, mold release, cleanup solvent, etc.), including the weight percent of each component.  The data may consist of Material Safety Data Sheets, manufacturer’s formulation data, or both as deemed acceptable by the AQD District Supervisor.  The permittee shall keep all records on file and make them available to the Department upon request.2  (R 336.1224, R 336.1225, R 336.1702, R 336.1901)

- The permittee shall keep the following information on a daily basis for each coating line within 
FG-IMCPULINES3:

- Gallons (with water) of each material (i.e. coating, mold release, cleanup solvent, etc.) used.

- VOC content (minus water and with water) of each material (i.e. coating, mold release, cleanup solvent, etc.) as applied.

- Gallons of cleanup/purge solvent reclaimed.

- VOC mass emission calculations determining the monthly emission rate in tons per calendar month for all coating lines combined within FG-IMCPULINES3.

- VOC mass emission calculations determining the annual emission rate in tons per 12-month rolling time period as determined at the end of each calendar month for all coating lines combined within 
FG-IMCPULINES3.

The permittee shall keep the records in a format acceptable to the AQD District Supervisor. The permittee shall keep all records on file and make them available to the Department upon request.2  (R 336.1205, R 336.1225, R 336.1299, R 336.1702, R 336.1901)

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the tables below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| 1. SV-3MOLDRELEASE | 182 | 322 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |
| 2. SV-3TOOLCLEAN1 | 182 | 322 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |
| 3. SV-3TOOLCLEAN2 | 182 | 322 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |
| 4. SV-3TOOLCLEAN3 | 182 | 322 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |
| 5. SV-3ROBOTICIMC11 SV-3ROBOTICIMC12 SV-3ROBOTICIMC13 | 18 for each stack2 | 38 for each stack2 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |
| 6. SV-3MANUALIMC1 SV-3MANUALIMC2 SV-3MANUALIMC3 | 18 for each stack2 | 38 for each stack2 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |
| 7. SV-3ROBOTICIMC21 SV-3ROBOTICIMC22 SV-3ROBOTICIMC23 | 18 for each stack2 | 38 for each stack2 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |
| 8. SV-3FLASH-OFF1 SV-3FLASH-OFF2 SV-3FLASH-OFF3 | 18 for each stack2 | 32 for each stack2 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |
| 9. SV-3PUBOOTH1 SV-3PUBOOTH2 SV-3PUBOOTH3 | 18 for each stack2 | 38 for each stack2 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |
| 10. SV-3CURE1 SV-3CURE2 SV-3CURE3 | 18 for each stack2 | 32 for each stack2 | R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d) |

11.  The exhaust gases from EU-3MASKSTATION shall not be directly discharged to the ambient air at any time.2  (R 336.1225, R 336.1901, R 336.2803, R 336.2804, 40 CFR 52.21(c) & (d))

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-MACTPPPP

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Each new, reconstructed, and existing affected source engaged in the surface coating of plastic parts and products, identified within each of the four subcategories listed in 40 CFR Part 63, Subpart PPPP, 40 CFR 63.4481(a)(2) to (5).  Surface coating is defined by 40 CFR 63.4481 as the application of coating to a substrate using, for example, spray guns or dip tanks.  Surface coating also includes associated activities, such as surface preparation, cleaning, mixing, and storage if they are directly related to the application of the coating.  This facility chooses to comply with Subpart PPPP using the compliant materials option written in 40 CFR 63.4491(a), and at this time the compliance option under 40 CFR 63.4491(c) for add-on controls have been removed from this flexible group.

Emission Units:  EU-1IMCPU1, EU-1IMCPU2, EU-1IMCPU3, EU-2IMCPU1, EU-2IMCPU2, EU-2IMCPU3, 
EU-3IMCPU1, EU-3IMCPU2, EU-3IMCPU3, EU-AutoPlasCoatLn

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

| Pollutant | Limit | Time Period/ Operating Scenario | Equipment | Monitoring/ Testing Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- | --- |
| 1. Organic HAP | 0.16 lb per lb of coating solids2 | 12-month rolling time period * | Existing - General Use Coating | SC V.1, V.2, SC VI.1 through VI.6 | 40 CFR 63.4490(b)(1) |
| * As determined at the end of each calendar month. |  |  |  |  |  |

2.	The permittee shall determine whether the organic HAP emission rate is equal to or less than the applicable emission limits in 40 CFR 63.4490 using at least one of the following three options, which are listed in 40 CFR 63.4491(a) through (c):

a.	Compliant material option,

b.	Emission rate without add-on controls option, or

c.	Emission rate with add-on controls option.

The permittee shall include all coatings, thinners and/or other additives, and cleaning materials used when determining the emission rate.2  (40 CFR 63.4491)

3.	Any coating operation(s) using the compliant material option or the emission rate without add-on controls option shall be in compliance with the applicable emission limits in 40 CFR 63.4490 at all times.2  (40 CFR 63.4500(a)(1))

4.	If the surface coating operation(s) meet the applicability criteria of more than one of the subcategory emission limits specified in 40 CFR 63.4490(a) or (b), the permittee may comply separately with each subcategory emission limit, or comply using one of the alternatives in 40 CFR 63.4490(c)(1) or (2).2  (40 CFR 63.4490(c))

II.  MATERIAL LIMIT(S)

For the compliant materials option, the permittee shall meet the material limits specified in the following table.

| Material | Limit | Time Period/ Operating Scenario | Equipment | Monitoring/ Testing Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- | --- |
| 1. Each Thinner and/or Additive | No Organic HAP 2 * | Continuous | Each Coating Operation using Compliant Material Option | SC VI.1, VI.2, VI.3, & VI.5 | 40 CFR 63.4491(a) |
| 2. Each Cleaning Material | No Organic HAP 2 * | Continuous | Each Coating Operation using Compliant Material Option | SC VI.1, VI.2, VI.3, & VI.5 | 40 CFR 63.4491(a) |
| * Determined according to 40 CFR 63.4541(a). |  |  |  |  |  |

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii), 40 CFR 63.4531)

1. 	The permittee shall determine the mass fraction of organic HAP for each material used, the mass fraction of coating solids for each coating, and the density of each material used in accordance with 40 CFR 63.4541, 40 CFR 63.4551, and/or 40 CFR 63.4561.2  (40 CFR 63.4541, 40 CFR 63.4551, 40 CFR 63.4561)

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii), 40 CFR 63.4531)

1.	The permittee shall conduct an initial compliance demonstration for the initial compliance period according to the requirements in 40 CFR 63.4541, 40 CFR 63.4551, or 40 CFR 63.4561.  The initial compliance period began on April 19, 2007 and ended April 30, 2008.2  (40 CFR 63.4483, 40 CFR 63.4540, 40 CFR 63.4550, 40 CFR 63.4560)

2.	The permittee shall keep all records required by 40 CFR 63.4530 in the format and timeframes outlined in 40 CFR 63.4531.2  (40 CFR 63.4542(d), 40 CFR 63.4552(d), 40 CFR 63.4563(j))

3.	The permittee shall maintain, at a minimum, the following records for each compliance period:

a.	A copy of each notification and report that is submitted to comply with 40 CFR Part 63, Subpart PPPP, and the documentation supporting each notification report.2  (40 CFR 63.4530(a))

b.	A current copy of information provided by materials suppliers or manufacturers, such as manufacturer’s formulation data, or test data used to determine the mass fraction of organic HAP and density of each coating, thinner and/or other additive, and cleaning material, and the mass fraction of coating solids for each coating.2  (40 CFR 63.4530(b))

c.	A list of the coating operations on which each compliance option was used, and the beginning and ending dates and times for each compliance option used.2  (40 CFR 63.4530(c)(1))

d.	For the compliant materials option, the calculation of the organic HAP content for each coating, using Equation 1 of 40 CFR 63.4541.2  (40 CFR 63.4530(c)(2))

e.	For the emission rate without add-on controls option, the calculation of the total mass of organic HAP emissions for the coatings, thinners and/or additives, and cleaning materials used each month using Equations 1, 1A through 1C and 2 of 40 CFR 63.4551; and, if applicable, the calculation used to determine mass of organic HAP in waste materials according to 40 CFR 63.4551(e)(4); the calculation of the total mass of coating solids used each month using Equation 2 of 40 CFR 63.4551; and the calculation of each 12-month organic HAP emission rate using Equation 3 of 40 CFR 63.4551.2  (40 CFR 63.4530(c)(3))

f.	The name and mass or volume of each coating, thinner and/or other additive, and cleaning material used during each compliance period.  If the compliant material option is used for all coatings at the affected source, the permittee may maintain purchase records for each material used rather than a record of the mass used.2  (40 CFR 63.4530(d))

g.	The mass fraction of organic HAP for each coating, thinner and/or additive, and cleaning material used during each compliance period.2  (40 CFR 63.4530(e))

h.	The mass fraction of coating solids for each coating used during each compliance period.2  (40 CFR 63.4530(f))

i.	The information specified in 40 CFR 63.4530(g)(1) through (3), if an allowance is used in Equation 1 of 40 CFR 63.4551 for organic HAP contained in waste materials sent to or designated for shipment to a treatment, storage, and disposal facility (TSDF) according to 40 CFR 63.4551(e)(4).2  (40 CFR 63.4530(g))

j.	The date, time, and duration of each deviation.2  (40 CFR 63.4530(h))

- The permittee shall include all coatings, thinners, and/or other additives, and cleaning materials used in the affected source when determining whether the organic HAP emission rate is equal to or less than the applicable emission limit in 40 CFR 63.4490.  (40 CFR 63.4491)

5.	For each coating used for the compliant coating option, the permittee shall demonstrate continuous compliance with the emission limit in 40 CFR 63.4490, for each compliance period, using Equation 1 of 40 CFR 63.4541.  For each thinner and cleaning material used, the permittee shall determine continuous compliance according to 40 CFR 63.4541(a).2  (40 CFR 63.4542)

6.	For any coating operation or group of coating operations using the emission rate without the add-on controls option, the permittee shall demonstrate continuous compliance with the applicable organic HAP emission limit in 40 CFR 63.4490, for each compliance period according to 40 CFR 63.4551(a) through (g).2  (40 CFR 63.4552)

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

4.	For the compliant material option, the use of any coating, thinner or cleaning material which does not meet the criteria specified in 40 CFR 63.4542(a) is a deviation that must be reported as specified in 40 CFR 63.4510(c)(6) and 40 CFR 63.4520(a)(5).2  (40 CFR 63.4542(b))

5.	For the emission rate without add-on controls, if the organic HAP emission rate for any compliance period exceeds the applicable emission limit specified in 40 CFR 63.4490, the permittee shall report this as a deviation as specified in 40 CFR 63.4510(c)(6) and 40 CFR 63.4520(a)(6).2  (40 CFR 63.4552(b))

6.	The permittee shall submit the applicable notifications specified in 40 CFR 63.7(b) and (c), 40 CFR 63.8(f)(4) and 40 CFR 63.9(b) through (e) and (h), an initial notification and a notification of compliance status as specified in 40 CFR 63.4510.2  (40 CFR Part 63, Subparts A and PPPP)

7.	The permittee shall submit all semiannual compliance reports as required by 40 CFR 63.4520.  Each semiannual compliance report shall identify which coating operation(s) used each compliance option, and if there were no deviations from the emission limitations in 40 CFR 63.4490, include a statement that the coating operations were in compliance.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.2  (40 CFR 63.4520, 40 CFR 63.4542(c), 40 CFR 63.4552(c), 40 CFR 63.4563(f))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| NA | NA | NA | NA |

IX.  OTHER REQUIREMENT(S)

1.	The permittee shall comply with all applicable provisions of the National Emission Standards for Hazardous Air Pollutants, as specified in 40 CFR Part 63, Subpart A and Subpart PPPP for Surface Coating of Plastic Parts and Products by the initial compliance date.2  (40 CFR, Part 63, Subparts A and PPPP)

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-MACT-ZZZZ-EMERGENCY RICE

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Existing CI and SI engines at an area source, Emergency and Black Start

Compliance date – May 3, 2013 for CI Engines

Compliance date – October 19, 2013 for SI Engines

Emission Units:  EU-Pump612, EU-Pump613, EU-Pump614, EU-WetWellPump, EU-Generac

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

- The permittee shall operate and maintain any affected CI RICE, including associated air pollution control equipment and monitoring equipment, in a manner consistent with safety and good air pollution control practices for minimizing emissions.  The general duty to minimize emissions does not require you to make any further efforts to reduce emissions if levels required by this standard have been achieved.  Determination of whether such operation and maintenance procedures are being used will be based on information available to the Administrator which may include, but is not limited to, monitoring results, review of operation and maintenance procedures, review of operation and maintenance records, and inspection of the source.  (40 CFR 63.6605(b))

- The permittee shall comply with the following requirements, except during periods of startup: (40 CFR 63.6603(a))

For CI Engines: (40 CFR 63.6603(a), Table 2d, item 4)

  - Change oil and filter every 500 hours of operation or annually, whichever comes first, except as allowed in SC III.4.

  - Inspect air cleaner every 1,000 hours of operation or annually, whichever comes first.

  - Inspect all hoses and belts every 500 hours of operation or annually, whichever comes first, and replace as necessary.

For SI Engines: (40 CFR 63.6603(a), Table 2d, item 5)

  - Change oil and filter every 500 hours of operation or annually, whichever comes first, except as allowed in SC III.4.

  - Inspect spark plugs every 1,000 hours of operation or annually, whichever comes first.

  - Inspect all hoses and belts every 500 hours of operation or annually, whichever comes first, and replace as necessary.

- The permittee shall operate and maintain the stationary RICE and after-treatment control device (if any) according to the manufacturer’s emission-related written instructions or develop your own maintenance plan which must provide to the extent practicable for the maintenance and operation of the engine in a manner consistent with good air-pollution control practice for minimizing emissions.  (40 CFR 62.6625(e), 40 CFR 63.6640(a), Table 6, item 9)

- The permittee may utilize an oil analysis program in order to extend the specified oil change requirement in 40 CFR 63.6603(a) and as listed in SC III.2.  The oil analysis program must be performed at the same frequency as oil changes are required.  The analysis program must analyze the parameters and keep records as required in 40 CFR 63.6625(i).  (40 CFR 63.6625(i))

- The permittee shall not allow the CI engine(s) to exceed 100 hours for Maintenance checks and readiness testing.  The owner or operator may petition the Administrator for approval of additional hours to be used for maintenance checks and readiness testing, but a petition is not required if the owner or operator maintains records indicating that Federal, State, or local standards require maintenance and testing of emergency RICE beyond 100 hours per year.  (40 CFR 63.6640(f)(ii))

- The permittee shall not allow the CI engine(s) to operate more than 50 hours per year for non-emergency situations, as allowed in 40 CFR 63.6640(f)(iii).  (40 CFR 63.6640(f)(iii))

IV.  DESIGN/EQUIPMENT PARAMETER(S)

- The permittee shall equip and maintain the CI engines with a non-resettable hour meter.  (40 CFR 63.6625(f))

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- If using the oil analysis program for CI Engine(s), the permittee shall test for Total Base Number, viscosity and percent water content.  (40 CFR 63.6625(i))

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- For each CI engine, the permittee shall keep records of the occurrence and duration of each malfunction of operation (i.e., process equipment) or the air pollution control and monitoring equipment.  (40 CFR 63.6655(a)(2), 40 CFR 63.6660)

- The permittee shall keep records of all required maintenance performed on the air pollution control and monitoring equipment.  (40 CFR 63.6655(a)(4), 40 CFR 63.6660)

- The permittee shall keep records of actions taken during periods of malfunction to minimize emissions in accordance with 40 CFR 63.6605(b), including corrective actions to restore malfunctioning process and air pollution control and monitoring equipment to its normal or usual manner of operation.  (40 CFR 63.6655(a)(5), 40 CFR 63.6660)

- The permittee shall keep records as required in SC III.3 to show continuous compliance with each emission or operating limit that applies.  (40 CFR 63.6655(d), 40 CFR 63.6660)

- The permittee shall keep records of the maintenance conducted on the stationary RICE in order to demonstrate that the permittee operated and maintained the stationary RICE and after-treatment control device (if any) according to the permittee’s maintenance plan.  (40 CFR 63.6655(e), 40 CFR 63.6660)

- The permittee shall keep records of the hours of operation of the engine that is recorded through the non-resettable hour meter.  The owner or operator must document how many hours are spent for emergency operation, including what classified the operation as emergency and how many hours are spent for non-emergency operation.  (40 CFR 63.6655(f), 40 CFR 63.6660)

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

1.	The permittee shall comply with all applicable provisions of the National Emission Standards for Hazardous Air Pollutants, as specified in 40 CFR Part 63, Subpart A and Subpart ZZZZ, as they apply to FG-MACT-ZZZZ-EMERGENCY RICE. The permittee may choose an alternative compliance method not listed in FG-MACT-ZZZZ-EMERGENCY RICE by complying with all applicable provisions required by Subpart ZZZZ for the compliance option chosen.  (40 CFR 70.6(9), 40 CFR 63.9(j), 40 CFR Part 63, Subparts A and ZZZZ)

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-BOILERMACT

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Gas 1 Fuel Subcategory requirements for existing Boilers/Process Heaters at major sources of Hazardous Air Pollutants per 40 CFR Part 63, Subpart DDDDD.  These existing boilers or process heaters must comply with this subpart no later than January 31, 2016, except as provided in 40 CFR 63.6(i).

Emission Units:  EU-166-77 and EU-403-74

“The collection at a major source of all existing industrial, commercial, and institutional boilers and process heaters within the unit designed to burn gas 1 fuel subcategory as defined in 40 CFR 63.7575.”

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

1.	The permittee shall only burn fuels as allowed in the Unit designed to burn gas 1 subcategory definition in 40 CFR 63.7575.  (40 CFR 63.7499(l))

III.  PROCESS/OPERATIONAL RESTRICTION(S)

- The permittee must meet the requirements in paragraphs (a)(1) and (3) of 40 CFR 63.7500, as listed below, except as provided in paragraphs (b) and (e) of 40 CFR 63.7500, stated in SC III.2 and SC III.3.  The permittee must meet these requirements at all times the affected unit is operating, except as provided in paragraph (f) of 40 CFR 63.7500, stated in SC III.4.  (40 CFR 63.7500(a))

  - The permittee must meet each work practice standard in Table 3 of 40 CFR Part 63, Subpart DDDDD that applies to the boiler or process heater, for each boiler or process heater at the source.  (40 CFR 63.7500(a)(1))

  - At all times, the permittee must operate and maintain any affected source (as defined in 40 CFR 63.7490, stated in SC IX.1), including associated air pollution control equipment and monitoring equipment, in a manner consistent with safety and good air pollution control practices for minimizing emissions. Determination of whether such operation and maintenance procedures are being used will be based on information available to the Administrator that may include, but is not limited to, monitoring results, review of operation and maintenance procedures, review of operation and maintenance records, and inspection of the source.  (40 CFR 63.7500(a)(3))

- As provided in 40 CFR 63.6(g), EPA may approve use of an alternative to the work practice standards.  (40 CFR 63.7500(b))

- Boilers and process heaters in the units designed to burn gas 1 fuels subcategory are not subject to the emission limits in Tables 1 and 2 or 11 through 13 of 40 CFR Part 63, Subpart DDDDD, or the operating limits in Table 4 of 40 CFR Part 63, Subpart DDDDD.  Boilers and process heaters in the units designed to burn gas 1 fuel subcategory with a heat input capacity:  (40 CFR 63.7500(e))

  - Of less than or equal to 5 million Btu per hour must complete a tune-up every 5 years as specified in 40 CFR 63.7540, stated in SC IX.7.  (40 CFR 63.7500(e))

  - Greater than 5 million Btu per hour and less than 10 million Btu per hour must complete a tune-up every 2 years as specified in 40 CFR 63.7540, stated in SC IX.7.  (40 CFR 63.7500(e))

- The above standards apply at all times the affected unit is operating, except during periods of startup and shutdown during which time the permittee must comply only with Table 3 of 40 CFR Part 63, Subpart DDDDD.  (40 CFR 63.7500(f))

- The permittee must complete an initial tune-up by following the procedures described in 40 CFR 63.7540(a)(10)(i) through (vi), stated in  SC IX.7, no later than the compliance date specified in 40 CFR 63.7495, stated in SC IX.3 (no later than January 31, 2016, except as provided in 40 CFR 63.6(i)).  The permittee must complete the one-time energy assessment specified in Table 3 of 40 CFR Part 63, Subpart DDDDD no later than the compliance date specified in 40 CFR 63.7495, stated in SC IX.3 (no later than January 31, 2016, except as provided in 40 CFR 63.6(i)).  (40 CFR 63.7510(e))

- If the permittee is required to meet an applicable tune-up work practice standard, the permittee must conduct an annual performance tune-up according to 40 CFR 63.7540(a)(10), stated in  SC IX.7.a biennial performance tune-up according to 40 CFR 63.7540(a)(11), stated in SC IX.7.b; or 5-year performance tune-up according to 40 CFR 63.7540(a)(12), stated in SC IX.7.c.  Each annual tune-up specified in 40 CFR 63.7540(a)(10) must be no more than 13 months after the previous tune-up.  Each biennial tune-up specified in 40 CFR 63.7540(a)(11) must be conducted no more than 25 months after the previous tune-up.  Each 5-year tune-up specified in 40 CFR 63.7540(a)(12) must be conducted no more than 61 months after the previous tune-up.  (40 CFR 63.7515(d))

- For startup and shutdown, the permittee must meet the work practice standards according to item 5 of Table 3 of 40 CFR Part 63, Subpart DDDDD.  (40 CFR 63.7540(d))

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- The permittee must keep records according to paragraphs (a)(1) and (2) of 40 CFR 63.7555, as listed below.  (40 CFR 63.7555(a))

- A copy of each notification and report that the permittee submitted to comply with 40 CFR Part 63, Subpart DDDDD, including all documentation supporting any Initial Notification or Notification of Compliance Status or semiannual compliance report that the permittee submitted, according to the requirements in 40 CFR 63.10(b)(2)(xiv).  (40 CFR 63.7555(a)(1))

- Records of performance tests, fuel analyses, or other compliance demonstrations and performance evaluations as required in 40 CFR 63.10(b)(2)(viii).  (40 CFR 63.7555(a)(2))

- If the permittee operates a unit in the unit designed to burn gas 1 subcategory that is subject to 40 CFR Part 63, Subpart DDDDD, and the permittee uses an alternative fuel other than natural gas, refinery gas, gaseous fuel subject to another subpart under 40 CFR Part 63, other gas 1 fuel, or gaseous fuel subject to another subpart of 40 CFR Part 63 or Parts 60, 61, or 65, the permittee must keep records of the total hours per calendar year that alternative fuel is burned and the total hours per calendar year that the unit operated during periods of gas curtailment or gas supply emergencies.  (40 CFR 63.7555(h))

- The permittee’s records must be in a form suitable and readily available for expeditious review, according to 40 CFR 63.10(b)(1).  (40 CFR 63.7560(a))

- As specified in 40 CFR 63.10(b)(1), the permittee must keep each record for 5 years following the date of each occurrence, measurement, maintenance, corrective action, report, or record.  (40 CFR 63.7560(b))

- The permittee must keep each record on site, or they must be accessible from on-site (for example, through a computer network), for at least 2 years after the date of each occurrence, measurement, maintenance, corrective action, report, or record, according to 40 CFR 63.10(b)(1).  The permittee can keep the records off site for the remaining 3 years.  (40 CFR 63.7560(c))

See Appendices 3-1 and 4-1

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

- The permittee must meet the notification requirements in 40 CFR 63.7545 according to the schedule in 40 CFR 63.7545, both stated in SC VII.8 through SC VII.13, and in Subpart A of 40 CFR 63.  (40 CFR 63.7495(d))

- If the permittee owns or operates an existing unit with a heat input capacity of less than 10 million Btu per hour or a unit in the unit designed to burn gas 1 subcategory, the permittee must submit a signed statement in the Notification of Compliance Status report that indicates that the permittee conducted a tune-up of the unit.

(40 CFR 63.7530(d))

- The permittee must include with the Notification of Compliance Status a signed certification that either the energy assessment was completed according to Table 3 of 40 CFR Part 63, Subpart DDDDD, and that the assessment is an accurate depiction of the facility at the time of the assessment, or that the maximum number of on-site technical hours specified in the definition of energy assessment applicable to the facility has been expended.  (40 CFR 63.7530(e))

- The permittee must submit the Notification of Compliance Status containing the results of the initial compliance demonstration according to the requirements in 40 CFR 63.7545(e), stated in SC VII.11.  (40 CFR 63.7530(f))

- The permittee must report each instance in which they did not meet each emission limit and operating limit in Tables 1 through 4 or 11 through 13 to this subpart that applies.  These instances are deviations from the emission limits or operating limits, respectively, in this subpart.  These deviations must be reported according to the requirements in 40 CFR 63.7550, cited in SC VII 16. (40 CFR 63.7540(b))

- The permittee must submit to the Administrator all of the notifications in 40 CFR 63.7(b) and (c), 40 CFR 63.8(e), (f)(4) and (6), and 40 CFR 63.9(b) through (h) that apply to the permittee by the dates specified.  (40 CFR 63.7545(a))

- As specified in 40 CFR 63.9(b)(2), if permittee starts up the affected source before January 31, 2013, the permittee must submit an Initial Notification not later than 120 days after January 31, 2013.  (40 CFR 63.7545(b))

- If the permittee is required to conduct an initial compliance demonstration as specified in 40 CFR 63.7530, stated in SC V.1, the permittee must submit a Notification of Compliance Status according to 40 CFR 63.9(h)(2)(ii).  For the initial compliance demonstration for each boiler or process heater, the permittee must submit the Notification of Compliance Status, including all performance test results and fuel analyses, before the close of business on the 60th day following the completion of all performance test and/or other initial compliance demonstrations for all boiler or process heaters at the facility according to 40 CFR 63.10(d)(2).  The Notification of Compliance Status report must contain all the information specified in paragraphs (e)(1) through (8), as applicable.  If the permittee is not required to conduct an initial compliance demonstration as specified in 40 CFR 63.7530(a), the Notification of Compliance Status must only contain the information specified in paragraphs (e)(1) and (8).  (40 CFR 63.7545(e))

  - A description of the affected unit(s) including identification of which subcategories the unit is in, the design heat input capacity of the unit, a description of the add-on controls used on the unit to comply with 40 CFR Part 63, Subpart DDDDD, description of the fuel(s) burned, including whether the fuel(s) were a secondary material determined by the permittee or the EPA through a petition process to be a non-waste under 40 CFR 241.3, whether the fuel(s) were a secondary material processed from discarded non-hazardous secondary materials within the meaning of 40 CFR 241.3, and justification for the selection of fuel(s) burned during the compliance demonstration.  (40 CFR 63.7545(e)(1))

  - In addition to the information required in 40 CFR 63.9(h)(2), your notification of compliance status must include the following certification(s) of compliance, as applicable, and signed by a responsible official:  (40 CFR 63.7545(e)(8))

i.	“This facility complies with the required initial tune-up according to the procedures in 40 CFR 63.7540(a)(10)(i) through (vi).”  (40 CFR 63.7545(e)(8)(i))

ii.	“This facility has had an energy assessment performed according to 40 CFR 63.7530(e).”  (40 CFR 63.7545(e)(8)(ii))

iii.	Except for units that burn only natural gas, refinery gas, or other gas 1 fuel, or units that qualify for a statutory exemption as provided in section 129(g)(1) of the Clean Air Act, include the following: “No secondary materials that are solid waste were combusted in any affected unit.”  (40 CFR 63.7545(e)(8)(iii)),)

- If the permittee operates a unit designed to burn natural gas, refinery gas, or other gas 1 fuels that is subject to 40 CFR Part 63, Subpart DDDDD, and the permittee intends to use a fuel other than natural gas, refinery gas, gaseous fuel subject to another subpart of 40 CFR Part 63, Part 60, Part 61, or Part 65, or other gas 1 fuel to fire the affected unit during a period of natural gas curtailment or supply interruption, as defined in 40 CFR 63.7575, the permittee must submit a notification of alternative fuel use within 48 hours of the declaration of each period of natural gas curtailment or supply interruption, as defined in 40 CFR 63.7575.  The notification must include the information specified in paragraphs (f)(1) through (5) of 40 CFR 63.7545, as listed below.  (40 CFR 63.7545(f))

- Company name and address.  (40 CFR 63.7545(f)(1))

- Identification of the affected unit.  (40 CFR 63.7545(f)(2))

- Reason the permittee is unable to use natural gas or equivalent fuel, including the date when the natural gas curtailment was declared or the natural gas supply interruption began.  (40 CFR 63.7545(f)(3))

- Type of alternative fuel that the permittee intends to use.  (40 CFR 63.7545(f)(4))

- Dates when the alternative fuel use is expected to begin and end.  (40 CFR 63.7545(f)(5))

- If the permittee intends to commence or recommence combustion of solid waste, the permittee must provide 30 days prior notice of the date upon which the permittee will commence or recommence combustion of solid waste.  The notification must identify:  (40 CFR 63.7545(g))

- The name of the owner or operator of the affected source, as defined in 40 CFR 63.7490, stated in SC IX.1, the location of the source, the boiler(s) or process heater(s) that will commence burning solid waste, and the date of the notice.  (40 CFR 63.7545(g)(1))

- The currently applicable subcategories under 40 CFR Part 63, Subpart DDDDD.  (40 CFR 63.7545(g)(2))

- The date on which the permittee became subject to the currently applicable emission limits.  (40 CFR 63.7545(g)(3))

- The date upon which the permittee will commence combusting solid waste.  (40 CFR 63.7545(g)(4))

- If the permittee has switched fuels or made a physical change to the boiler and the fuel switch or physical change resulted in the applicability of a different subcategory, the permittee must provide notice of the date upon which the permittee switched fuels or made the physical change within 30 days of the switch/change.  The notification must identify:  (40 CFR 63.7545(h))

- The name of the owner or operator of the affected source, as defined in 40 CFR 63.7490, stated in SC IX.1, the location of the source, the boiler(s) and process heater(s) that have switched fuels, were physically changed, and the date of the notice.  (40 CFR 63.7545(h)(1))

- The currently applicable subcategory under 40 CFR Part 63, Subpart DDDDD.  (40 CFR 63.7545(h)(2))

- The date upon which the fuel switch or physical change occurred.  (40 CFR 63.7545(h)(3))

- The permittee must submit each report in Table 9 of 40 CFR Part 63, Subpart DDDDD that applies.  (40 CFR 63.7550(a))

- Unless the EPA Administrator has approved a different schedule for submission of reports under 40 CFR 63.10(a), the permittee must submit each report, according to paragraph (h) of 40 CFR 63.7550, stated in SC VII.17, by the date in Table 9 of 40 CFR Part 63, Subpart DDDDD and according to the requirements in paragraphs (b)(1) through (4) of 40 CFR 63.7550, as listed below.  For units that are subject only to a requirement to conduct an annual tune-up according to 40 CFR 63.7540(a)(10), stated in SC IX.7.a, biennial tune-up according to 40 CFR 63.7540(a)(11), stated in SC IX7.b, or 5-year tune-up according to 40 CFR 63.7540(a)(12), stated in SC IX.7.c, and not subject to emission limits or operating limits, the permittee may submit only an annual, biennial, or 5-year compliance report, as applicable, as specified in paragraphs (b)(1) through (4) of 40 CFR 63.7550, as listed below, instead of a semiannual compliance report.  (40 CFR 63.7550(b))

- The first semiannual compliance report must cover the period beginning on the compliance date that is specified for each boiler or process heater in 40 CFR 63.7495, stated in SC IX.3, and ending on June 30 or December 31, whichever date is the first date that occurs at least 180 days after the compliance date that is specified for the source in 40 CFR 63.7495, stated in SC IX.3.  If submitting an annual, biennial, or 5-year compliance report, the first compliance report must cover the period beginning on the compliance date that is specified for each boiler or process heater in 40 CFR 63.7495 and ending on December 31 within 1, 2, or 5 years, as applicable, after the January 31, 2016 compliance date.(40 CFR 63.7550(b)(1))

- The first semiannual compliance report must be postmarked or submitted no later than July 31 or January 31, whichever date is the first date following the end of the first calendar half after the compliance date that is specified for each boiler or process heater in 40 CFR 63.7495, stated in SC IX.3.  The first annual, biennial, or 5-year compliance report must be postmarked or submitted no later than January 31.  (40 CFR 63.7550(b)(2), 40 CFR 63.10(a)(5))

- Each subsequent semiannual compliance report must cover the semiannual reporting period from January 1 through June 30 or the semiannual reporting period from July 1 through December 31. Annual, biennial, and 5-year compliance reports must cover the applicable 1, 2, or 5-year periods from January 1 to December 31.  (40 CFR 63.7550(b)(3))

- Each subsequent compliance report must be postmarked or submitted no later than September 15 or March 15, whichever date is the first date following the end of the semiannual reporting period. Annual, biennial, and 5-year compliance reports must be postmarked or submitted no later than March 15.  (40 CFR 63.7550(b)(4), 40 CFR 63.10(a)(5))

- A compliance report must contain the following information depending on how the permittee chooses to comply with the limits set in this rule.  (40 CFR 63.7550(c))

- If the facility is subject to the requirements of a tune-up, the permittee must submit a compliance report with the information in SC VII.18 (d)(i) through (iii) and (ix).  (40 CFR 63.7550(c)(1))

- If the permittee is complying with the fuel analysis a compliance report must be submitted with the information in SC VII.18(d)(i) through(iii) and (v) through (viii).   (40 CFR 63.7550(c)(2))

- 40 CFR 63.7550(c)(5) is as follows:

    - Company and Facility name and address.  (40 CFR 63.7550(c)(5)(i))

    - Process unit information, emissions limitations, and operating parameter limitations.  (40 CFR 63.7550(c)(5)(ii))

    - Date of report and beginning and ending dates of the reporting period.  (40 CFR 63.7550(c)(5)(iii))

    - The total operating time during the reporting period.  (40 CFR 63.7550(c)(5)(iv))

    - Include the date of the most recent tune-up for each unit subject to only the requirement to conduct an annual tune-up according to 40 CFR 63.7540(a)(10), stated in SC IX.7.a, biennial tune-up according to 40 CFR 63.7540(a)(10), stated in SC IX.7.b, or 5-year tune-up according to 40 CFR 63.7540(a)(12), stated in SC IX.7.c. Include the date of the most recent burner inspection if it was not done annually, biennially, or on a 5-year period and was delayed until the next scheduled or unscheduled unit shutdown.  (40 CFR 63.7550(c)(5)(xiv))

- The permittee must submit the reports according to the procedures specified in paragraphs (h)(1) through (3) of 40 CFR 63.7550, as listed below.  (40 CFR 63.7550(h))

- Within 60 days after the date of completing each performance test (defined in 40 CFR 63.2) required by 40 CFR Part 63, Subpart DDDDD, the permittee must submit the results of the performance tests, including any associated fuel analyses, following the procedure specified in either SC VII.19(a)(i) or (ii)

    - For data collected using test methods supported by the EPA’s Electronic Reporting Tool (ERT) as listed on the EPA’s ERT Web site (http://www.epa.gov/ttn/chief/ert/index.html), you must submit the results of the performance test to the  EPA via the Compliance and Emissions Data Reporting Interface CEDRI.  (CEDRI can be accessed through the EPA's Central Data Exchange (CDX) (https://cdx.epa.gov/).  Performance test data must be submitted in a file format generated through use of the EPA's ERT or an electronic file format consistent with the extensible markup language (XML) schema listed on the ERT website. If the permittee claims that some of the performance test information being submitted is confidential business information (CBI), the permittee must submit a complete file generated through the use of the EPA’s ERT or an alternate electronic file consistent with the XML schema listed on the EPA’s ERT website, including information claimed to be CBI, on a compact disc, flash drive,  or other commonly used electronic storage media  to the EPA.  The electronic media must be clearly marked as CBI and mailed to U.S. EPA/OAPQS/CORE CBI Office, Attention: Group Leader, Measurement Policy Group, MD C404-02, 4930 Old Page Road, Durham, NC 27703.  The same ERT or alternate file with the CBI omitted must be submitted to the EPA via the EPA’s CDX as described earlier in this paragraph. (40 CFR 63.7550(h)(1)(i))

    - For data collected using test methods that are not supported by the EPA’s ERT as listed on the EPA’s ERT website at the time of the test, you must submit the results of the performance test to the Administrator at the appropriate address listed in 40 CFR 63.13.  (40 CFR 63.7550(h)(1)(ii))

- Within 60 days after the date of completing each CEMS performance evaluation (as defined in 40 CFR 63.2), the permittee must submit the results of the performance evaluation following the procedure specified in either SC VII.19(b)(i) or (ii) below:

- For performance evaluations of continuous monitoring systems measuring relative accuracy test audit (RATA) pollutants that are supported by the EPA’s ERT as listed on the EPA’s ERT website at the time of the evaluation, the permittee must submit the results of the performance evaluation to the EPA via the CEDRI. (CEDRI can be accessed through the EPA’s CDX.)  Performance evaluation data must be submitted in a file format generated through the use of the EPA’s ERT or an alternate file format consistent with the XML Schema listed on the EPA’s ERT website.  If the permittee claims that some of the performance evaluation information being transmitted is CBI, you must submit a complete file generated through the use of the EPA’s ERT or an alternate electronic file consistent with the SML schema listed on the EPA’s ERT website, including information claimed to be CBI, on a compact disc, flash drive or other commonly used electronic storage media to the EPA.  The electronic media must be clearly marked as CBI and mailed to U.S. EPA/OAPQS/CORE CBI Office, Attention: Group Leader, Measurement Policy Group, MD C404-02, 4930 Old Page Rd., Durham, NC 27703.  The same ERT or alternate file with the CBI omitted must be submitted to the EPA via the EPA’s CDX as described earlier in this paragraph.    (40 CFR 63.7550(h)(2)(i))

- For any performance evaluations of continuous monitoring systems measuring RATA pollutants that are not supported by the EPA’s ERT as listed on the ERT website, the permittee must submit the results of the performance evaluation to the Administrator at the appropriate address listed in 40 CFR 63.13.  (40 CFR 63.7550(h)(2)(i)(ii)

- The permittee must submit all reports required by Table 9 of 40 CFR Part 63, Subpart DDDDD electronically to the EPA via the CEDRI. (CEDRI can be accessed through the EPA’s CDX.) The permittee must use the appropriate electronic report in CEDRI for this subpart.  Instead of using the electronic report in CEDRI for this subpart, the permittee may submit an alternate electronic file consistent with the XML schema listed on the CEDRI website (http://www.epa.gov/ttn/chief/cedri/index.html), once the XML schema is available,.  If the reporting form specific to 40 CFR Part 63, Subpart DDDDD is not available in CEDRI at the time that the report is due, the permittee must submit the report to the Administrator at the appropriate address listed in 40 CFR 63.13.  The permittee must begin submitting reports via CEDRI no later than

90-days after the form become available in CEDRI.  (40 CFR 63.7550(h)(3))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

1.	40 CFR Part 63, Subpart DDDDD applies to existing affected sources as described in paragraph (a)(1) of 40 CFR 63.7490, as listed below.  (40 CFR 63.7490(a))

a.	The affected source of 40 CFR Part 63, Subpart DDDDD is the collection at a major source of all existing industrial, commercial, and institutional boilers and process heaters within a subcategory as defined in 40 CFR 63.7575.  (40 CFR 63.7490(a)(1))

- A boiler or process heater is existing if it is not new or reconstructed, as defined below.  (40 CFR 63.7490(d))

- A boiler or process heater is new if the permittee commences construction of the boiler or process heater after June 4, 2010, and the permittee meets the applicability criteria at the time the permittee commences construction.  (40 CFR 63.7490(b))

b.	A boiler or process heater is reconstructed if the permittee meets the reconstruction criteria as defined in 40 CFR 63.2, the permittee commences reconstruction after June 4, 2010, and the permittee meets the applicability criteria at the time the permittee commence reconstruction.  (40 CFR 63.7490(c))

- If the permittee has an existing boiler or process heater, the permittee must comply with 40 CFR Part 63, Subpart DDDDD no later than January 31, 2016, except as provided in 40 CFR 63.6(i).  (40 CFR 63.7495(b))

- If the permittee has an area source that increases its emissions or its potential to emit such that it becomes a major source of HAP, paragraph (c)(2) of 40 CFR 63.7495, as listed below, applies to the permittee.

(40 CFR 63.7495(c))

a.	Any existing boiler or process heater at the existing source must be in compliance with 40 CFR Part 63, Subpart DDDDD within 3 years after the source becomes a major source.  (40 CFR 63.7495(c)(2))

- The permittee must be in compliance with the emission limits, work practice standards, and operating limits of 40 CFR Part 63, Subpart DDDDD.  These limits apply at all times the affected unit is operating except for the periods noted in 40 CFR 63.7500(f), stated in SC III.4.  (40 CFR 63.7505(a))

- For affected sources (as defined in 40 CFR 63.7490, stated in SC IX.1) that have not operated since the previous compliance demonstration and more than one year has passed since the previous compliance demonstration, the permittee must complete a subsequent tune-up by following the procedures described in 40 CFR 63.7540(a)(10)(i) through (vi), stated in SC IX.7.a, and the schedule described in 40 CFR 63.7540(a)(13), stated in SC IX.7.d, for units that are not operating at the time of their scheduled tune-up.  (40 CFR 63.7515(g))

- The permittee must demonstrate continuous compliance with the work practice standards in Table 3 of 40 CFR Part 63, Subpart DDDDD that applies according to the methods specified in paragraphs (a)(10) through (13) of 40 CFR 63.7540, as listed below.  (40 CFR 63.7540(a))

  - If the boiler or process heater has a heat input capacity of 10 million Btu per hour or greater, the permittee must conduct an annual tune-up of the boiler or process heater to demonstrate continuous compliance as specified in paragraphs (a)(10)(i) through (vi) of 40 CFR 63.7540, as listed below.  The tune-up must be conducted while burning the type of fuel (or fuels in case of units that routinely burn a mixture) that provided the majority of the heat input to the boiler or process heater over the 12 months prior to the tune-up.  This frequency does not apply to units with continuous oxygen trim systems that maintain an optimum air to fuel ratio.  (40 CFR 63.7540(a)(10))

i.	As applicable, inspect the burner, and clean or replace any components of the burner as necessary (the permittee may perform the burner inspection any time prior to tune-up or delay the burner inspection until the next scheduled unit shutdown).  Units that produce electricity for sale may delay the burner inspection until the first outage, not to exceed 36 months from the previous inspection.  At units where entry into a piece of process equipment or into a storage vessel is required to complete the tune-up inspections, inspections are required only during planned entries into the storage vessel or process equipment.  (40 CFR 63.7540(a)(10)(i))

ii.	Inspect the flame pattern, as applicable, and adjust the burner as necessary to optimize the flame pattern.  The adjustment should be consistent with the manufacturer's specifications, if available.  (40 CFR 63.7540(a)(10)(ii))

iii.	Inspect the system controlling the air-to-fuel ratio, as applicable, and ensure that it is correctly calibrated and functioning properly (the permittee may delay the inspection until the next scheduled unit shutdown).  Units that produce electricity for sale may delay the inspection until the first outage, not to exceed 36 months from the previous inspection.  (40 CFR 63.7540(a)(10)(iii))

iv.	Optimize total emissions of CO.  This optimization should be consistent with the manufacturer's specifications, if available, and with any NOX requirement to which the unit is subject.  (40 CFR 63.7540(a)(10)(iv))

v.	Measure the concentrations in the effluent stream of CO in parts per million, by volume, and oxygen in volume percent, before and after the adjustments are made (measurements may be either on a dry or wet basis, as long as it is the same basis before and after the adjustments are made).  Measurements may be taken using a portable CO analyzer.  (40 CFR 63.7540(a)(10)(v))

vi.	Maintain on-site and submit, if requested by the Administrator, a report containing the information in paragraphs (a)(10)(vi)(A) through (C) of 40 CFR 63.7540, as listed below.  (40 CFR 63.7540(a)(10)(vi))

- The concentrations of CO in the effluent stream in parts per million by volume, and oxygen in volume percent, measured at high fire or typical operating load, before and after the tune-up of the boiler or process heater.  (40 CFR 63.7540(a)(10)(vi)(A))

- A description of any corrective actions taken as a part of the tune-up.  (40 CFR 63.7540(a)(10)(vi)(B))

- The type and amount of fuel used over the 12 months prior to the tune-up, but only if the unit was physically and legally capable of using more than one type of fuel during that period. Units sharing a fuel meter may estimate the fuel used by each unit.  (40 CFR 63.7540(a)(10)(vi)(C))

- If the boiler or process heater has a heat input capacity of less than 10 million Btu per hour (except as specified in paragraph (a)(12) of 40 CFR 63.7540), the permittee must conduct a biennial tune-up of the boiler or process heater as specified in paragraphs (a)(10)(i) through (vi) of 40 CFR 63.7540 to demonstrate continuous compliance.  (40 CFR 63.7540(a)(11))

- If the boiler or process heater has a continuous oxygen trim system that maintains an optimum air to fuel ratio, or a heat input capacity of less than or equal to 5 million Btu per hour and the unit is in the units designed to burn gas 1 subcategory, the permittee must conduct a tune-up of the boiler or process heater every 5 years as specified in paragraphs (a)(10)(i) through (vi) of 40 CFR 63.7540 to demonstrate continuous compliance.  The permittee may delay the burner inspection specified in paragraph (a)(10)(i) of 40 CFR 63.7540 until the next scheduled or unscheduled unit shutdown, but the permittee must inspect each burner at least once every 72 months.  If an oxygen trim system is utilized on a unit without emission standards to reduce the tune-up frequency to once every 5 years, set the oxygen level no lower than the oxygen concentration measured during the most recent tune-up.  (40 CFR 63.7540(a)(12))

- If the unit is not operating on the required date for a tune-up, the tune-up must be conducted within 30 calendar days of startup.  (40 CFR 63.7540(a)(13))

- Table 10 of 40 CFR Part 63, Subpart DDDDD shows which parts of the General Provisions in 40 CFR 63.1 through 63.15 applies to the permittee.  (40 CFR 63.7565)

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-RULE287(2)(C)

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Any emission unit that emits air contaminants and is exempt from the requirements of Rule 201 pursuant to Rules 278, 278a and 287(2)(c).  Emission units installed/modified before December 20, 2016, may show compliance with Rule 287 in effect at the time of installation/modification.

Emission Units installed on or after December 20, 2016:   Any future emission unit that meets the requirements of this flexible group.

Emission Units installed prior to December 20, 2016:   EU-RULE287(2)(c)

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

| Material | Limit | Time Period/Operating Scenario | Equipment | Underlying Applicable Requirement |
| --- | --- | --- | --- | --- |
| - Coatings | 200 Gallons/month (minus water as applied) | Calendar month | Each emission unit | R 336.1287(2)(c)(i) |

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

1.	Any exhaust system installed on or after December 20, 2016, that serves only coating spray equipment shall be equipped with a dry filter control or water wash control which is installed, maintained, and operated in accordance with the manufacturer’s specifications, or the permittee develops a plan which provides to the extent practicable for the maintenance and operation of the equipment in a manner consistent with good air pollution control practices for minimizing emissions.  All emission units installed before December 20, 2016, with an exhaust system that serves only coating spray equipment must have a properly installed and operated particulate control system. (R 336.1213(2), R 336.1287(2)(c)(ii), R 336.1910)

V.  TESTING/SAMPLING

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The permittee shall maintain records of the following information for each emission unit for each calendar month using the methods outlined in the DEQ, AQD Rule 287(2)(c), Permit to Install Exemption Record form (EQP 3562) or in a format acceptable to the AQD District Supervisor.  (R 336.1213(3))

a.	Volume of coating used, as applied, minus water, in gallons.  (R 336.1287(2)(c)(iii))

b.	For emission units installed on or after December 20, 2016, documentation of any filter replacements or maintenance of water wash control for exhaust systems serving coating spray equipment or other documentation included in a plan developed by the owner or operator of the equipment.  For emission units installed before December 20, 2016, documentation that the exhaust system that serves only coating spray equipment is supplied with a properly installed and operating particulate control system.   (R 336.1213(3))

See Appendix 4-1

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

NA

## FG-RULE290

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Any emission unit that emits air contaminants and is exempt from the requirements of Rule 201 pursuant to Rules 278, 278a and 290.  Emission units installed/modified before December 20, 2016, may show compliance with Rule 290 in effect at the time of installation/modification.

Emission Units installed on or after December 20, 2016:  EU-RULE290 and any future emission unit that meets the requirements of this flexible group.

Emission Units installed prior to December 20, 2016:  EU-GlueRobot, EU-CastSkins

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

1.	Each emission unit that emits only noncarcinogenic volatile organic compounds or noncarcinogenic materials which are listed in Rule 122(f) as not contributing appreciably to the formation of ozone, if the total uncontrolled or controlled emissions of air contaminants are not more than 1,000 or 500 pounds per month, respectively.  (R 336.1290(2)(a)(i))

2.	Any emission unit for which CO2 equivalent emissions are not more than 6,250 tons per month and for which the total uncontrolled or controlled emissions of all other air contaminants are not more than 1,000 or 500 pounds per month, respectively, and all the following criteria listed below are met: (R 336.1290(2)(a)(ii))

a.	For toxic air contaminants, excluding noncarcinogenic volatile organic compounds and noncarcinogenic materials which are listed in Rule 122(f) as not contributing appreciably to the formation of ozone, with initial threshold screening levels greater than or equal to 0.04 micrograms per cubic meter and less than 2.0 micrograms per cubic meter, the uncontrolled or controlled emissions shall not exceed 20 or 10 pounds per month, respectively.

(R 336.1290(2)(a)(ii)(A))

b.	For toxic air contaminants with initial risk screening levels greater than or equal to 0.04 microgram per cubic meter, the uncontrolled or controlled emissions shall not exceed 20 or 10 pounds per month, respectively.  (R 336.1290(2)(a)(ii)(B))

c.	The emission unit shall not emit any toxic air contaminants, excluding non-carcinogenic volatile organic compounds and noncarcinogenic materials which are listed in Rule 122(f) as not contributing appreciably to the formation of ozone, with an initial threshold screening level or initial risk screening level less than 0.04 microgram per cubic meter.  (R 336.1290(2)(a)(ii)(C))

- For total mercury, the uncontrolled or controlled emissions shall not exceed 0.01 pounds per month from emission units installed on or after December 20, 2016.

(R 336.1290(2)(a)(ii)(D))

e.	For lead, the uncontrolled or controlled emissions shall not exceed 16.7 pounds per month from emission units installed on or after December 20, 2016.  (R 336.1290(2)(a)(ii)(E))

3.	Any emission unit that emits only particulate air contaminants without initial risk screening levels and other air contaminants that are exempted under Rule 290(2)(a)(i) or Rule 290(2)(a)(ii), if all the following provisions are met: (R 336.1290(2)(a)(iii))

a.	The particulate emissions are controlled by an appropriately designed and operated fabric filter collector or an equivalent control system which is designed to control particulate matter to a concentration of less than or equal to 0.01 pound of particulate per 1,000 pounds of exhaust gases and which does not have exhaust gas flow rate more than 30,000 actual cubic feet per minute.  (R 336.1290(2)(a)(iii)(A))

b.	The visible emissions from the emission unit are not more than 5% opacity in accordance with the methods contained in Rule 303.  (R 336.1290(2)(a)(iii)(B))

c.	The initial threshold screening level for each particulate toxic air contaminant, excluding nuisance particulate, is more than 2.0 micrograms per cubic meter.  (R 336.1290(2)(a)(iii)(C))

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

- The provisions of Rule 290 apply to each emission unit that is operating pursuant to Rule 290.  (R 336.1290)

- The following requirements apply to emission units installed on or after December 20, 2016, utilizing control equipment:

  - An air cleaning device for volatile organic compounds shall be installed, maintained, and operated in accordance with the manufacturer’s specifications.  Examples include the following (R 336.1290(2)(b)(i),

R 336.1910)

    - Oxidizers and condensers equipped with a continuously displayed temperature indication device.

    - Wet scrubbers equipped with a liquid flow rate monitor.

    - Dual stage carbon absorption where the first canister is monitored for breakthrough and replaced if breakthrough is detected.

  - An air cleaning device for particulate matter shall be installed, maintained, and operated in accordance with the manufacturer’s specifications or the permittee shall develop a plan that provides to the extent practicable for the maintenance and operation of the equipment in the manner consistent with good air pollution control practices for minimizing emissions.  It shall also be equipped to monitor appropriate indicators of performance, for example, static pressure drop, water pressure, and water flow rate.

(R 336.1290(2)(b)(ii), R 336.1910)

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The permittee shall maintain records of the following information for each emission unit for each calendar month using the methods outlined in the DEQ, AQD Rule 290; Permit to Install Exemption Record form (EQP 3558) or in a format that is acceptable to the AQD District Supervisor.  (R 336.1213(3))

a.	Records identifying each air contaminant that is emitted.  (R 336.1213(3))

b.	Records identifying if each air contaminant is controlled or uncontrolled.  (R 336.1213(3))

c.	Records identifying if each air contaminant is either carcinogenic or non-carcinogenic.  (R 336.1213(3))

d.	Records identifying the ITSL and IRSL, if established, of each air contaminant that is being emitted under the provisions of Rules 290(2)(a)(ii) and (iii).   (R 336.1213(3))

- Records of material use and calculations identifying the quality, nature, and quantity of the air contaminant emissions in sufficient detail to demonstrate that the actual emissions of the emission unit meet the emission limits outlined in this table and Rule 290.  Volatile organic compound emissions from units installed on or after December 20, 2016, shall be calculated using mass balance, generally accepted engineering calculations, or another method acceptable to the AQD District Supervisor.  (R 336.1213(3), R 336.1290(2)(d))

- Records are maintained on file for the most recent 2-year period and are made available to the department upon request.  (R 336.1213(3), R 336.1290(2)(e))

2.	The permittee shall maintain an inventory of each emission unit that is exempt pursuant to Rule 290.  This inventory shall include the following information.  (R 336.1213(3))

a.	The permittee shall maintain a written description of each emission unit as it is maintained and operated throughout the life of the emission unit.  (R 336.1290(2)(c), R 336.1213(3))

b.	For each emission unit that emits noncarcinogenic particulate air contaminants pursuant to Rule 290(2)(a)(iii), the permittee shall maintain a written description of the control device, including the designed control efficiency and the designed exhaust gas flow rate.  (R 336.1213(3))

3.	For each emission unit that emits noncarcinogenic particulate air contaminants pursuant to Rule 290(2)(a)(iii), the permittee shall perform a monthly visible emission observation of each stack or vent during routine operating conditions.  This observation need not be performed using Method 9.  The permittee shall keep a written record of the results of each observation.  (R 336.1213(3))

See Appendix 4-1

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-1

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.   OTHER REQUIREMENT(S)

NA

## FG-COLDCLEANERS

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Any cold cleaner that is grandfathered or exempt from Rule 201 pursuant to Rule 278, 278a and Rule 281(2)(h) or Rule 285(2)(r)(iv).  Existing cold cleaners were placed into operation prior to July 1, 1979.  New cold cleaners were placed into operation on or after July 1, 1979.

Emission Unit:   EU-COLDCLEANERS

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

1.	The permittee shall not use cleaning solvents containing more than five percent by weight of the following halogenated compounds: methylene chloride, perchloroethylene, trichloroethylene, 1,1,1-trichloroethane, carbon tetrachloride, chloroform, or any combination thereof.  (R 336.1213(2))

III.  PROCESS/OPERATIONAL RESTRICTION(S)

1.	Cleaned parts shall be drained for no less than 15 seconds or until dripping ceases.  (R 336.1611(2)(b), R 336.1707(3)(b))

2.	The permittee shall perform routine maintenance on each cold cleaner as recommended by the manufacturer.  (R 336.1213(3))

IV.  DESIGN/EQUIPMENT PARAMETER(S)

1.	The cold cleaner must meet one of the following design requirements:

a.	The air/vapor interface of the cold cleaner is no more than ten square feet.  (R 336.1281(2)(h))

b.	The cold cleaner is used for cleaning metal parts and the emissions are released to the general in-plant environment.  (R 336.1285((2)r)(iv))

2.	The cold cleaner shall be equipped with a device for draining cleaned parts.  (R 336.1611(2)(b), R 336.1707(3)(b))

3.	All new and existing cold cleaners shall be equipped with a cover and the cover shall be closed whenever parts are not being handled in the cold cleaner.  (R 336.1611(2)(a), R 336.1707(3)(a))

4.	The cover of a new cold cleaner shall be mechanically assisted if the Reid vapor pressure of the solvent is more than 0.3 psia or if the solvent is agitated or heated.  (R 336.1707(3)(a))

5.	If the Reid vapor pressure of any solvent used in a new cold cleaner is greater than 0.6 psia; or, if any solvent used in a new cold cleaner is heated above 120 degrees Fahrenheit, then the cold cleaner must comply with at least one of the following provisions:

a.	The cold cleaner must be designed such that the ratio of the freeboard height to the width of the cleaner is equal to or greater than 0.7.  (R 336.1707(2)(a))

b.	The solvent bath must be covered with water if the solvent is insoluble and has a specific gravity of more than 1.0.  (R 336.1707(2)(b))

c.	The cold cleaner must be controlled by a carbon adsorption system, condensation system, or other method of equivalent control approved by the AQD.  (R 336.1707(2)(c))

V.  TESTING/SAMPLING

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	For each new cold cleaner in which the solvent is heated, the solvent temperature shall be monitored and recorded at least once each calendar week during routine operating conditions.  (R 336.1213(3))

2.	The permittee shall maintain the following information on file for each cold cleaner:  (R 336.1213(3))

a.	A serial number, model number, or other unique identifier for each cold cleaner.

b.	The date the unit was installed, manufactured or that it commenced operation.

c.	The air/vapor interface area for any unit claimed to be exempt under Rule 281(2)(h).

d.	The applicable Rule 201 exemption.

e.	The Reid vapor pressure of each solvent used.

f.	If applicable, the option chosen to comply with Rule 707(2).

3.	The permittee shall maintain written operating procedures for each cold cleaner.  These written procedures shall be posted in an accessible, conspicuous location near each cold cleaner.  (R 336.1611(3), R 336.1707(4))

4.	As noted in Rule 611(2)(c) and Rule 707(3)(c), if applicable, an initial demonstration that the waste solvent is a safety hazard shall be made prior to storage in non-closed containers.  If the waste solvent is a safety hazard and is stored in non-closed containers, verification that the waste solvent is disposed of so that not more than 20 percent, by weight, is allowed to evaporate into the atmosphere shall be made on a monthly basis.  (R 336.1213(3), R 336.1611(2)(c), R 336.1707(3)(c))

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8-1

VIII. STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

NA

# E.  NON-APPLICABLE REQUIREMENTS

At the time of the ROP issuance, the AQD has determined that no non-applicable requirements have been identified for incorporation into the permit shield provision set forth in the General Conditions in Part A pursuant to Rule 213(6)(a)(ii).

# APPENDICES

## Appendix 1-1.  Acronyms and Abbreviations

| Common Acronyms |  | Pollutant / Measurement Abbreviations |  |
| --- | --- | --- | --- |
| AQD | Air Quality Division | acfm | Actual cubic feet per minute |
| BACT | Best Available Control Technology | BTU | British Thermal Unit |
| CAA | Clean Air Act | °C | Degrees Celsius |
| CAM | Compliance Assurance Monitoring | CO | Carbon Monoxide |
| CEM | Continuous Emission Monitoring | CO2e | Carbon Dioxide Equivalent |
| CFR | Code of Federal Regulations | dscf | Dry standard cubic foot |
| COM | Continuous Opacity Monitoring | dscm | Dry standard cubic meter |
| Department/ department | Michigan Department of Environmental Quality | °F | Degrees Fahrenheit |
|  |  | gr | Grains |
| EU | Emission Unit | HAP | Hazardous Air Pollutant |
| FG | Flexible Group | Hg | Mercury |
| GACS | Gallons of Applied Coating Solids | hr | Hour |
| GC | General Condition | HP | Horsepower |
| GHGs | Greenhouse Gases | H2S | Hydrogen Sulfide |
| HVLP | High Volume Low Pressure* | kW | Kilowatt |
| ID | Identification | lb | Pound |
| IRSL | Initial Risk Screening Level | m | Meter |
| ITSL | Initial Threshold Screening Level | mg | Milligram |
| LAER | Lowest Achievable Emission Rate | mm | Millimeter |
| MACT | Maximum Achievable Control Technology | MM | Million |
| MAERS | Michigan Air Emissions Reporting System | MW | Megawatts |
| MAP | Malfunction Abatement Plan | NMOC | Non-methane Organic Compounds |
| MDEQ | Michigan Department of Environmental Quality | NOx | Oxides of Nitrogen |
|  |  | ng | Nanogram |
| MSDS | Material Safety Data Sheet | PM | Particulate Matter |
| NA | Not Applicable | PM10 | Particulate Matter equal to or less than 10 microns in diameter |
| NAAQS | National Ambient Air Quality Standards |  |  |
| NESHAP | National Emission Standard for Hazardous Air Pollutants | PM2.5 | Particulate Matter equal to or less than 2.5 microns in diameter |
| NSPS | New Source Performance Standards | pph | Pounds per hour |
| NSR | New Source Review | ppm | Parts per million |
| PS | Performance Specification | ppmv | Parts per million by volume |
| PSD | Prevention of Significant Deterioration | ppmw | Parts per million by weight |
| PTE | Permanent Total Enclosure | psia | Pounds per square inch absolute |
| PTI | Permit to Install | psig | Pounds per square inch gauge |
| RACT | Reasonable Available Control Technology | scf | Standard cubic feet |
| ROP | Renewable Operating Permit | sec | Seconds |
| SC | Special Condition | SO2 | Sulfur Dioxide |
| SCR | Selective Catalytic Reduction | TAC | Toxic Air Contaminant |
| SNCR | Selective Non-Catalytic Reduction | Temp | Temperature |
| SRN | State Registration Number | THC | Total Hydrocarbons |
| TEQ | Toxicity Equivalence Quotient | tpy | Tons per year |
| USEPA/EPA | United States Environmental Protection Agency | µg | Microgram |
|  |  | µm | Micrometer or Micron |
| VE | Visible Emissions | VOC | Volatile Organic Compounds |
|  |  | yr | Year |

*For HVLP applicators, the pressure measured at the gun air cap shall not exceed 10 psig.

## Appendix 2-1.  Schedule of Compliance

The permittee certified in the ROP application that this stationary source is in compliance with all applicable requirements and the permittee shall continue to comply with all terms and conditions of this ROP.  A Schedule of Compliance is not required.  (R 336.1213(4)(a), R 336.1119(a)(ii))

## Appendix 3-1.  Monitoring Requirements

Specific monitoring requirement procedures, methods or specifications are detailed in Part A or the appropriate Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Therefore, this appendix is not applicable.

## Appendix 4-1.  Recordkeeping

The permittee shall use the following approved formats and procedures for the recordkeeping requirements referenced in FG-RULE287(2)(c).  Alternative formats must be approved by the AQD District Supervisor.

Rule 287(2)(c) Flexible Group

The permittee shall use the DEQ Rule 287(2)(c) Permit to Install Exemption Record form (EQP 3562) or an alternative format as approved by the AQD District Supervisor to document monthly records as required by R 336.1287(2)(c) and FG-RULE287(2)(c).

Rule 290 Flexible Group

The permittee shall use the DEQ Rule 290 Permit to Install Exemption Record form (EQP 3558) or an alternative format as approved by the AQD District Supervisor to document monthly records as required by R 336.1290 and FG-RULE290-1.

## Appendix 5-1.  Testing Procedures

There are no specific testing requirement plans or procedures for this ROP.  Therefore, this appendix is not applicable.

## Appendix 6-1.  Permits to Install

The following table lists any PTIs issued or ROP revision applications received since the effective date of the previously issued ROP No. MI-ROP-B2063-2012. Those ROP revision applications that are being issued concurrently with this ROP renewal are identified by an asterisk (*).  Those revision applications not listed with an asterisk were processed prior to this renewal.

Source-Wide PTI No. MI-PTI-B2063-2012b is being reissued as Source-Wide PTI No. MI-PTI-B2063-2018.

| Permit to Install Number | ROP Revision Application Number | Description of Equipment or Change | Corresponding Emission Unit(s) or Flexible Group(s) |
| --- | --- | --- | --- |
| 35-13 | NA | Automotive Interior Plastic Parts Coating Line | EU-AutoPlasCoatLn |

## Appendix 7-1.  Emission Calculations

There are no specific emission calculations to be used for this ROP.  Therefore, this appendix is not applicable.

## Appendix 8-1.  Reporting

A.  Annual, Semiannual, and Deviation Certification Reporting

The permittee shall use the MDEQ, AQD, Report Certification form (EQP 5736) and MDEQ, AQD, Deviation Report form (EQP 5737) for the annual, semiannual and deviation certification reporting referenced in the Reporting Section of the Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Alternative formats must meet the provisions of Rule 213(4)(c) and Rule 213(3)(c)(i), respectively, and be approved by the AQD District Supervisor.

B.  Other Reporting

Specific reporting requirement formats and procedures are detailed in Part A or the appropriate Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Therefore, Part B of this appendix is not applicable.

# SECTION 2 – Ford Motor Company – Saline Plant

# A.  GENERAL CONDITIONS

## Permit Enforceability

- All conditions in this permit are both federally enforceable and state enforceable unless otherwise noted. (R 336.1213(5))

- Those conditions that are hereby incorporated in a state-only enforceable Source-Wide PTI pursuant to Rule 201(2)(d) are designated by footnote one.  (R 336.1213(5)(a), R 336.1214a(5))

- Those conditions that are hereby incorporated in a federally enforceable Source-Wide PTI pursuant to Rule 201(2)(c) are designated by footnote two.  (R 336.1213(5)(b), R 336.1214a(3))

## General Provisions

- The permittee shall comply with all conditions of this ROP.  Any ROP noncompliance constitutes a violation of Act 451, and is grounds for enforcement action, for ROP revocation or revision, or for denial of the renewal of the ROP.  All terms and conditions of this ROP that are designated as federally enforceable are enforceable by the Administrator of the United States Environmental Protection Agency (USEPA) and by citizens under the provisions of the federal Clean Air Act (CAA).  Any terms and conditions based on applicable requirements which are designated as “state-only” are not enforceable by the USEPA or citizens pursuant to the CAA.  (R 336.1213(1)(a))

- It shall not be a defense for the permittee in an enforcement action that it would have been necessary to halt or reduce the permitted activity in order to maintain compliance with the conditions of this ROP.  (R 336.1213(1)(b))

- This ROP may be modified, revised, or revoked for cause.  The filing of a request by the permittee for a permit modification, revision, or termination, or a notification of planned changes or anticipated noncompliance does not stay any ROP term or condition.  This does not supersede or affect the ability of the permittee to make changes, at the permittee’s own risk, pursuant to Rule 215 and Rule 216.  (R 336.1213(1)(c))

- The permittee shall allow the department, or an authorized representative of the department, upon presentation of credentials and other documents as may be required by law and upon stating the authority for and purpose of the investigation, to perform any of the following activities:  (R 336.1213(1)(d))

  - Enter, at reasonable times, a stationary source or other premises where emissions-related activity is conducted or where records must be kept under the conditions of the ROP.

  - Have access to and copy, at reasonable times, any records that must be kept under the conditions of the ROP.

  - Inspect, at reasonable times, any of the following:

    - Any stationary source.

    - Any emission unit.

    - Any equipment, including monitoring and air pollution control equipment.

    - Any work practices or operations regulated or required under the ROP.

  - As authorized by Section 5526 of Act 451, sample or monitor at reasonable times substances or parameters for the purpose of assuring compliance with the ROP or applicable requirements.

- The permittee shall furnish to the department, within a reasonable time, any information the department may request, in writing, to determine whether cause exists for modifying, revising, or revoking the ROP or to determine compliance with this ROP.  Upon request, the permittee shall also furnish to the department copies of any records that are required to be kept as a term or condition of this ROP.  For information which is claimed by the permittee to be confidential, consistent with the requirements of the 1976 PA 442, MCL §15.231 et seq., and known as the Freedom of Information Act, the person may also be required to furnish the records directly to the USEPA together with a claim of confidentiality.  (R 336.1213(1)(e))

- A challenge by any person, the Administrator of the USEPA, or the department to a particular condition or a part of this ROP shall not set aside, delay, stay, or in any way affect the applicability or enforceability of any other condition or part of this ROP.  (R 336.1213(1)(f))

- The permittee shall pay fees consistent with the fee schedule and requirements pursuant to Section 5522 of Act 451.  (R 336.1213(1)(g))

- This ROP does not convey any property rights or any exclusive privilege.  (R 336.1213(1)(h))

## Equipment & Design

- Any collected air contaminants shall be removed as necessary to maintain the equipment at the required operating efficiency.  The collection and disposal of air contaminants shall be performed in a manner so as to minimize the introduction of contaminants to the outer air.  Transport of collected air contaminants in Priority I and II areas requires the use of material handling methods specified in Rule 370(2).2  (R 336.1370)

- Any air cleaning device shall be installed, maintained, and operated in a satisfactory manner and in accordance with the Michigan Air Pollution Control rules and existing law.  (R 336.1910)

## Emission Limits

- Unless otherwise specified in this ROP, the permittee shall comply with Rule 301, which states, in part, “Except as provided in subrules 2, 3, and 4 of this rule, a person shall not cause or permit to be discharged into the outer air from a process or process equipment a visible emission of a density greater than the most stringent of the following:”2  (R 336.1301(1))

  - A 6-minute average of 20% opacity, except for one 6-minute average per hour of not more than 27% opacity.

  - A limit specified by an applicable federal new source performance standard.

The grading of visible emissions shall be determined in accordance with Rule 303.

- The permittee shall not cause or permit the emission of an air contaminant or water vapor in quantities that cause, alone or in reaction with other air contaminants, either of the following:

  - Injurious effects to human health or safety, animal life, plant life of significant economic value, or property.1  (R 336.1901(a))

  - Unreasonable interference with the comfortable enjoyment of life and property.1  (R 336.1901(b))

## Testing/Sampling

- The department may require the owner or operator of any source of an air contaminant to conduct acceptable performance tests, at the owner’s or operator’s expense, in accordance with Rule 1001 and Rule 1003, under any of the conditions listed in Rule 1001(1).2  (R 336.2001)

- Any required performance testing shall be conducted in accordance with Rule 1001(2), Rule 1001(3) and Rule 1003.  (R 336.2001(2), R 336.2001(3), R 336.2003(1))

- Any required test results shall be submitted to the Air Quality Division (AQD) in the format prescribed by the applicable reference test method within 60 days following the last date of the test.  (R 336.2001(5))

## Monitoring/Recordkeeping

- Records of any periodic emission or parametric monitoring required in this ROP shall include the following information specified in Rule 213(3)(b)(i), where appropriate.  (R 336.1213(3)(b))

  - The date, location, time, and method of sampling or measurements.

  - The dates the analyses of the samples were performed.

  - The company or entity that performed the analyses of the samples.

  - The analytical techniques or methods used.

  - The results of the analyses.

  - The related process operating conditions or parameters that existed at the time of sampling or measurement.

- All required monitoring data, support information and all reports, including reports of all instances of deviation from permit requirements, shall be kept and furnished to the department upon request for a period of not less than 5 years from the date of the monitoring sample, measurement, report or application.  Support information includes all calibration and maintenance records and all original strip-chart recordings, or other original data records, for continuous monitoring instrumentation and copies of all reports required by the ROP.  (R 336.1213(1)(e), R 336.1213(3)(b)(ii))

## Certification & Reporting

- Except for the alternate certification schedule provided in Rule 213(3)(c)(iii)(B), any document required to be submitted to the department as a term or condition of this ROP shall contain an original certification by a Responsible Official which states that, based on information and belief formed after reasonable inquiry, the statements and information in the document are true, accurate, and complete.  (R 336.1213(3)(c))

- A Responsible Official shall certify to the appropriate AQD District Office and to the USEPA that the stationary source is and has been in compliance with all terms and conditions contained in the ROP except for deviations that have been or are being reported to the appropriate AQD District Office pursuant to Rule 213(3)(c).  This certification shall include all the information specified in Rule 213(4)(c)(i) through (v) and shall state that, based on information and belief formed after reasonable inquiry, the statements and information in the certification are true, accurate, and complete.  The USEPA address is:  USEPA, Air Compliance Data - Michigan, Air and Radiation Division, 77 West Jackson Boulevard, Chicago, Illinois 60604-3507.  (R 336.1213(4)(c))

- The certification of compliance shall be submitted annually for the term of this ROP as detailed in the special conditions, or more frequently if specified in an applicable requirement or in this ROP.  (R 336.1213(4)(c))

- The permittee shall promptly report any deviations from ROP requirements and certify the reports.  The prompt reporting of deviations from ROP requirements is defined in Rule 213(3)(c)(ii) as follows, unless otherwise described in this ROP.  (R 336.1213(3)(c))

  - For deviations that exceed the emissions allowed under the ROP, prompt reporting means reporting consistent with the requirements of Rule 912 as detailed in Condition 25.  All reports submitted pursuant to this paragraph shall be promptly certified as specified in Rule 213(3)(c)(iii).

  - For deviations which exceed the emissions allowed under the ROP and which are not reported pursuant to Rule 912 due to the duration of the deviation, prompt reporting means the reporting of all deviations in the semiannual reports required by Rule 213(3)(c)(i).  The report shall describe reasons for each deviation and the actions taken to minimize or correct each deviation.

  - For deviations that do not exceed the emissions allowed under the ROP, prompt reporting means the reporting of all deviations in the semiannual reports required by Rule 213(3)(c)(i).  The report shall describe the reasons for each deviation and the actions taken to minimize or correct each deviation.

- For reports required pursuant to Rule 213(3)(c)(ii), prompt certification of the reports is described in Rule 213(3)(c)(iii) as either of the following:  (R 336.1213(3)(c))

  - Submitting a certification by a Responsible Official with each report which states that, based on information and belief formed after reasonable inquiry, the statements and information in the report are true, accurate, and complete.

  - Submitting, within 30 days following the end of a calendar month during which one or more prompt reports of deviations from the emissions allowed under the ROP were submitted to the department pursuant to Rule 213(3)(c)(ii), a certification by a Responsible Official which states that; “based on information and belief formed after reasonable inquiry, the statements and information contained in each of the reports submitted during the previous month were true, accurate, and complete.”  The certification shall include a listing of the reports that are being certified.  Any report submitted pursuant to Rule 213(3)(c)(ii) that will be certified on a monthly basis pursuant to this paragraph shall include a statement that certification of the report will be provided within 30 days following the end of the calendar month.

- Semiannually for the term of the ROP as detailed in the special conditions, or more frequently if specified, the permittee shall submit certified reports of any required monitoring to the appropriate AQD District Office.  All instances of deviations from ROP requirements during the reporting period shall be clearly identified in the reports.  (R 336.1213(3)(c)(i))

- On an annual basis, the permittee shall report the actual emissions, or the information necessary to determine the actual emissions, of each regulated air pollutant as defined in Rule 212(6) for each emission unit utilizing the emissions inventory forms provided by the department.  (R 336.1212(6))

- The permittee shall provide notice of an abnormal condition, start-up, shutdown, or malfunction that results in emissions of a hazardous or toxic air pollutant which continue for more than one hour in excess of any applicable standard or limitation, or emissions of any air contaminant continuing for more than two hours in excess of an applicable standard or limitation, as required in Rule 912, to the appropriate AQD District Office.  The notice shall be provided not later than two business days after the start-up, shutdown, or discovery of the abnormal conditions or malfunction.  Notice shall be by any reasonable means, including electronic, telephonic, or oral communication.  Written reports, if required under Rule 912, must be submitted to the appropriate AQD District Supervisor within 10 days after the start-up or shutdown occurred, within 10 days after the abnormal conditions or malfunction has been corrected, or within 30 days of discovery of the abnormal conditions or malfunction, whichever is first.  The written reports shall include all of the information required in Rule 912(5) and shall be certified by a Responsible Official in a manner consistent with the CAA.2  (R 336.1912)

## Permit Shield

- Compliance with the conditions of the ROP shall be considered compliance with any applicable requirements as of the date of ROP issuance, if either of the following provisions is satisfied.  (R 336.1213(6)(a)(i), R 336.1213(6)(a)(ii))

  - The applicable requirements are included and are specifically identified in the ROP.

  - The permit includes a determination or concise summary of the determination by the department that other specifically identified requirements are not applicable to the stationary source.

Any requirements identified in Part E of this ROP have been identified as non-applicable to this ROP and are included in the permit shield.

- Nothing in this ROP shall alter or affect any of the following:

  - The provisions of Section 303 of the CAA, emergency orders, including the authority of the USEPA under Section 303 of the CAA.  (R 336.1213(6)(b)(i))

  - The liability of the owner or operator of this source for any violation of applicable requirements prior to or at the time of this ROP issuance.  (R 336.1213(6)(b)(ii))

  - The applicable requirements of the acid rain program, consistent with Section 408(a) of the CAA.  (R 336.1213(6)(b)(iii))

  - The ability of the USEPA to obtain information from a source pursuant to Section 114 of the CAA.  (R 336.1213(6)(b)(iv))

- The permit shield shall not apply to provisions incorporated into this ROP through procedures for any of the following:

  - Operational flexibility changes made pursuant to Rule 215.  (R 336.1215(5))

  - Administrative Amendments made pursuant to Rule 216(1)(a)(i)-(iv).  (R 336.1216(1)(b)(iii))

  - Administrative Amendments made pursuant to Rule 216(1)(a)(v) until the amendment has been approved by the department.  (R 336.1216(1)(c)(iii))

  - Minor Permit Modifications made pursuant to Rule 216(2).  (R 336.1216(2)(f))

  - State-Only Modifications made pursuant to Rule 216(4) until the changes have been approved by the department.  (R 336.1216(4)(e))

- Expiration of this ROP results in the loss of the permit shield.  If a timely and administratively complete application for renewal is submitted not more than 18 months, but not less than 6 months, before the expiration date of the ROP, but the department fails to take final action before the end of the ROP term, the existing ROP does not expire until the renewal is issued or denied, and the permit shield shall extend beyond the original ROP term until the department takes final action.  (R 336.1217(1)(c), R 336.1217(1)(a))

## Revisions

- For changes to any process or process equipment covered by this ROP that do not require a revision of the ROP pursuant to Rule 216, the permittee must comply with Rule 215.  (R 336.1215, R 336.1216)

- A change in ownership or operational control of a stationary source covered by this ROP shall be made pursuant to Rule 216(1).  (R 336.1219(2))

- For revisions to this ROP, an administratively complete application shall be considered timely if it is received by the department in accordance with the time frames specified in Rule 216.  (R 336.1210(10))

- Pursuant to Rule 216(1)(b)(iii), Rule 216(2)(d) and Rule 216(4)(d), after a change has been made, and until the department takes final action, the permittee shall comply with both the applicable requirements governing the change and the ROP terms and conditions proposed in the application for the modification.  During this time period, the permittee may choose to not comply with the existing ROP terms and conditions that the application seeks to change.  However, if the permittee fails to comply with the ROP terms and conditions proposed in the application during this time period, the terms and conditions in the ROP are enforceable.  (R 336.1216(1)(c)(iii), R 336.1216(2)(d), R 336.1216(4)(d))

## Reopenings

- A ROP shall be reopened by the department prior to the expiration date and revised by the department under any of the following circumstances:

  - If additional requirements become applicable to this stationary source with three or more years remaining in the term of the ROP, but not if the effective date of the new applicable requirement is later than the ROP expiration date.  (R 336.1217(2)(a)(i))

  - If additional requirements pursuant to Title IV of the CAA become applicable to this stationary source.  (R 336.1217(2)(a)(ii))

  - If the department determines that the ROP contains a material mistake, information required by any applicable requirement was omitted, or inaccurate statements were made in establishing emission limits or the terms or conditions of the ROP.  (R 336.1217(2)(a)(iii))

  - If the department determines that the ROP must be revised to ensure compliance with the applicable requirements.  (R 336.1217(2)(a)(iv))

## Renewals

- For renewal of this ROP, an administratively complete application shall be considered timely if it is received by the department not more than 18 months, but not less than 6 months, before the expiration date of the ROP.  (R 336.1210(9))

## Stratospheric Ozone Protection

- If the permittee is subject to Title 40 of the Code of Federal Regulations (CFR), Part 82 and services, maintains, or repairs appliances except for motor vehicle air conditioners (MVAC), or disposes of appliances containing refrigerant, including MVAC and small appliances, or if the permittee is a refrigerant reclaimer, appliance owner or a manufacturer of appliances or recycling and recovery equipment, the permittee shall comply with all applicable standards for recycling and emissions reduction pursuant to 40 CFR Part 82, Subpart F.

- If the permittee is subject to 40 CFR Part 82, and performs a service on motor (fleet) vehicles when this service involves refrigerant in the MVAC, the permittee is subject to all the applicable requirements as specified in 40 CFR Part 82, Subpart B, Servicing of Motor Vehicle Air Conditioners.  The term “motor vehicle” as used in Subpart B does not include a vehicle in which final assembly of the vehicle has not been completed by the original equipment manufacturer.  The term MVAC as used in Subpart B does not include the air-tight sealed refrigeration system used for refrigerated cargo or an air conditioning system on passenger buses using Hydrochlorofluorocarbon-22 refrigerant.

## Risk Management Plan

- If subject to Section 112(r) of the CAA and 40 CFR Part 68, the permittee shall register and submit to the USEPA the required data related to the risk management plan for reducing the probability of accidental releases of any regulated substances listed pursuant to Section 112(r)(3) of the CAA as amended in 40 CFR 68.130.  The list of substances, threshold quantities, and accident prevention regulations promulgated under 40 CFR Part 68, do not limit in any way the general duty provisions under Section 112(r)(1).

- If subject to Section 112(r) of the CAA and 40 CFR Part 68, the permittee shall comply with the requirements of 40 CFR Part 68, no later than the latest of the following dates as provided in 40 CFR 68.10(a):

  - June 21, 1999,

  - Three years after the date on which a regulated substance is first listed under 40 CFR 68.130, or

  - The date on which a regulated substance is first present above a threshold quantity in a process.

- If subject to Section 112(r) of the CAA and 40 CFR Part 68, the permittee shall submit any additional relevant information requested by any regulatory agency necessary to ensure compliance with the requirements of 40 CFR Part 68.

- If subject to Section 112(r) of the CAA and 40 CFR Part 68, the permittee shall annually certify compliance with all applicable requirements of Section 112(r) as detailed in Rule 213(4)(c)).  (40 CFR Part 68)

## Emission Trading

- Emission averaging and emission reduction credit trading are allowed pursuant to any applicable interstate or regional emission trading program that has been approved by the Administrator of the USEPA as a part of Michigan’s State Implementation Plan.  Such activities must comply with Rule 215 and Rule 216. (R 336.1213(12))

## Permit to Install (PTI)

- The process or process equipment included in this permit shall not be reconstructed, relocated, or modified unless a PTI authorizing such action is issued by the department, except to the extent such action is exempt from the PTI requirements by any applicable rule.2  (R 336.1201(1))

- The department may, after notice and opportunity for a hearing, revoke PTI terms or conditions if evidence indicates the process or process equipment is not performing in accordance with the terms and conditions of the PTI or is violating the department’s rules or the CAA.2  (R 336.1201(8), Section 5510 of Act 451)

- The terms and conditions of a PTI shall apply to any person or legal entity that now or hereafter owns or operates the process or process equipment at the location authorized by the PTI.  If a new owner or operator submits a written request to the department pursuant to Rule 219 and the department approves the request, this PTI will be amended to reflect the change of ownership or operational control.  The request must include all of the information required by Subrules (1)(a), (b) and (c) of Rule 219.  The written request shall be sent to the appropriate AQD District Supervisor, MDEQ.2  (R 336.1219)

- If the installation, reconstruction, relocation, or modification of the equipment for which PTI terms and conditions have been approved has not commenced within 18 months of the original PTI issuance date, or has been interrupted for 18 months, the applicable terms and conditions from that PTI, as incorporated into the ROP, shall become void unless otherwise authorized by the department.  Furthermore, the person to whom that PTI was issued, or the designated authorized agent, shall notify the department via the Supervisor, Permit Section, MDEQ, AQD, P. O. Box 30260, Lansing, Michigan 48909, if it is decided not to pursue the installation, reconstruction, relocation, or modification of the equipment allowed by the terms and conditions from that PTI.2  (R 336.1201(4))

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

# B.  SOURCE-WIDE CONDITIONS

Part B outlines the Source-Wide Terms and Conditions that apply to this stationary source.  The permittee is subject to these special conditions for the stationary source in addition to the general conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply to this source, NA (not applicable) has been used in the table.  If there are no Source-Wide Conditions, this section will be left blank.

# C.  EMISSION UNIT CONDITIONS

Part C outlines terms and conditions that are specific to individual emission units listed in the Emission Unit Summary Table.  The permittee is subject to the special conditions for each emission unit in addition to the General Conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply, NA (not applicable) has been used in the table.  If there are no conditions specific to individual emission units, this section will be left blank.

# D.  FLEXIBLE GROUP CONDITIONS

Part D outlines the terms and conditions that apply to more than one emission unit.  The permittee is subject to the special conditions for each flexible group in addition to the General Conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply, NA (not applicable) has been used in the table.  If there are no special conditions that apply to more than one emission unit, this section will be left blank.

# E.  NON-APPLICABLE REQUIREMENTS

At the time of the ROP issuance, the AQD has determined that no non-applicable requirements have been identified for incorporation into the permit shield provision set forth in the General Conditions in Part A pursuant to Rule 213(6)(a)(ii).

# APPENDICES

## Appendix 1-2.  Acronyms and Abbreviations

| Common Acronyms |  | Pollutant / Measurement Abbreviations |  |
| --- | --- | --- | --- |
| AQD | Air Quality Division | acfm | Actual cubic feet per minute |
| BACT | Best Available Control Technology | BTU | British Thermal Unit |
| CAA | Clean Air Act | °C | Degrees Celsius |
| CAM | Compliance Assurance Monitoring | CO | Carbon Monoxide |
| CEM | Continuous Emission Monitoring | CO2e | Carbon Dioxide Equivalent |
| CFR | Code of Federal Regulations | dscf | Dry standard cubic foot |
| COM | Continuous Opacity Monitoring | dscm | Dry standard cubic meter |
| Department/ department | Michigan Department of Environmental Quality | °F | Degrees Fahrenheit |
|  |  | gr | Grains |
| EU | Emission Unit | HAP | Hazardous Air Pollutant |
| FG | Flexible Group | Hg | Mercury |
| GACS | Gallons of Applied Coating Solids | hr | Hour |
| GC | General Condition | HP | Horsepower |
| GHGs | Greenhouse Gases | H2S | Hydrogen Sulfide |
| HVLP | High Volume Low Pressure* | kW | Kilowatt |
| ID | Identification | lb | Pound |
| IRSL | Initial Risk Screening Level | m | Meter |
| ITSL | Initial Threshold Screening Level | mg | Milligram |
| LAER | Lowest Achievable Emission Rate | mm | Millimeter |
| MACT | Maximum Achievable Control Technology | MM | Million |
| MAERS | Michigan Air Emissions Reporting System | MW | Megawatts |
| MAP | Malfunction Abatement Plan | NMOC | Non-methane Organic Compounds |
| MDEQ | Michigan Department of Environmental Quality | NOx | Oxides of Nitrogen |
|  |  | ng | Nanogram |
| MSDS | Material Safety Data Sheet | PM | Particulate Matter |
| NA | Not Applicable | PM10 | Particulate Matter equal to or less than 10 microns in diameter |
| NAAQS | National Ambient Air Quality Standards |  |  |
| NESHAP | National Emission Standard for Hazardous Air Pollutants | PM2.5 | Particulate Matter equal to or less than 2.5 microns in diameter |
| NSPS | New Source Performance Standards | pph | Pounds per hour |
| NSR | New Source Review | ppm | Parts per million |
| PS | Performance Specification | ppmv | Parts per million by volume |
| PSD | Prevention of Significant Deterioration | ppmw | Parts per million by weight |
| PTE | Permanent Total Enclosure | psia | Pounds per square inch absolute |
| PTI | Permit to Install | psig | Pounds per square inch gauge |
| RACT | Reasonable Available Control Technology | scf | Standard cubic feet |
| ROP | Renewable Operating Permit | sec | Seconds |
| SC | Special Condition | SO2 | Sulfur Dioxide |
| SCR | Selective Catalytic Reduction | TAC | Toxic Air Contaminant |
| SNCR | Selective Non-Catalytic Reduction | Temp | Temperature |
| SRN | State Registration Number | THC | Total Hydrocarbons |
| TEQ | Toxicity Equivalence Quotient | tpy | Tons per year |
| USEPA/EPA | United States Environmental Protection Agency | µg | Microgram |
|  |  | µm | Micrometer or Micron |
| VE | Visible Emissions | VOC | Volatile Organic Compounds |
|  |  | yr | Year |

*For HVLP applicators, the pressure measured at the gun air cap shall not exceed 10 psig.

## Appendix 2-2.  Schedule of Compliance

The permittee certified in the ROP application that this stationary source is in compliance with all applicable requirements and the permittee shall continue to comply with all terms and conditions of this ROP.  A Schedule of Compliance is not required.  (R 336.1213(4)(a), R 336.1119(a)(ii))

## Appendix 3-2.  Monitoring Requirements

Specific monitoring requirement procedures, methods or specifications are detailed in Part A or the appropriate Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Therefore, this appendix is not applicable.

## Appendix 4-2.  Recordkeeping

Specific recordkeeping requirement formats and procedures are detailed in Part A or the appropriate Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Therefore, this appendix is not applicable.

## Appendix 5-2.  Testing Procedures

There are no specific testing requirement plans or procedures for this ROP.  Therefore, this appendix is not applicable.

## Appendix 6-2.  Permits to Install

The following table lists any PTIs issued or ROP revision applications received since the effective date of the previously issued ROP No. MI-ROP-B2063-2012. Those ROP revision applications that are being issued concurrently with this ROP renewal are identified by an asterisk (*).  Those revision applications not listed with an asterisk were processed prior to this renewal.

Source-Wide PTI No MI-PTI-B2063-2012b is being reissued as Source-Wide PTI No. MI-PTI-B2063-2018.

| Permit to Install Number | ROP Revision Application Number | Description of Equipment or Change | Corresponding Emission Unit(s) or Flexible Group(s) |
| --- | --- | --- | --- |
| 68-13 | 201300150 | Soil vapor extraction operation occurring at Operable Unit 6 (former Paint Sludge Pit). | EU-OU-6 |

## Appendix 7-2.  Emission Calculations

There are no specific emission calculations to be used for this ROP.  Therefore, this appendix is not applicable.

## Appendix 8-2.  Reporting

A.  Annual, Semiannual, and Deviation Certification Reporting

The permittee shall use the MDEQ, AQD, Report Certification form (EQP 5736) and MDEQ, AQD, Deviation Report form (EQP 5737) for the annual, semiannual and deviation certification reporting referenced in the Reporting Section of the Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Alternative formats must meet the provisions of Rule 213(4)(c) and Rule 213(3)(c)(i), respectively, and be approved by the AQD District Supervisor.

B.  Other Reporting

Specific reporting requirement formats and procedures are detailed in Part A or the appropriate Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Therefore, Part B of this appendix is not applicable.