|  | MICHIGAN DEPARTMENT OF ENVIRONMENTAL QUALITY AIR QUALITY DIVISION |  |
| --- | --- | --- |
| EFFECTIVE DATE: February 27, 2019 ISSUED TO Molded Plastics Industries, Incorporated State Registration Number (SRN): N0034 LOCATED AT 2382 Jarco Drive, Holt, Ingham County, Michigan 48842 |  |  |
| RENEWABLE OPERATING PERMIT Permit Number: MI-ROP-N0034-2019 Expiration Date: February 27, 2024 Administratively Complete ROP Renewal Application Due Between August 27, 2022 and August 27, 2023 This Renewable Operating Permit (ROP) is issued in accordance with and subject to Section 5506(3) of Part 55, Air Pollution Control, of the Natural Resources and Environmental Protection Act, 1994 PA 451, as amended (Act 451). Pursuant to Rule 210(1) of the administrative rules promulgated under Act 451, this ROP constitutes the permittee’s authority to operate the stationary source identified above in accordance with the general conditions, special conditions and attachments contained herein. Operation of the stationary source and all emission units listed in the permit are subject to all applicable future or amended rules and regulations pursuant to Act 451 and the federal Clean Air Act. |  |  |

SOURCE-WIDE PERMIT TO INSTALL Permit Number: MI-PTI-N0034-2019 This Permit to Install (PTI) is issued in accordance with and subject to Section 5505(1) of Act 451. Pursuant to Rule 214a of the administrative rules promulgated under Act 451, the terms and conditions herein, identified by the underlying applicable requirement citation of Rule 201(1)(a), constitute a federally enforceable PTI. The PTl terms and conditions do not expire and remain in effect unless the criteria of Rule 201(6) are met. Operation of all emission units identified in the PTI is subject to all applicable future or amended rules and regulations pursuant to Act 451 and the federal Clean Air Act.

Michigan Department of Environmental Quality

______________________________________

Brad Myott, Lansing District Supervisor

TABLE OF CONTENTS

AUTHORITY AND ENFORCEABILITY	3

A.  GENERAL CONDITIONS	4

Permit Enforceability	4

General Provisions	4

Equipment & Design	5

Emission Limits	5

Testing/Sampling	5

Monitoring/Recordkeeping	6

Certification & Reporting	6

Permit Shield	7

Revisions	8

Reopenings	8

Renewals	9

Stratospheric Ozone Protection	9

Risk Management Plan	9

Emission Trading	9

Permit to Install (PTI)	10

B.  SOURCE-WIDE CONDITIONS	11

C.  EMISSION UNIT SPECIAL CONDITIONS	14

EMISSION UNIT SUMMARY TABLE	14

EU-SPRAYBOOTHGL1	15

EU-SPRAYBOOTHRS1	18

D.  FLEXIBLE GROUP SPECIAL CONDITIONS	21

FLEXIBLE GROUP SUMMARY TABLE	21

FG-COMPOSITESMACT	23

FG-MACT PPPP	26

FG-MOLDRELEASE	30

FG-MACT DDDDD	32

FG-RULE 287(2)(c)	36

FG-COLDCLEANERS	38

E.  NON-APPLICABLE REQUIREMENTS	40

APPENDICES	41

Appendix 1.  Acronyms and Abbreviations	41

Appendix 2.  Schedule of Compliance	42

Appendix 3.  Monitoring Requirements	42

Appendix 4.  Recordkeeping	42

Appendix 5.  Testing Procedures	43

Appendix 6.  Permits to Install	43

Appendix 7.  Emission Calculations	43

Appendix 8.  Reporting	43

Appendix 9.  MACT - Subpart WWWW Tables 1 and 3	44

# AUTHORITY AND ENFORCEABILITY

For the purpose of this permit, the permittee is defined as any person who owns or operates an emission unit at a stationary source for which this permit has been issued.  The department is defined in Rule 104(d) as the Director of the Michigan Department of Environmental Quality (MDEQ) or his or her designee.

The permittee shall comply with all specific details in the permit terms and conditions and the cited underlying applicable requirements.  All terms and conditions in this ROP are both federally enforceable and state enforceable unless otherwise footnoted.  Certain terms and conditions are applicable to most stationary sources for which an ROP has been issued.  These general conditions are included in Part A of this ROP.  Other terms and conditions may apply to a specific emission unit, several emission units which are represented as a flexible group, or the entire stationary source which is represented as a Source-Wide group.  Special conditions are identified in Parts B, C, D and/or the appendices.

In accordance with Rule 213(2)(a), all underlying applicable requirements are identified for each ROP term or condition.  All terms and conditions that are included in a PTI are streamlined, subsumed and/or is state-only enforceable will be noted as such.

In accordance with Section 5507 of Act 451, the permittee has included in the ROP application a compliance certification, a schedule of compliance, and a compliance plan.  For applicable requirements with which the source is in compliance, the source will continue to comply with these requirements.  For applicable requirements with which the source is not in compliance, the source will comply with the detailed schedule of compliance requirements that are incorporated as an appendix in this ROP.  Furthermore, for any applicable requirements effective after the date of issuance of this ROP, the stationary source will meet the requirements on a timely basis, unless the underlying applicable requirement requires a more detailed schedule of compliance.

Issuance of this permit does not obviate the necessity of obtaining such permits or approvals from other units of government as required by law.

# A.  GENERAL CONDITIONS

## Permit Enforceability

- All conditions in this permit are both federally enforceable and state enforceable unless otherwise noted. (R 336.1213(5))

- Those conditions that are hereby incorporated in a state-only enforceable Source-Wide PTI pursuant to Rule 201(2)(d) are designated by footnote one.  (R 336.1213(5)(a), R 336.1214a(5))

- Those conditions that are hereby incorporated in a federally enforceable Source-Wide PTI pursuant to Rule 201(2)(c) are designated by footnote two.  (R 336.1213(5)(b), R 336.1214a(3))

## General Provisions

- The permittee shall comply with all conditions of this ROP.  Any ROP noncompliance constitutes a violation of Act 451, and is grounds for enforcement action, for ROP revocation or revision, or for denial of the renewal of the ROP.  All terms and conditions of this ROP that are designated as federally enforceable are enforceable by the Administrator of the United States Environmental Protection Agency (USEPA) and by citizens under the provisions of the federal Clean Air Act (CAA).  Any terms and conditions based on applicable requirements which are designated as “state-only” are not enforceable by the USEPA or citizens pursuant to the CAA.  (R 336.1213(1)(a))

- It shall not be a defense for the permittee in an enforcement action that it would have been necessary to halt or reduce the permitted activity in order to maintain compliance with the conditions of this ROP.  (R 336.1213(1)(b))

- This ROP may be modified, revised, or revoked for cause.  The filing of a request by the permittee for a permit modification, revision, or termination, or a notification of planned changes or anticipated noncompliance does not stay any ROP term or condition.  This does not supersede or affect the ability of the permittee to make changes, at the permittee’s own risk, pursuant to Rule 215 and Rule 216.  (R 336.1213(1)(c))

- The permittee shall allow the department, or an authorized representative of the department, upon presentation of credentials and other documents as may be required by law and upon stating the authority for and purpose of the investigation, to perform any of the following activities:  (R 336.1213(1)(d))

  - Enter, at reasonable times, a stationary source or other premises where emissions-related activity is conducted or where records must be kept under the conditions of the ROP.

  - Have access to and copy, at reasonable times, any records that must be kept under the conditions of the ROP.

  - Inspect, at reasonable times, any of the following:

    - Any stationary source.

    - Any emission unit.

    - Any equipment, including monitoring and air pollution control equipment.

    - Any work practices or operations regulated or required under the ROP.

  - As authorized by Section 5526 of Act 451, sample or monitor at reasonable times substances or parameters for the purpose of assuring compliance with the ROP or applicable requirements.

- The permittee shall furnish to the department, within a reasonable time, any information the department may request, in writing, to determine whether cause exists for modifying, revising, or revoking the ROP or to determine compliance with this ROP.  Upon request, the permittee shall also furnish to the department copies of any records that are required to be kept as a term or condition of this ROP.  For information which is claimed by the permittee to be confidential, consistent with the requirements of the 1976 PA 442, MCL §15.231 et seq., and known as the Freedom of Information Act, the person may also be required to furnish the records directly to the USEPA together with a claim of confidentiality.  (R 336.1213(1)(e))

- A challenge by any person, the Administrator of the USEPA, or the department to a particular condition or a part of this ROP shall not set aside, delay, stay, or in any way affect the applicability or enforceability of any other condition or part of this ROP.  (R 336.1213(1)(f))

- The permittee shall pay fees consistent with the fee schedule and requirements pursuant to Section 5522 of Act 451.  (R 336.1213(1)(g))

- This ROP does not convey any property rights or any exclusive privilege.  (R 336.1213(1)(h))

## Equipment & Design

- Any collected air contaminants shall be removed as necessary to maintain the equipment at the required operating efficiency.  The collection and disposal of air contaminants shall be performed in a manner so as to minimize the introduction of contaminants to the outer air.  Transport of collected air contaminants in Priority I and II areas requires the use of material handling methods specified in Rule 370(2).2  (R 336.1370)

- Any air cleaning device shall be installed, maintained, and operated in a satisfactory manner and in accordance with the Michigan Air Pollution Control rules and existing law.  (R 336.1910)

## Emission Limits

- Unless otherwise specified in this ROP, the permittee shall comply with Rule 301, which states, in part, “Except as provided in Subrules 2, 3, and 4 of this rule, a person shall not cause or permit to be discharged into the outer air from a process or process equipment a visible emission of a density greater than the most stringent of the following:”2  (R 336.1301(1))

  - A 6-minute average of 20% opacity, except for one 6-minute average per hour of not more than 27% opacity.

  - A limit specified by an applicable federal new source performance standard.

The grading of visible emissions shall be determined in accordance with Rule 303.

- The permittee shall not cause or permit the emission of an air contaminant or water vapor in quantities that cause, alone or in reaction with other air contaminants, either of the following:

  - Injurious effects to human health or safety, animal life, plant life of significant economic value, or property.1  (R 336.1901(a))

  - Unreasonable interference with the comfortable enjoyment of life and property.1  (R 336.1901(b))

## Testing/Sampling

- The department may require the owner or operator of any source of an air contaminant to conduct acceptable performance tests, at the owner’s or operator’s expense, in accordance with Rule 1001 and Rule 1003, under any of the conditions listed in Rule 1001(1).2  (R 336.2001)

- Any required performance testing shall be conducted in accordance with Rule 1001(2), Rule 1001(3) and Rule 1003.  (R 336.2001(2), R 336.2001(3), R 336.2003(1))

- Any required test results shall be submitted to the Air Quality Division (AQD) in the format prescribed by the applicable reference test method within 60 days following the last date of the test.  (R 336.2001(5))

## Monitoring/Recordkeeping

- Records of any periodic emission or parametric monitoring required in this ROP shall include the following information specified in Rule 213(3)(b)(i), where appropriate.  (R 336.1213(3)(b))

  - The date, location, time, and method of sampling or measurements.

  - The dates the analyses of the samples were performed.

  - The company or entity that performed the analyses of the samples.

  - The analytical techniques or methods used.

  - The results of the analyses.

  - The related process operating conditions or parameters that existed at the time of sampling or measurement.

- All required monitoring data, support information and all reports, including reports of all instances of deviation from permit requirements, shall be kept and furnished to the department upon request for a period of not less than 5 years from the date of the monitoring sample, measurement, report or application.  Support information includes all calibration and maintenance records and all original strip-chart recordings, or other original data records, for continuous monitoring instrumentation and copies of all reports required by the ROP.  (R 336.1213(1)(e), R 336.1213(3)(b)(ii))

## Certification & Reporting

- Except for the alternate certification schedule provided in Rule 213(3)(c)(iii)(B), any document required to be submitted to the department as a term or condition of this ROP shall contain an original certification by a Responsible Official which states that, based on information and belief formed after reasonable inquiry, the statements and information in the document are true, accurate, and complete.  (R 336.1213(3)(c))

- A Responsible Official shall certify to the appropriate AQD District Office and to the USEPA that the stationary source is and has been in compliance with all terms and conditions contained in the ROP except for deviations that have been or are being reported to the appropriate AQD District Office pursuant to Rule 213(3)(c).  This certification shall include all the information specified in Rule 213(4)(c)(i) through (v) and shall state that, based on information and belief formed after reasonable inquiry, the statements and information in the certification are true, accurate, and complete.  The USEPA address is:  USEPA, Air Compliance Data - Michigan, Air and Radiation Division, 77 West Jackson Boulevard, Chicago, Illinois 60604-3507.  (R 336.1213(4)(c))

- The certification of compliance shall be submitted annually for the term of this ROP as detailed in the special conditions, or more frequently if specified in an applicable requirement or in this ROP.  (R 336.1213(4)(c))

- The permittee shall promptly report any deviations from ROP requirements and certify the reports.  The prompt reporting of deviations from ROP requirements is defined in Rule 213(3)(c)(ii) as follows, unless otherwise described in this ROP.  (R 336.1213(3)(c))

  - For deviations that exceed the emissions allowed under the ROP, prompt reporting means reporting consistent with the requirements of Rule 912 as detailed in Condition 25.  All reports submitted pursuant to this paragraph shall be promptly certified as specified in Rule 213(3)(c)(iii).

  - For deviations which exceed the emissions allowed under the ROP and which are not reported pursuant to Rule 912 due to the duration of the deviation, prompt reporting means the reporting of all deviations in the semiannual reports required by Rule 213(3)(c)(i).  The report shall describe reasons for each deviation and the actions taken to minimize or correct each deviation.

  - For deviations that do not exceed the emissions allowed under the ROP, prompt reporting means the reporting of all deviations in the semiannual reports required by Rule 213(3)(c)(i).  The report shall describe the reasons for each deviation and the actions taken to minimize or correct each deviation.

- For reports required pursuant to Rule 213(3)(c)(ii), prompt certification of the reports is described in Rule 213(3)(c)(iii) as either of the following:  (R 336.1213(3)(c))

  - Submitting a certification by a Responsible Official with each report which states that, based on information and belief formed after reasonable inquiry, the statements and information in the report are true, accurate, and complete.

  - Submitting, within 30 days following the end of a calendar month during which one or more prompt reports of deviations from the emissions allowed under the ROP were submitted to the department pursuant to Rule 213(3)(c)(ii), a certification by a Responsible Official which states that; “based on information and belief formed after reasonable inquiry, the statements and information contained in each of the reports submitted during the previous month were true, accurate, and complete.”  The certification shall include a listing of the reports that are being certified.  Any report submitted pursuant to Rule 213(3)(c)(ii) that will be certified on a monthly basis pursuant to this paragraph shall include a statement that certification of the report will be provided within 30 days following the end of the calendar month.

- Semiannually for the term of the ROP as detailed in the special conditions, or more frequently if specified, the permittee shall submit certified reports of any required monitoring to the appropriate AQD District Office.  All instances of deviations from ROP requirements during the reporting period shall be clearly identified in the reports.  (R 336.1213(3)(c)(i))

- On an annual basis, the permittee shall report the actual emissions, or the information necessary to determine the actual emissions, of each regulated air pollutant as defined in Rule 212(6) for each emission unit utilizing the emissions inventory forms provided by the department.  (R 336.1212(6))

- The permittee shall provide notice of an abnormal condition, start-up, shutdown, or malfunction that results in emissions of a hazardous or toxic air pollutant which continue for more than one hour in excess of any applicable standard or limitation, or emissions of any air contaminant continuing for more than two hours in excess of an applicable standard or limitation, as required in Rule 912, to the appropriate AQD District Office.  The notice shall be provided not later than two business days after the start-up, shutdown, or discovery of the abnormal conditions or malfunction.  Notice shall be by any reasonable means, including electronic, telephonic, or oral communication.  Written reports, if required under Rule 912, must be submitted to the appropriate AQD District Supervisor within 10 days after the start-up or shutdown occurred, within 10 days after the abnormal conditions or malfunction has been corrected, or within 30 days of discovery of the abnormal conditions or malfunction, whichever is first.  The written reports shall include all of the information required in Rule 912(5) and shall be certified by a Responsible Official in a manner consistent with the CAA.2  (R 336.1912)

## Permit Shield

- Compliance with the conditions of the ROP shall be considered compliance with any applicable requirements as of the date of ROP issuance if either of the following provisions is satisfied.  (R 336.1213(6)(a)(i), R 336.1213(6)(a)(ii))

  - The applicable requirements are included and are specifically identified in the ROP.

  - The permit includes a determination or concise summary of the determination by the department that other specifically identified requirements are not applicable to the stationary source.

Any requirements identified in Part E of this ROP have been identified as non-applicable to this ROP and are included in the permit shield.

- Nothing in this ROP shall alter or affect any of the following:

  - The provisions of Section 303 of the CAA, emergency orders, including the authority of the USEPA under Section 303 of the CAA.  (R 336.1213(6)(b)(i))

  - The liability of the owner or operator of this source for any violation of applicable requirements prior to or at the time of this ROP issuance.  (R 336.1213(6)(b)(ii))

  - The applicable requirements of the acid rain program, consistent with Section 408(a) of the CAA.  (R 336.1213(6)(b)(iii))

  - The ability of the USEPA to obtain information from a source pursuant to Section 114 of the CAA.  (R 336.1213(6)(b)(iv))

- The permit shield shall not apply to provisions incorporated into this ROP through procedures for any of the following:

  - Operational flexibility changes made pursuant to Rule 215.  (R 336.1215(5))

  - Administrative Amendments made pursuant to Rule 216(1)(a)(i)-(iv).  (R 336.1216(1)(b)(iii))

  - Administrative Amendments made pursuant to Rule 216(1)(a)(v) until the amendment has been approved by the department.  (R 336.1216(1)(c)(iii))

  - Minor Permit Modifications made pursuant to Rule 216(2).  (R 336.1216(2)(f))

  - State-Only Modifications made pursuant to Rule 216(4) until the changes have been approved by the department.  (R 336.1216(4)(e))

- Expiration of this ROP results in the loss of the permit shield.  If a timely and administratively complete application for renewal is submitted not more than 18 months, but not less than 6 months, before the expiration date of the ROP, but the department fails to take final action before the end of the ROP term, the existing ROP does not expire until the renewal is issued or denied, and the permit shield shall extend beyond the original ROP term until the department takes final action.  (R 336.1217(1)(c), R 336.1217(1)(a))

## Revisions

- For changes to any process or process equipment covered by this ROP that do not require a revision of the ROP pursuant to Rule 216, the permittee must comply with Rule 215.  (R 336.1215, R 336.1216)

- A change in ownership or operational control of a stationary source covered by this ROP shall be made pursuant to Rule 216(1).  (R 336.1219(2))

- For revisions to this ROP, an administratively complete application shall be considered timely if it is received by the department in accordance with the time frames specified in Rule 216.  (R 336.1210(10))

- Pursuant to Rule 216(1)(b)(iii), Rule 216(2)(d) and Rule 216(4)(d), after a change has been made, and until the department takes final action, the permittee shall comply with both the applicable requirements governing the change and the ROP terms and conditions proposed in the application for the modification.  During this time period, the permittee may choose to not comply with the existing ROP terms and conditions that the application seeks to change.  However, if the permittee fails to comply with the ROP terms and conditions proposed in the application during this time period, the terms and conditions in the ROP are enforceable.  (R 336.1216(1)(c)(iii), R 336.1216(2)(d), R 336.1216(4)(d))

## Reopenings

- A ROP shall be reopened by the department prior to the expiration date and revised by the department under any of the following circumstances:

  - If additional requirements become applicable to this stationary source with three or more years remaining in the term of the ROP, but not if the effective date of the new applicable requirement is later than the ROP expiration date.  (R 336.1217(2)(a)(i))

  - If additional requirements pursuant to Title IV of the CAA become applicable to this stationary source.  (R 336.1217(2)(a)(ii))

  - If the department determines that the ROP contains a material mistake, information required by any applicable requirement was omitted, or inaccurate statements were made in establishing emission limits or the terms or conditions of the ROP.  (R 336.1217(2)(a)(iii))

  - If the department determines that the ROP must be revised to ensure compliance with the applicable requirements.  (R 336.1217(2)(a)(iv))

## Renewals

- For renewal of this ROP, an administratively complete application shall be considered timely if it is received by the department not more than 18 months, but not less than 6 months, before the expiration date of the ROP.  (R 336.1210(9))

## Stratospheric Ozone Protection

- If the permittee is subject to Title 40 of the Code of Federal Regulations (CFR), Part 82 and services, maintains, or repairs appliances except for motor vehicle air conditioners (MVAC), or disposes of appliances containing refrigerant, including MVAC and small appliances, or if the permittee is a refrigerant reclaimer, appliance owner or a manufacturer of appliances or recycling and recovery equipment, the permittee shall comply with all applicable standards for recycling and emissions reduction pursuant to 40 CFR Part 82, Subpart F.

- If the permittee is subject to 40 CFR Part 82 and performs a service on motor (fleet) vehicles when this service involves refrigerant in the MVAC, the permittee is subject to all the applicable requirements as specified in 40 CFR Part 82, Subpart B, Servicing of Motor Vehicle Air Conditioners.  The term “motor vehicle” as used in Subpart B does not include a vehicle in which final assembly of the vehicle has not been completed by the original equipment manufacturer.  The term MVAC as used in Subpart B does not include the air-tight sealed refrigeration system used for refrigerated cargo or an air conditioning system on passenger buses using Hydrochlorofluorocarbon-22 refrigerant.

## Risk Management Plan

- If subject to Section 112(r) of the CAA and 40 CFR Part 68, the permittee shall register and submit to the USEPA the required data related to the risk management plan for reducing the probability of accidental releases of any regulated substances listed pursuant to Section 112(r)(3) of the CAA as amended in 40 CFR 68.130.  The list of substances, threshold quantities, and accident prevention regulations promulgated under 40 CFR Part 68, do not limit in any way the general duty provisions under Section 112(r)(1).

- If subject to Section 112(r) of the CAA and 40 CFR Part 68, the permittee shall comply with the requirements of 40 CFR Part 68, no later than the latest of the following dates as provided in 40 CFR 68.10(a):

  - June 21, 1999,

  - Three years after the date on which a regulated substance is first listed under 40 CFR 68.130, or

  - The date on which a regulated substance is first present above a threshold quantity in a process.

- If subject to Section 112(r) of the CAA and 40 CFR Part 68, the permittee shall submit any additional relevant information requested by any regulatory agency necessary to ensure compliance with the requirements of 40 CFR Part 68.

- If subject to Section 112(r) of the CAA and 40 CFR Part 68, the permittee shall annually certify compliance with all applicable requirements of Section 112(r) as detailed in Rule 213(4)(c)).  (40 CFR Part 68)

## Emission Trading

- Emission averaging and emission reduction credit trading are allowed pursuant to any applicable interstate or regional emission trading program that has been approved by the Administrator of the USEPA as a part of Michigan’s State Implementation Plan.  Such activities must comply with Rule 215 and Rule 216. (R 336.1213(12))

## Permit to Install (PTI)

- The process or process equipment included in this permit shall not be reconstructed, relocated, or modified unless a PTI authorizing such action is issued by the department, except to the extent such action is exempt from the PTI requirements by any applicable rule.2  (R 336.1201(1))

- The department may, after notice and opportunity for a hearing, revoke PTI terms or conditions if evidence indicates the process or process equipment is not performing in accordance with the terms and conditions of the PTI or is violating the department’s rules or the CAA.2  (R 336.1201(8), Section 5510 of Act 451)

- The terms and conditions of a PTI shall apply to any person or legal entity that now or hereafter owns or operates the process or process equipment at the location authorized by the PTI.  If a new owner or operator submits a written request to the department pursuant to Rule 219 and the department approves the request, this PTI will be amended to reflect the change of ownership or operational control.  The request must include all of the information required by Subrules (1)(a), (b) and (c) of Rule 219.  The written request shall be sent to the appropriate AQD District Supervisor, MDEQ.2  (R 336.1219)

- If the installation, reconstruction, relocation, or modification of the equipment for which PTI terms and conditions have been approved has not commenced within 18 months of the original PTI issuance date, or has been interrupted for 18 months, the applicable terms and conditions from that PTI, as incorporated into the ROP, shall become void unless otherwise authorized by the department.  Furthermore, the person to whom that PTI was issued, or the designated authorized agent, shall notify the department via the Supervisor, Permit Section, MDEQ, AQD, P. O. Box 30260, Lansing, Michigan 48909, if it is decided not to pursue the installation, reconstruction, relocation, or modification of the equipment allowed by the terms and conditions from that PTI.2  (R 336.1201(4))

Footnotes:

1This condition is state-only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

# B.  SOURCE-WIDE CONDITIONS

Part B outlines the Source-Wide Terms and Conditions that apply to this stationary source.  The permittee is subject to these special conditions for the stationary source in addition to the general conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply to this source, NA (not applicable) has been used in the table.  If there are no Source-Wide Conditions, this section will be left blank.

SOURCE-WIDE CONDITIONS

DESCRIPTION

All process equipment source-wide including equipment covered by other permits, grandfathered equipment and exempt equipment.

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMITS

| Pollutant | Limit | Time Period/ Operating Scenario | Equipment | Testing/ Monitoring Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- | --- |
| - Styrene (CAS# 100-42-5) | 50.0 tpy1 | 12-month rolling time period as determined at the end of each calendar month. | All process equipment source-wide | SC VI.1 | R 336.1225(2) |
| - Ethylbenzene (CAS# 100-41-4) | 12.5 tpy1 | 12-month rolling time period as determined at the end of each calendar month. | All process equipment source-wide | SC VI.2 | R 336.1225(2) |

II.  MATERIAL LIMITS

NA

III.  PROCESS/OPERATIONAL RESTRICTIONS

NA

IV.  DESIGN/EQUIPMENT PARAMETERS

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1201(3))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1201(3))

- The permittee shall keep the following information on a calendar month basis for all process equipment source-wide:

a.	The identity and amount (in pounds) of each styrene containing material used.

b.	The styrene content of each material used.

c.	The appropriate emission factor for each styrene containing material used (The Unified Emission Factors (UEF-1-2011a) Table 1 for Open Molding of Composites from the American Composites Manufacturers Association (ACMA), October 2011 may be used, or an alternate factor approved by the AQD District Supervisor may be used).

d.	Styrene emission calculations determining the monthly emission rate in tons per calendar month.

e.	Styrene emission calculations determining the annual emission rate in tons per 12-month rolling time period as determined at the end of each calendar month.

The permittee shall keep records using mass balance or an alternate method and format acceptable to the AQD District Supervisor.  The permittee shall keep all records on file and make them available to the Department upon request.1  (R 336.1225)

- The permittee shall keep the following information on a calendar month basis for all process equipment source-wide:

a.	The identity and amount (in pounds) of each ethylbenzene containing material used.

b.	The ethylbenzene content of each ethylbenzene containing material used.

d.	Ethylbenzene emission calculations determining the monthly emission rate in tons per calendar month.

e.	Ethylbenzene emission calculations determining the annual emission rate in tons per 12-month rolling time period as determined at the end of each calendar month.

The permittee shall keep records using mass balance or an alternate method and format acceptable to the AQD District Supervisor.  The permittee shall keep all records on file and make them available to the Department upon request.1  (R 336.1225)

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8

VIII.  STACK/VENT RESTRICTIONS

NA

IX.  OTHER REQUIREMENTS

NA

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

# C.  EMISSION UNIT SPECIAL CONDITIONS

Part C outlines terms and conditions that are specific to individual emission units listed in the Emission Unit Summary Table.  The permittee is subject to the special conditions for each emission unit in addition to the General Conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply, NA (not applicable) has been used in the table.  If there are no conditions specific to individual emission units, this section will be left blank.

## EMISSION UNIT SUMMARY TABLE

The descriptions provided below are for informational purposes and do not constitute enforceable conditions.

| Emission Unit ID | Emission Unit Description (Including Process Equipment & Control Device(s)) | Installation Date/ Modification Date | Flexible Group ID |
| --- | --- | --- | --- |
| EU-SPRAYBOOTHGL1 | Spray booth for application of gel coat on open molds. | 11-1991 | FG-MOLDRELEASE FG-COMPOSITESMACT |
| EU-SPRAYBOOTHRS1 | Spray booth for application of resin on molds. | 11-1991 | FG-MOLDRELEASE FG-COMPOSITESMACT |
| EU-PAINTBOOTH | Spray booth and oven for application of paints on molded parts. | 03-2003 | FG-RULE 287(2)(c) FG-MACT PPPP |
| EU-GERREF OVEN | 1.2 MM Btu/hr natural gas fired process oven. | NA | FG-MACT DDDDD |
| EU-DUKES OVEN | 0.8 MM Btu/hr natural gas fired process oven. | NA | FG-MACT DDDDD |
| EU-COLDCLEANER | Any cold cleaner that is grandfathered or exempt from Rule 201 pursuant to Rule 278, Rule 278a and Rule 281(2)(h) or Rule 285(2)(r)(iv). Existing cold cleaners were placed into operation prior to July 1, 1979. New cold cleaners were placed into operation on or after July 1, 1979. | NA | FG-COLDCLEANERS |

## EU-SPRAYBOOTHGL1

EMISSION UNIT CONDITIONS

DESCRIPTION

Spray booth for application of gel coat on open molds.

Flexible Group ID:  FG-MOLDRELEASE and FG-COMPOSITESMACT

POLLUTION CONTROL EQUIPMENT

Fabric Overspray Filters

I.  EMISSION LIMIT(S)

| Pollutant | Limit | Time Period/ Operating Scenario | Equipment | Monitoring/ Testing Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- | --- |
| - VOC | 12.6 tons2* | Per 12 month rolling time period as determined at the end of each month | EU-SPRAYBOOTHGL1 | SC VI.1 SC VI.3 | R 336.1702(a) |

*	VOC emission limits are based upon the emission factors identified in “American National Standards Institute – Estimating Emission Factors from Open Molding and Other Composite Processes,” ACMA UEF-1-2011a, EF Table 1: Unified Emission Factors of Open Molding of Composites (Revised and Approved: 10/13/2009).

II.  MATERIAL LIMIT(S)

1.	The permittee shall not exceed the styrene monomer and methyl methacrylate (MMA) content limits listed in the following table for FG-SPRAYBOOTHGL1.2  (R 336.1225, R 336.1702(a))

| Material ID | Maximum Styrene Content (% wt) | Maximum (MMA) (% wt) |
| --- | --- | --- |
| - Tooling Gelcoats | 45.0 | 0.0 |
| b. Production-type Gelcoats | 39.0 | 10.0 |

III.  PROCESS/OPERATIONAL RESTRICTION(S)

1.	The permittee shall capture all waste gelcoats, mold releases and solvents used in EU-SPRAYBOOTHGL1 and store them in closed containers.  The permittee shall dispose of all waste materials in an acceptable manner in compliance with all applicable state rules and federal regulations.2  (R 336.1224, R 336.1702(a))

2.	The permittee shall dispose of spent filters in a manner which minimizes the introduction of air contaminants to the outer air.2  (R 336.1224, R 336.1370)

- The permittee shall handle all VOC and/or HAPs containing materials in a manner to minimize the generation of fugitive emissions.  The permittee shall keep containers covered at all times except when operator access is necessary.2  (R 336.1224, R 336.1225, R 336.1702(a))

IV.  DESIGN/EQUIPMENT PARAMETER(S)

1.	The permittee shall not operate EU-SPRAYBOOTHGL1 unless all exhaust filters are installed, maintained and operated in a satisfactory manner.2  (R 336.1301, R 336.1331)

2.	The permittee shall equip and maintain EU-SPRAYBOOTHGL1 with mechanical applicators or technology with equivalent or lower styrene emission rates.2  (R 336.1225, R 336.1702(a))

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The permittee shall complete all required calculations in a format acceptable to the AQD District Supervisor by the end of the calendar month, for the previous calendar month, unless otherwise specified in any monitoring/recordkeeping special condition.2  (R 336.1224, R 336.1225, R 336.1702)

2.	The permittee shall maintain a current listing from the manufacturer of the chemical composition of each material (i.e. resin, gel coat, catalyst, cleanup/purge solvent, etc.), including the weight percent of each component.  The data may consist of Material Safety Data Sheets, manufacturer's formulation data, or both as deemed acceptable by the AQD District Supervisor.  The permittee shall keep all records on file for a period of at least five years and make them available to the Department upon request.2  (R 336.1224, R 336.1225, R 336.1702(a))

3.	The permittee shall keep the following information for each calendar month for EU-SPRAYBOOTHGL1:

a.	The identity and amount (in pounds) of each gelcoat used.

b.	The styrene, MMA, and VOC content of each gelcoat used.

c.	The appropriate emission factors for each raw material used (The Unified Emission Factors (UEF-1-2011a) Table 1 for Open Molding of Composites from the American Composites Manufacturers Association (ACMA), October 2011 may be used, or an alternate factor approved by the AQD District Supervisor may be used).

d.	VOC mass emission calculations determining the monthly emission rate in tons per calendar month, and the annual emission rate in tons per 12-month rolling time period as determined at the end of each calendar month.

The permittee shall keep the records using mass balance or in a format acceptable to the AQD District Supervisor.  The permittee shall keep all records and make them available to the Department upon request.2

(R 336.1224, R 336.1225, R 336.1702(a))

See Appendix 7

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| - SVSTACKVENTGEL01 | 402 | 272 | R 336.1225, 40 CFR 52.21(c) & (d) |

IX.  OTHER REQUIREMENT(S)

1.	The permittee shall comply with all applicable provisions of the National Emission Standards for Hazardous Air Pollutants, as specified in 40 CFR Part 63, Subparts A and  WWWW for Reinforced Plastic Composites Production.2  (40 CFR Part 63, Subparts A and WWWW)

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## EU-SPRAYBOOTHRS1

EMISSION UNIT CONDITIONS

DESCRIPTION

Spray booth for application of resin on molds

Flexible Group ID:  FG-MOLDRELEASE and FG-COMPOSITESMACT

POLLUTION CONTROL EQUIPMENT

Fabric Overspray Filters

I.  EMISSION LIMIT(S)

| Pollutant | Limit | Time Period/ Operating Scenario | Equipment | Monitoring/ Testing Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- | --- |
| - VOC | 15.4 tons2* | 12-month rolling time period as determined at the end of each calendar month | EU-SPRAYBOOTHRS1 | SC VI.3 | R336.1225, R336.1702(a) |

*	VOC emission limits are based upon the emission factors identified in “American National Standards Institute – Estimating Emission Factors from Open Molding and Other Composite Processes,” ACMA UEF-1-2011a, EF Table 1: Unified Emission Factors of Open Molding of Composites (Revised and Approved: 10/13/2009).

II.  MATERIAL LIMIT(S)

1.	The styrene content of any resin used in EU-SPRAYBOOTHRS1 shall not exceed 39.0% percent by weight.2  (R 336.1225, R 336.1702(a))

III.  PROCESS/OPERATIONAL RESTRICTION(S)

1.	The permittee shall capture all waste resins, catalysts, mold releases and solvents used in EU-SPRAYBOOTHRS1 and store them in closed containers.  The permittee shall dispose of all waste materials in an acceptable manner in compliance with all applicable state rules and federal regulations.2

(R 336.1224, R 336.1702(a))

2.	The permittee shall dispose of spent filters in a manner which minimizes the introduction of air contaminants to the outer air.2  (R 336.1224, R 336.1370)

3.	The permittee shall handle all VOC and/or HAPs containing materials in a manner to minimize the generation of fugitive emissions.  The permittee shall keep containers covered at all times except when operator access is necessary.2  (R 336.1224, R 336.1225, R 336.1702(a))

IV.  DESIGN/EQUIPMENT PARAMETER(S)

1.	The permittee shall not operate EU-SPRAYBOOTHRS1 unless all exhaust filters are installed, maintained and operated in a satisfactory manner.2  (R 336.1301, R 336.1331)

2.	The permittee shall equip and maintain EU-SPRAYBOOTHRS1 with mechanical applicators or technology with equivalent or lower styrene emission rates.2  (R 336.1225, R 336.1702(a))

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The permittee shall complete all required calculations in a format acceptable to the AQD District Supervisor by the end of the calendar month, for the previous calendar month, unless otherwise specified in any monitoring/recordkeeping special condition.2  (R 336.1224, R 336.1225, R 336.1702)

2.	The permittee shall maintain a current listing from the manufacturer of the chemical composition of each material (i.e. resin, gel coat, catalyst, cleanup/purge solvent, etc.), including the weight percent of each component.  The data may consist of Material Safety Data Sheets, manufacturer's formulation data, or both as deemed acceptable by the AQD District Supervisor.  The permittee shall keep all records on file for a period of at least five years and make them available to the Department upon request.2  (R 336.1224, R 336.1225, R 336.1702(a))

3.	The permittee shall keep the following information for each calendar month for EU-SPRAYBOOTHRS1:

a.	The identity and amount (in pounds) of each resin and catalyst.

b.	The styrene and VOC content of each resin and catalyst used.

c.	The appropriate emission factors for each raw material used (The Unified Emission Factors (UEF-1-2011a) Table 1 for Open Molding of Composites from the American Composites Manufacturers Association (ACMA), October 2011 may be used, or an alternate factor approved by the AQD District Supervisor may be used).

d.	VOC mass emission calculations determining the monthly emission rate in tons per calendar month, and the annual emission rate in tons per 12-month rolling time period as determined at the end of each calendar month.

The permittee shall keep the records using mass balance in a format acceptable to the AQD District Supervisor.  The permittee shall keep all records and make them available to the AQD upon request.2  (R 336.1224, R 336.1225, R 336.1702(a))

See Appendix 7

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8

VIII.  STACK/VENT RESTRICTION(S)

The exhaust gases from the stacks listed in the table below shall be discharged unobstructed vertically upwards to the ambient air unless otherwise noted:

| Stack & Vent ID | Maximum Exhaust Dimensions (inches) | Minimum Height Above Ground (feet) | Underlying Applicable Requirements |
| --- | --- | --- | --- |
| - SVSTACKVENTGEL01 | 402 | 272 | R 336.1225, 40 CFR 52.21(c) & (d) |

IX.  OTHER REQUIREMENT(S)

1.	The permittee shall comply with all applicable provisions of the National Emission Standards for Hazardous Air Pollutants, as specified in 40 CFR Part 63, Subparts A and WWWW for Reinforced Plastic Composites Production.2  (40 CFR Part 63, Subparts A and WWWW)

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

# D.  FLEXIBLE GROUP SPECIAL CONDITIONS

Part D outlines the terms and conditions that apply to more than one emission unit.  The permittee is subject to the special conditions for each flexible group in addition to the General Conditions in Part A and any other terms and conditions contained in this ROP.

The permittee shall comply with all specific details in the special conditions and the underlying applicable requirements cited.  If a specific condition type does not apply, NA (not applicable) has been used in the table.  If there are no special conditions that apply to more than one emission unit, this section will be left blank.

## FLEXIBLE GROUP SUMMARY TABLE

The descriptions provided below are for informational purposes and do not constitute enforceable conditions.

| Flexible Group ID | Flexible Group Description | Associated Emission Unit IDs |
| --- | --- | --- |
| FG-COMPOSITESMACT | Open Molding Operations emitting less than 100 TPY of HAP and subject to the Reinforced Plastic Composites Production NESHAP - 40 CFR Part 63, Subpart WWWW. | EU-SPRAYBOOTHGL1, EU-SPRAYBOOTHRS1, other open molding processes, repair operations, material mixers, equipment clean up, and HAP-containing material storage, all used in the manufacture of reinforced plastic composites. |
| FG-MACT PPPP | Each new, reconstructed, and existing affected source engaged in the surface coating of plastic parts and products, identified within each of the four subcategories listed in 40 CFR Part 63, Subpart PPPP, 40 CFR 63.4481(a)(2) to (5). Surface coating is defined by 40 CFR 63.4481 as the application of coating to a substrate using, for example, spray guns or dip tanks. Surface coating also includes associated activities, such as surface preparation, cleaning, mixing, and storage if they are directly related to the application of the coating. | EU-PAINTBOOTH |
| FG-MOLDRELEASE | Mold release and Acetone usage. | EU-SPRAYBOOTHGL1, EU-SPRAYBOOTHRS1 |
| FG-MACT DDDDD | Requirements for existing Gas 1, (Natural Gas only) for existing Boilers and Process Heaters at major sources of Hazardous Air Pollutants per 40 CFR Part 63, Subpart DDDDD. These existing boilers or process heaters must comply with this subpart no later than January 31, 2016, except as provided in 40 CFR 63.6(i). | EU-GERREF OVEN, EU-DUKES OVEN |
| FG-RULE 287(2)(c) | Any emission unit that emits air contaminants and is exempt from the requirements of Rule 201 pursuant to Rule 278, Rule 278a and Rule 287(2)(c). Emission units installed/modified before December 20, 2016, may show compliance with Rule 287 in effect at the time of installation/modification. | EU-PAINTBOOTH |
| FG-COLDCLEANERS | Any cold cleaner that is grandfathered or exempt from Rule 201 pursuant to Rule 278, Rule 278a and Rule 281(2)(h) or Rule 285(2)(r)(iv). Existing cold cleaners were placed into operation prior to July 1, 1979. New cold cleaners were placed into operation on or after July 1, 1979. | EU-COLDCLEANER |

## FG-COMPOSITESMACT

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Open molding, closed molding, mixing, and cleaning operations emitting less than 100 TPY of HAP and subject to the Reinforced Plastic Composites Production NESHAP - 40 CFR Part 63, Subpart WWWW.

Emission Units:  EU-SPRAYBOOTHGL1, EU-SPRAYBOOTHRS1, other molding operations, material mixers, equipment clean up, storage of HAP-containing materials, and repair operations, all used in the manufacture of reinforced plastic composites.

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

- The permittee shall not operate FG-COMPOSITESMACT except in compliance with the organic HAP emissions limits in 40 CFR Part 63, Subpart WWWW, Table 3.  (40 CFR 63.5805)

- The permittee must use one or a combination of the following methods to meet the standards for open molding operations in 40 CFR Part 63, Subpart WWWW, Table 3..  (40 CFR 63.5810)

  - Demonstrate that an individual resin or gel coat, as applied, meets the applicable emission limit in 40 CFR Part 63, Subpart WWWW, Table 3..  (40 CFR 63.5810(a))

  - Demonstrate that, on average, the facility meets the individual organic HAP emissions limits for each unique combination of operation type and resin application method or gel coat type shown in Table 3 to this subpart that applies to the facility.  (40 CFR 63.5810(b))

  - Demonstrate compliance with a weighted average emission limit.  Demonstrate each month that the facility meets each weighted average of the organic HAP emissions limits in Table 3 to this subpart that apply the weighted average organic HAP emissions limit for all open molding operations.  (40 CFR 63.5810(c))

  - Meet the organic HAP emissions limit for one application method and use the same resin(s) for all application methods of that resin type.  This option is limited to resins of the same type.  The resin types for which this option may be used are non-corrosion-resistant, corrosion-resistant and/or high strength, and tooling.  (40 CFR 63.5810(d))

- The permittee may switch between the compliance options in 2(a) through 2(d).  When changing to an option based on a 12-month rolling average, the facility must base the average on the previous 12 months of data calculated using the compliance option the facility is changing to, unless the facility previously used an option that did not require the facility to maintain records of resin or gel coat.  In this case, the facility must immediately begin collecting resin and gel coat and demonstrate compliance 12 months after changing options.  (40 CFR 63.5810)

See Appendix 9

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

- For each new or existing closed molding operation using compression/injection molding, the permittee shall uncover, unwrap or expose only one charge per mold cycle per compression/injection molding machine.  For machines with multiple molds, one charge means sufficient material to fill all molds for one cycle.  For machines with robotic loaders, no more than one charge may be exposed prior to the loader.  For machines fed by hoppers, sufficient material may be uncovered to fill the hopper.  Hoppers must be closed when not adding materials. Materials may be uncovered to feed to slitting machines.  Materials must be recovered after slitting.  (40 CFR 63.5805, Table 4)

- For each cleaning operation, the permittee shall not use cleaning solvents that contain HAP, except that styrene may be used as a cleaner in closed systems, and organic HAP containing cleaners may be used to clean cured resin from application equipment.  Application equipment includes any equipment that directly contacts resin.  (40 CFR 63.5805, Table 4)

- For each HAP-containing materials storage operation, the permittee shall keep containers that store HAP-containing materials closed or covered except during the addition or removal of materials.  Bulk HAP-containing materials storage tanks may be vented as necessary for safety.  (40 CFR 63.5805, Table 4)

- For each mixing operation, the permittee shall use mixer covers with no visible gaps present in the mixer covers, except that gaps of up to 1 inch are permissible around mixer shafts and any required instrumentation.  (40 CFR 63.5805, Table 4)

- For each mixing operation, the permittee shall close any mixer vents when actual mixing is occurring, except that venting is allowed during addition of materials, or as necessary prior to adding materials or opening the cover for safety.  Vents routed to a 95% efficient control device are exempt from this requirement.  (40 CFR 63.5805, Table 4)

- For each mixing operation, the permittee shall keep the mixer covers closed while actual mixing is occurring, except when adding materials or changing covers to the mixing vessels.  (40 CFR 63.5805, Table 4)

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- The HAP content of any resin, gel coat, etc., as received and as applied, shall be determined using Material Safety Data Sheets, manufacturer's formulation data, or both as deemed acceptable by the AQD District Supervisor.  Upon request of the AQD District Supervisor, the manufacturer’s HAP formulation data shall be verified using EPA Test Method 311.  Determinations of organic HAP content shall also be in compliance with the requirements specified in 40 CFR Part 63, Subpart WWWW.  (40 CFR 63.5797)

See Appendix 5

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- Facilities meeting an organic HAP emissions standard based on a 12-month rolling average must begin collecting data on the compliance date in order to demonstrate compliance.  (40 CFR 63.5800)

- The permittee shall determine organic HAP emission factors in accordance with 40 CFR Part 63, Subpart WWWW, 40 CFR 63.5910 and Table 1 for the purposes of compliance demonstration.  (40 CFR 63.5796)

- The permittee shall demonstrate compliance with the applicable standards in 40 CFR Part 63, Subpart WWWW, by monitoring and keeping records in accordance with 40 CFR Part 63, Subpart WWWW.  (40 CFR 63.5895)

- The permittee shall maintain records in accordance with 40 CFR Part 63, Subpart WWWW. (40 CFR 63.5915)

See Appendix 9

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

- The permittee shall submit the applicable notifications and reports by the dates specified in 40 CFR Part 63, Subpart WWWW, Table 13 and Table 14. to the AQD in accordance with , 40 CFR 63.5905 and 40 CFR 63.5910, respectively.  (40 CFR Part 63, Subparts A and WWWW)

See Appendix 8

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

- The permittee shall comply with all applicable provisions of the National Emission Standards for Hazardous Air Pollutants as specified in 40 CFR Part 63, Subparts A and WWWW, as they apply to FG-COMPOSITESMACT.  (40 CFR Part 63, Subparts A and WWWW)

- See EU-SPRAYBOOTHGL1, EU-SPRAYBOOTHRS1, for additional requirements.

Footnotes:

1This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-MACT PPPP

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Each new, reconstructed, and existing affected source engaged in the surface coating of plastic parts and products, identified within each of the four subcategories listed in 40 CFR Part 63, Subpart PPPP, 40 CFR 63.4481(a)(2) to (5).  Surface coating is defined by 40 CFR 63.4481 as the application of coating to a substrate using, for example, spray guns or dip tanks.  Surface coating also includes associated activities, such as surface preparation, cleaning, mixing, and storage if they are directly related to the application of the coating.

Emission Unit:  EU-PAINTBOOTH

POLLUTION CONTROL EQUIPMENT

Dry panel filters

I.  EMISSION LIMIT(S)

| Pollutant | Limit | Time Period/ Operating Scenario | Equipment | Monitoring/ Testing Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- | --- |
| - Organic HAP | 0.16 lb. per lb. of coating solids | 12-month rolling time period as determined at the end of each calendar month. | Existing - General Use Coating | SC V.1, SC VI.1 through SC VI.5 | 40 CFR 63.4490(b)(1) |
| - Organic HAP | 0.45 lb. per lb. of coating solids | 12-month rolling time period as determined at the end of each calendar month. | Existing - Automotive Lamp Coating | SC V.1, SC VI.1 through SC VI.5 | 40 CFR 63.4490(b)(2) |
| - Organic HAP | 0.26 lb. per lb. of coating solids | 12-month rolling time period as determined at the end of each calendar month. | Existing - Thermoplastic Olefin (TPO) Coating | SC V.1, SC VI.1 through SC VI.5 | 40 CFR 63.4490(b)(3) |
| - Organic HAP | 1.34 lb. per lb. of coating solids | 12-month rolling time period as determined at the end of each calendar month. | Existing - Assembled On-road Vehicle Coating | SC V.1, SC VI.1 through SC VI.5 | 40 CFR 63.4490(b)(4) |

5.	The permittee shall determine whether the organic HAP emission rate is equal to or less than the applicable emission limits in 40 CFR 63.4490 using at least one of the following three options, which are listed in 40 CFR 63.4491(a) through (c):

a.	Compliant material option,

b.	Emission rate without add-on controls option, or

c.	Emission rate with add-on controls option.

The permittee shall include all coatings, thinners and/or other additives, and cleaning materials used when determining the emission rate.  (40 CFR 63.4491)

6.	Any coating operation(s) using the compliant material option or the emission rate without add-on controls option shall be in compliance with the applicable emission limits in 40 CFR 63.4490 at all times.  (40 CFR 63.4500(a)(1))

7.	If the surface coating operation(s) meet the applicability criteria of more than one of the subcategory emission limits specified in 40 CFR 63.4490(a) or (b), the permittee may comply separately with each subcategory emission limit or comply using one of the alternatives in 40 CFR 63.4490(c)(1) or (2).  (40 CFR 63.4490(c))

II.  MATERIAL LIMIT(S)

For the compliant materials option, the permittee shall meet the material limits specified in the following table.

| Material | Limit | Time Period/ Operating Scenario | Equipment | Monitoring/ Testing Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- | --- |
| - Each Thinner and/or Additive | No Organic HAP * | Continuous | Each Coating Operation using Compliant Material Option | SC VI.1, SC VI.2, SC VI.3, SC VI.5 | 40 CFR 63.4491(a) |
| - Each Cleaning Material | No Organic HAP * | Continuous | Each Coating Operation using Compliant Material Option | SC VI.1, SC VI.2, SC VI.3, SC VI.5 | 40 CFR 63.4491(a) |

* Determined according to 40 CFR 63.4541(a).

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii), 40 CFR 63.4531)

1.	The permittee shall determine the mass fraction of organic HAP for each material used, the mass fraction of coating solids for each coating, and the density of each material used in accordance with 40 CFR 63.4541, 40 CFR 63.4551, and/or 40 CFR 63.4561.  (40 CFR 63.4541, 40 CFR 63.4551, 40 CFR 63.4561)

See Appendix 5

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii), 40 CFR 63.4531)

1.	The permittee shall conduct an initial compliance demonstration for the initial compliance period according to the requirements in 40 CFR 63.4541, 40 CFR 63.4551, or 40 CFR 63.4561.  The initial compliance period begins on the applicable compliance date specified in 40 CFR 63.4483 and ends on the last day of the 12th month following the compliance date.  If the compliance date occurs on any day other than the first of the month, then the compliance period extends through that month plus the next 12 months.  (40 CFR 63.4483, 40 CFR 63.4540, 40 CFR 63.4550, 40 CFR 63.4560)

2.	The permittee shall keep all records required by 40 CFR 63.4530 in the format and timeframes outlined in 40 CFR 63.4531.  (40 CFR 63.4542(d), 40 CFR 63.4552(d), 40 CFR 63.4563(j))

3.	The permittee shall maintain, at a minimum, the following records for each compliance period:

a.	A copy of each notification and report that is submitted to comply with 40 CFR Part 63, Subpart PPPP, and the documentation supporting each notification report.  (40 CFR 63.4530(a))

b.	A current copy of information provided by materials suppliers or manufacturers, such as manufacturer’s formulation data, or test data used to determine the mass fraction of organic HAP and density of each coating, thinner and/or other additive, and cleaning material, and the mass fraction of coating solids for each coating.  (40 CFR 63.4530(b))

c.	A list of the coating operations on which each compliance option was used, and the beginning and ending dates and times for each compliance option used.  (40 CFR 63.4530(c)(1))

d.	For the compliant materials option, the calculation of the organic HAP content for each coating, using Equation 1 of 40 CFR 63.4541.  (40 CFR 63.4530(c)(2))

e.	For the emission rate without add-on controls option, the calculation of the total mass of organic HAP emissions for the coatings, thinners and/or additives, and cleaning materials used each month using Equations 1, 1A through 1C and 2 of 40 CFR 63.4551; and, if applicable, the calculation used to determine mass of organic HAP in waste materials according to 40 CFR 63.4551(e)(4); the calculation of the total mass of coating solids used each month using Equation 2 of 40 CFR 63.4551; and the calculation of each 12-month organic HAP emission rate using Equation 3 of 40 CFR 63.4551.  (40 CFR 63.4530(c)(3))

f.	The name and mass or volume of each coating, thinner and/or other additive, and cleaning material used during each compliance period.  If the compliant material option is used for all coatings at the affected source, the permittee may maintain purchase records for each material used rather than a record of the mass used.   (40 CFR 63.4530(d))

g.	The mass fraction of organic HAP for each coating, thinner and/or additive, and cleaning material used during each compliance period.  (40 CFR 63.4530(e))

h.	The mass fraction of coating solids for each coating used during each compliance period.  (40 CFR 63.4530(f))

i.	The information specified in 40 CFR 63.4530(g)(1) through (3), if an allowance is used in Equation 1 of 40 CFR 63.4551 for organic HAP contained in waste materials sent to or designated for shipment to a treatment, storage, and disposal facility (TSDF) according to 40 CFR 63.4551(e)(4).  (40 CFR 63.4530(g))

j.	The date, time, and duration of each deviation.  (40 CFR 63.4530(h))

4.	For each coating used for the compliant coating option, the permittee shall demonstrate continuous compliance with the emission limit in 40 CFR 63.4490, for each compliance period, using Equation 1 of 40 CFR 63.4541.  For each thinner and cleaning material used, the permittee shall determine continuous compliance according to 40 CFR 63.4541(a).  (40 CFR 63.4542)

5.	For any coating operation or group of coating operations using the emission rate without add-on controls option, the permittee shall demonstrate continuous compliance with the applicable organic HAP emission limit in 40 CFR 63.4490, for each compliance period according to 40 CFR 63.4551(a) through (g). (40 CFR 63.4552)

See Appendix 7

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

4.	For the compliant material option, the use of any coating, thinner or cleaning material which does not meet the criteria specified in 40 CFR 63.4542(a) is a deviation that must be reported as specified in 40 CFR 63.4510(c)(6) and 40 CFR 63.4520(a)(5).  (40 CFR 63.4542(b))

5.	For the emission rate without add-on controls, if the organic HAP emission rate for any compliance period exceeds the applicable emission limit specified in 40 CFR 63.4490, the permittee shall report this as a deviation as specified in 40 CFR 63.4510(c)(6) and 40 CFR 63.4520(a)(6).  (40 CFR 63.4552(b))

6.	The permittee shall submit the applicable notifications specified in 40 CFR 63.7(b) and (c), 40 CFR 63.8(f)(4) and 40 CFR 63.9(b) through (e) and (h), an initial notification and a notification of compliance status as specified in 40 CFR 63.4510.  (40 CFR Part 63, Subparts A and PPPP)

7.	The permittee shall submit all semiannual compliance reports as required by 40 CFR 63.4520.  Each semiannual compliance report shall identify which coating operation(s) used each compliance option, and if there were no deviations from the emission limitations in 40 CFR 63.4490, include a statement that the coating operations were in compliance.  (40 CFR 63.4520, 40 CFR 63.4542(c), 40 CFR 63.4552(c), 40 CFR 63.4563(f))

See Appendix 8

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

1.	The permittee shall comply with all applicable provisions of the National Emission Standards for Hazardous Air Pollutants, as specified in 40 CFR Part 63, Subparts A and PPPP for Surface Coating of Plastic Parts and Products by the initial compliance date.  (40 CFR Part 63, Subparts A and PPPP)

Footnotes:

1This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-MOLDRELEASE

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Mold release and acetone usage

Emission Units:  EU-SPRAYBOOTHGL1, EU-SPRAYBOOTHRS1

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

| Pollutant | Limit | Time Period/ Operating Scenario | Equipment | Monitoring/ Testing Method | Underlying Applicable Requirements |
| --- | --- | --- | --- | --- | --- |
| - Acetone | 16.2 tons1 | 12 month rolling time period as determined at the end of each month | EU-SPRAYBOOTHGL1, EU-SPRAYBOOTHRS1 | SC VI.3 | R336.1224 |
| - VOC (from mold release usage, excluding acetone) | 4.1 tons2 | 12 month rolling time period as determined at the end of each month | EU-SPRAYBOOTHGL1, EU-SPRAYBOOTHRS1 | SC VI.3 | R336.1225, R336.1702(a), |

II.  MATERIAL LIMIT(S)

NA

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The permittee shall complete all required calculations in a format acceptable to the AQD District Supervisor by the end of the calendar month, for the previous calendar month, unless otherwise specified in any monitoring/recordkeeping special condition.2  (R 336.1224, R 336.1225, R 336.1702)

2.	The permittee shall maintain a current listing from the manufacturer of the chemical composition of each material (i.e. mold release, cleanup/purge solvent, etc.), including the weight percent of each component.  The data may consist of Material Safety Data Sheets, manufacturer's formulation data, or both as deemed acceptable by the AQD District Supervisor.  The permittee shall keep all records on file for a period of at least five years and make them available to the Department upon request.2  (R 336.1224, R 336.1225, R 336.1702(a))

3.	The permittee shall keep the following information for each calendar month for FG-MOLDRELEASE:

a.	The identity and amount (in pounds) of each mold release, and cleanup solvent used.

b.	The VOC content of each mold release and cleanup solvent used.

c.	The acetone content of each mold release and cleanup solvent used.

d.	VOC mass emission calculations determining the monthly emission rate in tons per calendar month, and the annual emission rate in tons per 12-month rolling time period as determined at the end of each calendar month.

e.	Acetone mass emission calculations determining the monthly emission rate in tons per calendar month, and the annual emission rate in tons per 12-month rolling time period as determined at the end of each calendar month.

The permittee shall keep the records in a format acceptable to the AQD District Supervisor.  The permittee shall keep all records and make them available to the Department upon request.2  (R 336.1224, R 336.1225, R 336.1702(a))

See Appendix 7

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

NA

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

## FG-MACT DDDDD

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Requirements for existing Small (<10 MMBtu), Gas 1, (Natural Gas only) Boilers and Process Heaters at major sources of Hazardous Air Pollutants per 40 CFR Part 63, Subpart DDDDD.  These existing boilers or process heaters must comply with this subpart no later than January 31, 2016, except as provided in 40 CFR 63.6(i).

Emission Units:  EU-GERREF OVEN, EU-DUKES OVEN

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

1.	The permittee shall only burn natural gas as defined in 40 CFR 63.7575.  (40 CFR 63.7499(l))

III.  PROCESS/OPERATIONAL RESTRICTION(S)

- The permittee must meet the tune-up and Energy Assessment work practice standards for each applicable boiler or process heater at the source.  (40 CFR 63.7500(a)(1), 40 CFR Part 63, Subpart DDDDD, Table 3, Nos. 1-4)

- The permittee must operate and maintain affected sources in a manner consistent with safety and good air pollution control practices for minimizing emissions.  Determination of whether such operation and maintenance procedures are being used will be based on information available to the Administrator that may include, but is not limited to, monitoring results, review of operation and maintenance procedures, review of operation and maintenance records, and inspection of the source.  (40 CFR 63.7500(a)(3))

- The permittee may obtain approval from the Administrator to use an alternative to the work practice standards noted in SC III.1 and/or SC III.2.  (40 CFR 63.7500(b))

- The permittee must:

  - Complete a tune-up every 5 years (61 months) for boilers/process heaters less than or equal to 5 million Btu per hour.  (40 CFR 63.7500(e), 40 CFR 63.7515(d))

  - Complete a tune-up every 2 years (25 months) for boilers greater than 5 million Btu per hour and less than 10 million Btu per hour.  (40 CFR 63.7500(e), 40 CFR 63.7515(d))

  - Complete a tune-up annually (13 months) for boilers greater than 10 million Btu per hour.  (40 CFR 63.7540(a)(10), 40 CFR 63.7515(d))

  - Conduct the tune-up within 30 calendar days of startup, if the unit is not operating on the required date for a tune-up.  (40 CFR 63.7540(a)(13))

  - Follow the procedures described in SC IX 4.a through 4.f for all initial and subsequent tune ups.

(40 CFR 63.7540(a)(10), 40 CFR Part 63, Subpart DDDDD, Table 3)

  - Complete the Initial tune ups on all affected units no later than January 31, 2016, except as provided in 40 CFR 63.7510(j) and 40 CFR 63.7540(a)(13).

- The permittee must complete the one-time energy assessment no later than January 31, 2016.

(40 CFR 63.7510(e))

IV.  DESIGN/EQUIPMENT PARAMETER(S)

NA

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

- The permittee must keep a copy of each notification and report submitted to comply with 40 CFR Part 63, Subpart DDDDD, including all documentation supporting any Initial Notification or Notification of Compliance Status or semiannual compliance report that the permittee submitted, according to the requirements in 40 CFR 63.10(b)(2)(xiv).  (40 CFR 63.7555(a)(1))

- The permittee must keep each record on site, or they must be accessible from on-site (for example, through a computer network), for at least 2 years after the date of each occurrence, measurement, maintenance, corrective action, report, or record.  The permittee can keep the records off site for the remaining 3 years.  (40 CFR 63.7560(a), (b), and (c))

See Appendix 7

VII.  REPORTING

- Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

- Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

- Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

- The permittee must submit a Notification of Compliance Status that includes each boiler or process heater before the close of business on the 60th day following the completion of the initial compliance demonstrations for all boiler or process heaters at the facility.  The Notification of Compliance Status report must contain the following information and must be submitted within 60 days of January 31, 2016.  (40 CFR 63.7545(e))

  - A description of the affected unit(s) including identification of which subcategories the unit is in, the design heat input capacity of the unit, a description of the add-on controls used on the unit to comply with 40 CFR Part 63, Subpart DDDDD, description of the fuel(s) burned.  (40 CFR 63.7545(e)(1))

  - Certification(s) of compliance, as applicable, and signed by a responsible official:  (40 CFR 63.7545(e)(8))

i.	“This facility completed the required initial tune-up for all of the boilers and process heaters covered by 40 CFR Part 63, Subpart DDDDD at this site according to the procedures in 40 CFR 63.7540(a)(10)(i) through (vi).”  (40 CFR 63.7545(e)(8)(i))

ii.	“This facility has had an energy assessment performed according to 40 CFR 63.7530(e).”  (40 CFR 63.7545(e)(8)(ii))

- Unless the EPA Administrator has approved a different schedule for submission of reports under 40 CFR 63.10(a), the permittee must submit each report, according to paragraph (h) of 40 CFR 63.7550, stated in SC VII.7, by the date in Table 9 of 40 CFR Part 63, Subpart DDDDD and according to the requirements in paragraphs (b)(1) through (4) of 40 CFR 63.7550, as listed below.  For units that are subject only to a requirement to conduct an annual tune-up according to 40 CFR 63.7540(a)(10), stated in SC IX.4.a, biennial tune-up according to 40 CFR 63.7540(a)(11), stated in SC IX.4.b, or 5-year tune-up according to 40 CFR 63.7540(a)(12), stated in SC IX.4.c, and not subject to emission limits or operating limits, the permittee may submit only an annual, biennial, or 5-year compliance report, as applicable, as specified in paragraphs (b)(1) through (4) of 40 CFR 63.7550, as listed below, instead of a semiannual compliance report.

(40 CFR 63.7550(b))

- The first semiannual compliance report must cover the period beginning on January 31, 2016 and ending on December 31.  When submitting an annual, biennial, or 5-year compliance report, the first compliance report must cover the period beginning on January 31, 2016 and ending on December 31 within 1, 2, or 5 years, as applicable, after the compliance date that is specified in 40 CFR 63.7495.   (40 CFR 63.7550(b)(1))

- The first semiannual compliance report must be postmarked or submitted no later than September 15 or March 15, whichever date is the first date following the end of the first calendar half after January 31, 2016.  The first annual, biennial, or 5-year compliance report must be postmarked or submitted no later than March 15.  (40 CFR 63.7550(b)(2), 40 CFR 63.7550(b)(5))

- Each subsequent semiannual compliance report must cover the semiannual reporting period from January 1 through June 30 or the semiannual reporting period from July 1 through December 31.  Annual, biennial, and 5-year compliance reports must cover the applicable 1-, 2-, or 5-year periods from January 1 to December 31.  (40 CFR 63.7550(b)(3))

- Each subsequent semiannual compliance report must be postmarked or submitted no later than September 15 or March 15, whichever date is the first date following the end of the semiannual reporting period.  Annual, biennial, and 5-year compliance reports must be postmarked or submitted no later than March 15.  (40 CFR 63.7550(b)(4), 40 CFR 63.7550(b)(5))

- The permittee must include the following information in the compliance report.  (40 CFR 63.7550(c), 40 CFR 63.7550(c)(1))

- Company and Facility name and address.  (40 CFR 63.7550(c)(5)(i))

- Process unit information, emissions limitations, and operating parameter limitations.  (40 CFR 63.7550(c)(5)(ii))

- Date of report and beginning and ending dates of the reporting period.  (40 CFR 63.7550(c)(5)(iii))

- Include the date of the most recent tune-up for each unit. Include the date of the most recent burner inspection if it was not done annually, biennially, or on a 5-year period and was delayed until the next scheduled or unscheduled unit shutdown.  (40 CFR 63.7550(c)(5)(xiv))

- Statement by a responsible official with that official's name, title, and signature, certifying the truth, accuracy, and completeness of the content of the report.  (40 CFR 63.7550(c)(5)(xvii)

- The permittee must submit the reports according to the procedures specified in paragraph (h)(3) of 40 CFR 63.7550, as listed below.  (40 CFR 63.7550(h))

  - The permittee must submit all reports required by Table 9 of 40 CFR Part 63, Subpart DDDDD electronically to the EPA via the Compliance and Emissions Data Reporting Interface (CEDRI).  The CEDRI can be accessed through the EPA's CDX.  The permittee must use the appropriate electronic report in the CEDRI for 40 CFR Part 63, Subpart DDDDD.  Instead of using the electronic report in the CEDRI for 40 CFR Part 63, Subpart DDDDD, the permittee may submit an alternate electronic file consistent with the XML schema listed on the CEDRI Web site (http://www.epa.gov/ttn/chief/cedri/index.html), once the XML schema is available.  If the reporting form specific to 40 CFR Part 63, Subpart DDDDD is not available in the CEDRI at the time that the report is due, the permittee must submit the report to the Administrator at the appropriate address listed in 40 CFR 63.13.  The permittee must begin submitting reports via CEDRI no later than 90-days after the form becomes available in the CEDRI.  (40 CFR 63.7550(h)(3))

See Appendix 8

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

- The permittee must comply with 40 CFR Part 63, Subpart DDDDD no later than January 31, 2016, for existing boilers and process heaters, unless an extension has been granted per 40 CFR 63.6(i).  (40 CFR 63.7495(b))

- The permittee must be in compliance with the applicable work practice standards.  (40 CFR 63.7505(a))

- For affected sources (as defined in 40 CFR 63.7490) that have not operated since the previous compliance demonstration and more than one year has passed since the previous compliance demonstration, the permittee must complete a subsequent tune-up within 30 days of startup by following the procedures described in SC IX 4.a through 4.f.  (40 CFR 63.7515(g))

- The permittee must demonstrate continuous compliance with the tune-up requirement by completing the following:  (40 CFR 63.7540(a))

- Inspect the burner, and clean or replace any components of the burner as necessary (the permittee may perform the burner inspection any time prior to tune-up or delay the burner inspection until the next scheduled unit shutdown).  At units where entry into a piece of process equipment or into a storage vessel is required to complete the tune-up inspections, inspections are required only during planned entries into the storage vessel or process equipment.  (40 CFR 63.7540(a)(10)(i))

- Inspect the flame pattern, as applicable, and adjust the burner as necessary to optimize the flame pattern. The adjustment should be consistent with the manufacturer's specifications, if available.  (40 CFR 63.7540(a)(10)(ii))

- Inspect the system controlling the air-to-fuel ratio, as applicable, and ensure that it is correctly calibrated and functioning properly (the permittee may delay the inspection until the next scheduled unit shutdown). Units that produce electricity for sale may delay the inspection until the first outage, not to exceed 36 months from the previous inspection.  (40 CFR 63.7540(a)(10)(iii))

- Optimize total emissions of CO.  This optimization should be consistent with the manufacturer's specifications, if available, and with any NOX requirement to which the unit is subject.  (40 CFR 63.7540(a)(10)(iv))

- Measure the concentrations in the effluent stream of CO in parts per million, by volume, and oxygen in volume percent, before and after the adjustments are made (measurements may be either on a dry or wet basis, as long as it is the same basis before and after the adjustments are made).  Measurements may be taken using a portable CO analyzer.  (40 CFR 63.7540(a)(10)(v))

- Maintain on-site and submit, if requested by the Administrator, the most recent periodic report containing the information as listed below.  (40 CFR 63.7540(a)(10)(vi))

- The concentrations of CO in the effluent stream in parts per million by volume, and oxygen in volume percent, measured at high fire or typical operating load, before and after the tune-up of the boiler or process heater.  (40 CFR 63.7540(a)(10)(vi)(A))

- A description of any corrective actions taken as a part of the tune-up.  (40 CFR 63.7540(a)(10)(vi)(B))

- The type and amount of fuel used over the 12 months prior to the tune-up, but only if the unit was physically and legally capable of using more than one type of fuel during that period.  Units sharing a fuel meter may estimate the fuel used by each unit.  (40 CFR 63.7540(a)(10)(vi)(C))

5.	If the boiler or process heater has a heat input capacity of less than or equal to 5 million Btu per hour, the permittee may delay the burner inspection specified in SC IX 4.a until the next scheduled or unscheduled unit shutdown, but the permittee must inspect each burner at least once every 72 months.  If an oxygen trim system is utilized on a unit without emission standards to reduce the tune-up frequency to once every 5 years, set the oxygen level no lower than the oxygen concentration measured during the most recent tune-up. (40 CFR 63.7540(a)(12))

Footnotes:

1 This condition is state only enforceable and was established pursuant to Rule 201(1)(b).

2 This condition is federally enforceable and was established pursuant to Rule 201(1)(a).

FG-RULE 287(2)(c)

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Any emission unit that emits air contaminants and is exempt from the requirements of Rule 201 pursuant to Rule 278, Rule 278a and Rule 287(2)(c).  Emission units installed/modified before December 20, 2016, may show compliance with Rule 287 in effect at the time of installation/modification.

Emission Units installed on or after December 20, 2016:  NA

Emission Units installed prior to December 20, 2016:  EU-PAINTBOOTH

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

| Material | Limit | Time Period/Operating Scenario | Equipment | Underlying Applicable Requirement |
| --- | --- | --- | --- | --- |
| - Coatings | 200 Gallons/month (minus water as applied) | Calendar month | Each emission unit | R 336.1287(2)(c)(i) |

III.  PROCESS/OPERATIONAL RESTRICTION(S)

NA

IV.  DESIGN/EQUIPMENT PARAMETER(S)

1.	Any exhaust system installed on or after December 20, 2016, that serves only coating spray equipment shall be equipped with a dry filter control or water wash control which is installed, maintained, and operated in accordance with the manufacturer’s specifications, or the permittee develops a plan which provides to the extent practicable for the maintenance and operation of the equipment in a manner consistent with good air pollution control practices for minimizing emissions.  All emission units installed before December 20, 2016, with an exhaust system that serves only coating spray equipment must have a properly installed and operated particulate control system. (R 336.1213(2), R 336.1287(2)(c)(ii), R 336.1910)

V.  TESTING/SAMPLING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	The permittee shall maintain records of the following information for each emission unit for each calendar month using the methods outlined in the DEQ, AQD Rule 287(2)(c), Permit to Install Exemption Record form (EQP 3562) or in a format acceptable to the AQD District Supervisor.  (R 336.1213(3))

a.	Volume of coating used, as applied, minus water, in gallons.  (R 336.1287(2)(c)(iii))

b.	Documentation of any filter replacements or maintenance of water wash control for exhaust systems serving coating spray equipment or other documentation included in a plan developed by the owner or operator of the equipment.  (R 336.1213(3))

See Appendix 4

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8

VIII.  STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

NA

## FG-COLDCLEANERS

FLEXIBLE GROUP CONDITIONS

DESCRIPTION

Any cold cleaner that is grandfathered or exempt from Rule 201 pursuant to Rule 278, 278a and Rule 281(2)(h) or Rule 285(2)(r)(iv).  Existing cold cleaners were placed into operation prior to July 1, 1979.  New cold cleaners were placed into operation on or after July 1, 1979.

Emission Unit:  EU-COLDCLEANER

POLLUTION CONTROL EQUIPMENT

NA

I.  EMISSION LIMIT(S)

NA

II.  MATERIAL LIMIT(S)

1.	The permittee shall not use cleaning solvents containing more than five percent by weight of the following halogenated compounds: methylene chloride, perchloroethylene, trichloroethylene, 1,1,1-trichloroethane, carbon tetrachloride, chloroform, or any combination thereof.  (R 336.1213(2))

III.  PROCESS/OPERATIONAL RESTRICTION(S)

1.	Cleaned parts shall be drained for no less than 15 seconds or until dripping ceases.  (R 336.1611(2)(b), R 336.1707(3)(b))

2.	The permittee shall perform routine maintenance on each cold cleaner as recommended by the manufacturer.  (R 336.1213(3))

IV.  DESIGN/EQUIPMENT PARAMETER(S)

1.	The cold cleaner must meet one of the following design requirements:

a.	The air/vapor interface of the cold cleaner is no more than ten square feet.  (R 336.1281(2)(h))

b.	The cold cleaner is used for cleaning metal parts and the emissions are released to the general in-plant environment.  (R 336.1285((2)r)(iv))

2.	The cold cleaner shall be equipped with a device for draining cleaned parts.  (R 336.1611(2)(b), R 336.1707(3)(b))

3.	All new and existing cold cleaners shall be equipped with a cover and the cover shall be closed whenever parts are not being handled in the cold cleaner.  (R 336.1611(2)(a), R 336.1707(3)(a))

4.	The cover of a new cold cleaner shall be mechanically assisted if the Reid vapor pressure of the solvent is more than 0.3 psia or if the solvent is agitated or heated.  (R 336.1707(3)(a))

5.	If the Reid vapor pressure of any solvent used in a new cold cleaner is greater than 0.6 psia; or, if any solvent used in a new cold cleaner is heated above 120 degrees Fahrenheit, then the cold cleaner must comply with at least one of the following provisions:

a.	The cold cleaner must be designed such that the ratio of the freeboard height to the width of the cleaner is equal to or greater than 0.7.  (R 336.1707(2)(a))

b.	The solvent bath must be covered with water if the solvent is insoluble and has a specific gravity of more than 1.0.  (R 336.1707(2)(b))

c.	The cold cleaner must be controlled by a carbon adsorption system, condensation system, or other method of equivalent control approved by the AQD.  (R 336.1707(2)(c))

V.  TESTING/SAMPLING

NA

VI.  MONITORING/RECORDKEEPING

Records shall be maintained on file for a period of five years.  (R 336.1213(3)(b)(ii))

1.	For each new cold cleaner in which the solvent is heated, the solvent temperature shall be monitored and recorded at least once each calendar week during routine operating conditions.  (R 336.1213(3))

2.	The permittee shall maintain the following information on file for each cold cleaner:  (R 336.1213(3))

a.	A serial number, model number, or other unique identifier for each cold cleaner.

b.	The date the unit was installed, manufactured or that it commenced operation.

c.	The air/vapor interface area for any unit claimed to be exempt under Rule 281(2)(h).

d.	The applicable Rule 201 exemption.

e.	The Reid vapor pressure of each solvent used.

f.	If applicable, the option chosen to comply with Rule 707(2).

3.	The permittee shall maintain written operating procedures for each cold cleaner.  These written procedures shall be posted in an accessible, conspicuous location near each cold cleaner.  (R 336.1611(3), R 336.1707(4))

4.	As noted in Rule 611(2)(c) and Rule 707(3)(c), if applicable, an initial demonstration that the waste solvent is a safety hazard shall be made prior to storage in non-closed containers.  If the waste solvent is a safety hazard and is stored in non-closed containers, verification that the waste solvent is disposed of so that not more than 20 percent, by weight, is allowed to evaporate into the atmosphere shall be made on a monthly basis.  (R 336.1213(3), R 336.1611(2)(c), R 336.1707(3)(c))

VII.  REPORTING

1.	Prompt reporting of deviations pursuant to General Conditions 21 and 22 of Part A.  (R 336.1213(3)(c)(ii))

2.	Semiannual reporting of monitoring and deviations pursuant to General Condition 23 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for reporting period July 1 to December 31 and September 15 for reporting period January 1 to June 30.  (R 336.1213(3)(c)(i))

3.	Annual certification of compliance pursuant to General Conditions 19 and 20 of Part A.  The report shall be postmarked or received by the appropriate AQD District Office by March 15 for the previous calendar year.  (R 336.1213(4)(c))

See Appendix 8

VIII. STACK/VENT RESTRICTION(S)

NA

IX.  OTHER REQUIREMENT(S)

NA

# E.  NON-APPLICABLE REQUIREMENTS

At the time of the ROP issuance, the AQD has determined that no non-applicable requirements have been identified for incorporation into the permit shield provision set forth in the General Conditions in Part A pursuant to Rule 213(6)(a)(ii).

# APPENDICES

## Appendix 1.  Acronyms and Abbreviations

| Common Acronyms |  | Pollutant / Measurement Abbreviations |  |
| --- | --- | --- | --- |
| AQD | Air Quality Division | acfm | Actual cubic feet per minute |
| BACT | Best Available Control Technology | BTU | British Thermal Unit |
| CAA | Clean Air Act | °C | Degrees Celsius |
| CAM | Compliance Assurance Monitoring | CO | Carbon Monoxide |
| CEM | Continuous Emission Monitoring | CO2e | Carbon Dioxide Equivalent |
| CFR | Code of Federal Regulations | dscf | Dry standard cubic foot |
| COM | Continuous Opacity Monitoring | dscm | Dry standard cubic meter |
| Department/ department | Michigan Department of Environmental Quality | °F | Degrees Fahrenheit |
|  |  | gr | Grains |
| EU | Emission Unit | HAP | Hazardous Air Pollutant |
| FG | Flexible Group | Hg | Mercury |
| GACS | Gallons of Applied Coating Solids | hr | Hour |
| GC | General Condition | HP | Horsepower |
| GHGs | Greenhouse Gases | H2S | Hydrogen Sulfide |
| HVLP | High Volume Low Pressure* | kW | Kilowatt |
| ID | Identification | lb | Pound |
| IRSL | Initial Risk Screening Level | m | Meter |
| ITSL | Initial Threshold Screening Level | mg | Milligram |
| LAER | Lowest Achievable Emission Rate | mm | Millimeter |
| MACT | Maximum Achievable Control Technology | MM | Million |
| MAERS | Michigan Air Emissions Reporting System | MW | Megawatts |
| MAP | Malfunction Abatement Plan | NMOC | Non-methane Organic Compounds |
| MDEQ | Michigan Department of Environmental Quality | NOx | Oxides of Nitrogen |
|  |  | ng | Nanogram |
| MSDS | Material Safety Data Sheet | PM | Particulate Matter |
| NA | Not Applicable | PM10 | Particulate Matter equal to or less than 10 microns in diameter |
| NAAQS | National Ambient Air Quality Standards |  |  |
| NESHAP | National Emission Standard for Hazardous Air Pollutants | PM2.5 | Particulate Matter equal to or less than 2.5 microns in diameter |
| NSPS | New Source Performance Standards | pph | Pounds per hour |
| NSR | New Source Review | ppm | Parts per million |
| PS | Performance Specification | ppmv | Parts per million by volume |
| PSD | Prevention of Significant Deterioration | ppmw | Parts per million by weight |
| PTE | Permanent Total Enclosure | % | Percent |
| PTI | Permit to Install | psia | Pounds per square inch absolute |
| RACT | Reasonable Available Control Technology | psig | Pounds per square inch gauge |
| ROP | Renewable Operating Permit | scf | Standard cubic feet |
| SC | Special Condition | sec | Seconds |
| SCR | Selective Catalytic Reduction | SO2 | Sulfur Dioxide |
| SNCR | Selective Non-Catalytic Reduction | TAC | Toxic Air Contaminant |
| SRN | State Registration Number | Temp | Temperature |
| TEQ | Toxicity Equivalence Quotient | THC | Total Hydrocarbons |
| USEPA/EPA | United States Environmental Protection Agency | tpy | Tons per year |
| VE | Visible Emissions | µg | Microgram |
|  |  | µm | Micrometer or Micron |
|  |  | VOC | Volatile Organic Compounds |
|  |  | yr | Year |

*For HVLP applicators, the pressure measured at the gun air cap shall not exceed 10 psig.

## Appendix 2.  Schedule of Compliance

The permittee certified in the ROP application that this stationary source is in compliance with all applicable requirements and the permittee shall continue to comply with all terms and conditions of this ROP.  A Schedule of Compliance is not required.  (R 336.1213(4)(a), R 336.1119(a)(ii))

## Appendix 3.  Monitoring Requirements

Specific monitoring requirement procedures, methods or specifications are detailed in Part A or the appropriate Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Therefore, this appendix is not applicable.

## Appendix 4.  Recordkeeping

The permittee shall use the following approved formats and procedures for the recordkeeping requirements referenced in FG RULE287(2)(c).  Alternative formats must be approved by the AQD District Supervisor.

| COMPLETE THE MONTHLY COATING USAGE LOG FOR EACH SURFACE COATING LINE USING THE EXEMPTION IN RULE 287(2)(c). |  |
| --- | --- |
| INSTRUCTIONS FOR COMPLETING THE MONTHLY COATING USAGE LOG: |  |
| Columns |  |
| Columns (a) and (b): | Identify the name of the coating manufacturer and the product identification number. This information can be obtained from the coating container or the MSDS. |
| Column (c): | List the coating type. This may include but not be limited to the following: precoat, primer/primer surfacer, primer sealer, topcoat, thinners, and reducers. |
| Column (d): | Record the volume of coating used, as applied, minus water, in gallons. At the end of the month, total the quantities in column (d). This total should not exceed 200 gallons. [To find the volume as applied, minus water, multiply the amount used by 1 minus the volume fraction of water in the coating. For example, if you use 5 gallons of a coating that is 40% water by volume, multiply 5 by (1-0.40). This calculation yields a coating usage of 3 gallons, as applied, minus water.] |
| Column (e) | Initials of operator or owner. |
| Column (f) | Record the volume of cleanup solvents used in gallons. Even though Rule 287(2)(c) does not address cleanup solvent usage, it is advisable to keep track of this usage. Facilities that receive Michigan Air Pollution Reporting Forms should include their usage of cleanup solvent on the forms. |
| SOURCE NAME: |  |
| MONTH/YEAR: |  |

## Appendix 5.  Testing Procedures

There are no specific testing requirement plans or procedures for this ROP.  Therefore, this appendix is not applicable.

## Appendix 6.  Permits to Install

The following table lists any PTIs issued or ROP revision applications received since the effective date of the previously issued ROP No. MI-ROP-N0034-2013. Those ROP revision applications that are being issued concurrently with this ROP renewal are identified by an asterisk (*).  Those revision applications not listed with an asterisk were processed prior to this renewal.

Source-Wide PTI No MI-PTI-N0034-2013 is being reissued as Source-Wide PTI No. MI-PTI-N0034-2019.

| Permit to Install Number | ROP Revision Application Number | Description of Equipment or Change | Corresponding Emission Unit(s) or Flexible Group(s) |
| --- | --- | --- | --- |
| 106-18 | NA | Revisions to emission limits and record keeping | EU-SPRAYBOOTHGL1, EU-SPRAYBOOTHRS1, FG-MOLDRELEASE |

## Appendix 7.  Emission Calculations

The permittee shall use the following calculations, or an alternative method that is acceptable to the DEQ-AQD, in conjunction with monitoring, testing or recordkeeping data, to determine compliance with the applicable requirements referenced in EU SPRAYBOOTHGL1, EU SPRAYBOOTHRS1, and FG MOLDRELEASE.

## Appendix 8.  Reporting

A.  Annual, Semiannual, and Deviation Certification Reporting

The permittee shall use the MDEQ, AQD, Report Certification form (EQP 5736) and MDEQ, AQD, Deviation Report form (EQP 5737) for the annual, semiannual and deviation certification reporting referenced in the Reporting Section of the Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Alternative formats must meet the provisions of Rule 213(4)(c) and Rule 213(3)(c)(i), respectively, and be approved by the AQD District Supervisor.

B.  Other Reporting

Specific reporting requirement formats and procedures are detailed in Part A or the appropriate Source-Wide, Emission Unit and/or Flexible Group Special Conditions.  Therefore, Part B of this appendix is not applicable.

## Appendix 9.  MACT - Subpart WWWW Tables 1 and 3

As specified in 40 CFR 63.5805, you must meet the following organic HAP emissions limits that apply to you:

Table 3 to Subpart WWWW of Part 63—Organic HAP Emissions Limits for Specific Open Molding, Centrifugal Casting, Pultrusion and Continuous Lamination/Casting Operations

| If your operation type is . . . | And you use . . . | 1Your organic HAP emissions limit is . . . |
| --- | --- | --- |
| 1. open molding—corrosion-resistant and/or high strength (CR/HS) | a. mechanical resin application b. filament application c. manual resin application | 113 lb/ton 171 lb/ton 123 lb/ton |
| 2. open molding—non-CR/HS | a. mechanical resin application b. filament application c. manual resin application | 88 lb/ton 188 lb/ton 87 lb/ton |
| 3. open molding—tooling | a. mechanical resin application b. manual resin application | 254 lb/ton 157 lb/ton |
| 4. open molding—low-flame spread/low-smoke products | a. mechanical resin application b. filament application c. manual resin application | 497 lb/ton 270 lb/ton 238 lb/ton |
| 5. open molding—shrinkage controlled resins2 | a. mechanical resin application b. filament application c. manual resin application | 354 lb/ton 215 lb/ton 180 lb/ton |
| 6. open molding—gel coat3 | a. tooling gel coating b. white/off white pigmented gel coating c. all other pigmented gel coating d. CR/HS or high-performance gel coat e. fire retardant gel coat f. clear production gel coat | 440 lb/ton 267 lb/ton 377 lb/ton 605 lb/ton 854 lb/ton 522 lb/ton |
| 7. centrifugal casting—CR/HS | a. resin application with the mold closed, and the mold is vented during spinning and cure b. resin application with the mold closed, and the mold is not vented during spinning and cure c. resin application with the mold open, and the mold is vented during spinning and cure d. resin application with the mold open, and the mold is not vented during spinning and cure | 25 lb/ton4 NA - this is considered to be a closed molding operation. 25 lb/ton4 Use the appropriate open molding emission limit.5 |
| 8. centrifugal casting—non-CR/HS | a. resin application with the mold closed, and the mold is vented during spinning and cure b. resin application with the mold closed, and mold is not vented during the spinning and cure c. resin application with the mold open, and the mold is vented during spinning and cure d. resin application with the mold open, and the mold is not vented during spinning and cure | 20 lb/ton4 NA - this is considered to be a closed molding operation. 20 lb/ton4 Use the appropriate open molding emission limit.5 |
| 9. pultrusion6 | N/A | Reduce total organic HAP emissions by at least 60 weight percent. |

| 10. continuous lamination/casting | N/A | Reduce total organic HAP emissions by at least 58.5 weight percent or not exceed an organic HAP emissions limit of 15.7 lbs of organic HAP per ton of neat resin plus and neat gel coat plus. |
| --- | --- | --- |

1Organic HAP emissions limits for open molding and centrifugal casting are expressed as lb/ton. You must be at or below these values based on a 12-month rolling average.

2This emission limit applies regardless of whether the shrinkage controlled resin is used as a production resin or a tooling resin.

3If you only apply gel coat with manual application, for compliance purposes treat the gel coat as if it were applied using atomized spray guns to determine both emission limits and emission factors.  If you use multiple application methods and any portion of a specific gel coat is applied using non-atomized spray, you may use the non-atomized spray gel coat equation to calculate an emission factor for the manually applied portion of that gel coat. Otherwise, use the atomized spray gel coat application equation to calculate emission factors.

4For compliance purposes, calculate your emission factor using only the appropriate centrifugal casting equation in item 2 of Table 1 to this subpart, or a site specific emission factor for after the mold is closed as discussed in 63.5796.

5Calculate your emission factor using the appropriate open molding covered cure emission factor in item 1 of Table 1 to this subpart, or a site specific emission factor as discussed in 63.5796.

6Pultrusion machines that produce parts that meet the following criteria: 1,000 or more reinforcements or the glass equivalent of 1,000 ends of 113 yield roving or more; and have a cross sectional area of 60 square inches or more are not subject to this requirement. Their requirement is the work practice of air flow management which is described in Table 4 to this subpart.